| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0148 |
| --- | --- |

| Title: | Deblocking filter using adaptive weighting factors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ann-Kathrin Seifert Thomas Wedi Matthias Narroschke Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | Ann-Kathrin.Seifert@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is part of CE10. It presents the results for a Deblocking filter using adaptive weighting factors which is based on JCTVC-G574. Five adaptive weighting factors are introduced into the deblocking filter of the luminance signal. Each weighting factor weights the value, by which the HM5.0 deblocking filter would modify a sample. The weighting can be interpreted as an adjustment of the frequency responses of the deblocking filter. In this contribution, the weighting factors are estimated at encoder side by minimizing the mean squared error between the deblocked signal and the original input signal considering the subjective importance of samples. However, any other estimation method is also possible since the encoded weighting factors are coded and transmitted to the receiver side. It is also possible to use default weighting factors, which lead to deblocking filter operations equivalent to the ones of HM5.0. For the presented estimation method, the measured BD-bit rates are: I-HE: -0.1%, I-LC: -0.1%, RA-HE: 0.0%, RA-LC: 0.0%, RA-HE10: -0.2%, LD-HE(B): 0.0%, LD-LC(B): 0.1%, LD-HE(P): 0.1%, LD-LC(P): 0.2%. The measured encoder/decoder run time ratios are around 100%. Blocking artifacts are visibly removed, especially for high QP coder configurations. It is proposed to include this proposal into the next version of the HM.

# Introduction

The current HM 5.0 [1] [2] applies hybrid coding. The generalized block diagram of the hybrid encoder is shown in Figure 1. In a first step, the input signal to be coded is predicted block-wise by either motion compensated prediction or Intra prediction. The resulting prediction error is block-wise transform coded by applying an approximation of the discrete cosine transform, also known as Integer DCT, followed by a quantization of the coefficients. Due to the block wise motion compensated prediction and a block wise prediction error coding, so called blocking artifacts often become visible in the decoded images. These blocking artifacts tend to be annoying for human observers. In order to reduce these annoying blocking artifacts, a deblocking filter is applied. The deblocked signal is further filtered by the use of sample adaptive offsets (SAO) and an adaptive loop filter (ALF) before being output and stored for further predictions. Both, SAO and ALF estimate control parameters based on the original input signal s and on the signal, these filters operate on, which is s'' for SAO and s for ALF. This enables SAO and ALF to adapt to varying statistics of the input signals. The control parameters are coded and transmitted to the receiver. Figure 2 illustrates the signal before and after the deblocking filter for a region of the example test sequence Kimono. In the next Section 2, the deblocking filter of HM5.0 is described in detail. Section 3 presents the proposed deblocking filter using adaptive weighting factors, Section 5 the experiments and results. Conclusions are drawn in Section 6.

Figure 1: Generalized block diagram of a hybrid encoder according to HM5.0 [1].

Figure 2: Illustration of the signal before and after the deblocking filter for a region of the example test sequence Kimono.

# Deblocking filter according to HM5.0

Deblocking of images is performed based on edge segments of 8 consecutive lines or columns. The respective sides of an edge segment are denoted as A and B. The segment A corresponds to the left neighboring partition to B for vertical edges and to the above neighboring partition to B for horizontal edges. The samples on the respective sides of an edge segment are denoted as also shown in Figure 3.

Figure 3: Notation of a segment of a vertical edge for deblocking.

For each line and column of the luminance signal, it is decided based on various conditions, if a strong filter, a weak filter, or no filter is applied. Details on the conditions can be found in [1] and [2]. The filter operations of the strong and the weak filter are shown in the subsequent sections 2.1 and 2.2.

## Strong luminance filter

p0i'=p2i+2⋅p1i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i'=p1i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

p1i'=p2i+p1i+p0i+q0i+2>>2

q1i'=p0i+q0i+q1i+q2i+2>>2

p2i'=2⋅p3i+3⋅p2i+p1i+p0i+q0i+4>>3

q2i'=p0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

The operator “>>x” defines a bit-shift operation by x bits to the right.

## Weak luminance filter

Δweak,i=Clip3−tc,tc,9⋅q0i−p0i−3⋅q1i−p1i+8>>4

p0i'=Clipp0i+Δweak,i

q0i'=Clipq0i−Δweak,i

Δweak1,i=Clip3−(tc>>1),(tc>>1),((((p2i+p0i+1)>>1)−p1i+Δweak,i)>>1)

p1i'=Clipp1i+Δweak1,i

Δweak2,i=Clip3−(tc>>1),(tc>>1),((((q2i+q0i+1)>>1)−q1i−Δweak,i)>>1)

q1i'=Clipq1i+Δweak2,i

Herby, the clipping Clipx function is defined by

Clipx=0;x<0max_value;x>max_valuex;else.

Herby, the clipping function Clip3a,b,x is defined by

Clip3a,b,x=a;x<ab;x>bx;else.

# Deblocking filter using adaptive weighting factors

The deblocking of a sample sk' according to HM5.0 can be written in the form

sk''=sk'+Δk.

Hereby, Δk is the value by which the deblocking filter modifies a sample sk'. The value Δk results from a filter operation in combination with a clipping operation. In the case of the weak luminance filter, these modification values are Δweak, see section 2.2.

In order to enable an adaptation of the deblocking filter to varying signal statistics with a negligible increase of computational expense, adaptive weighting factors are introduced into the deblocking filter, which weight the values Δk by an adaptive weighting factor α:

sk''=sk'+Δk⋅α.

Three separate weighting factors are used for the strong filter. Each one is selected based on the distance of the sample to be filtered from the block boundary. For the weak filter, two separate weighting factors are used. The selection is performed respectively. Thus, in total five weighting factors are used.

All five weighting factors are estimated on the encoder side based on the original input signal s and on the reconstructed signal s', on which the deblocking filter operates on. The estimated weighting factors are quantized, coded and transmitted to the decoder, see Figure 4. However, other estimation methods are also possible.

Figure 4: Generalized block diagram of a hybrid encoder according to HM5.0 [1], extended by the proposed deblocking filter using adaptive weighting factors.

In the following three sections, the filter operations, which include the quantized weighting factors, are shown in detail.

## Strong luminance filter

Δp0,i=p2i+2⋅p1i−6⋅p0i+2⋅q0i+q1i

Δq0,i=p1i+2⋅p0i−6⋅q0i+2⋅q1i+q2i

Δp1,i=p2i−3⋅p1i+p0i+q0i

Δq1,i=p0i+q0i−3⋅q1i+q2i

Δp2,i=2⋅p3i−5⋅p2i+p1i+p0i+q0i

Δq2,i=p0i+q0i+q1i−5⋅q2i+2⋅q3i

p2i'=Clipp2i<<6+Δp2,i⋅α1+32>>6

q2i'=Clipq2i<<6+Δq2,i⋅α1+32>>6

p1i'=Clipp1i<<6+Δp1,i⋅α2+32>>6

q1i'=Clipq1i<<6+Δq1,i⋅α2+32>>6

p0i'=Clipp0i<<6+Δp0,i⋅α3+32>>6

q0i'=Clipq0i<<6+Δq0,i⋅α3+32>>6

## Weak luminance filter

Δweak,i=Clip3−tc,tc,9⋅q0i−p0i−3⋅q1i−p1i+8>>4

p0i'=Clipp0i<<6+Δweak,i⋅α4+32>>6

q0i'=Clipq0i<<6−Δweak.i⋅α4+32>>6

Δweak1,i=Clip3−(tc>>1),(tc>>1),((((p2i+p0i+1)>>1)−p1i+Δweak,i)>>1)

p1i'=Clipp1i<<6+Δweak1,i⋅α5+32>>6

Δweak2,i=Clip3−(tc>>1),(tc>>1),((((q2i+q0i+1)>>1)−q1i−Δweak,i)>>1)

q1i'=Clipq1i<<6+Δweak2,i⋅α5+32>>6

## Estimation of weights

The weights associated to the samples p1, p2, q1, and q2 are estimated by minimization of the mean squared reconstruction error between the original input signal s and the output signal s'' of the deblocking filter:

Es−s''2=Es−s'+Δ⋅α2→min

The weights associated to the samples p0 and q0 are estimated differently. In a first step, sample pairs p0 and q0, that are important for the human visual system, are determined. For this determination, the non-normalized AC-coefficients of a hadamard transform are calculated by using the eight samples p3, p2, p1, p0, q0, q1, q2 and q3 of the reconstructed signal s' and of the original signal s:

c1,s'=p3s'p2s'p1s'p0s'q0s'q1s'q2s'q3s'⋅1111−1−1−1−1T

c1,s=p3sp2sp1sp0sq0sq1sq2sq3s⋅1111−1−1−1−1T

The value c1,s'2 is a measure for the energy of the change of the reconstructed signal at the block boundary, c1,s2 is the same measure for the original signal. Sample pairs are considered important if c1,s'2−1.2⋅c1,s2>0, otherwise not.

In a second step, the weights associated to the subjectively important sample pairs p0 and q0 are estimated by minimization of the mean squared reconstruction error between the original input signal and the output signal of the deblocking filter.

Finally, all estimated weighting factors are limited to the ones shown in Figure 5 for the cases drawn in blue.

## Coding of weighting factors

The five estimated weights α1,…,α5 are quantized using a quantization step size of 1/64. The quantized weights are coded within the slice header using prediction between them and variable length coding of the resulting prediction error. No arithmetic coding is applied.

# Frequency responses

By the introduction of the weighting factors, the frequency responses of the filters are modified. In the following Figure 5, the frequency responses of the three strong and of the two weak filters are illustrated for the case α=1, which is equivalent to HM5.0, and for example cases α>1.0. It can be seen how the characteristic of each filter changes by increasing α.

Figure 5: Illustration of frequency responses.

# Experiments and results

The deblocking filter using adaptive weighting factors has been integrated into a software provided by Toshiba to the CE10 participants. This software is HM5.0 [1] extended by the integration of the adopted slice level parameters for the deblocking as described in JCTVC-G174 [6]. This software is used as a reference. Following the common conditions [4], the performance is evaluated for all seven mandatory test cases, which are Intra, Random access, Low delay B each in high efficiency and low complexity operation mode and test case Random access, high efficiency, 10 bit. Additionally results are given for two optional test cases, which are Low delay P in high efficiency and low complexity operation mode. For all run time measurements, computers of the same configuration are used.

The BD-rate results as well as the encoder-/decoder run time ratios compared to the reference are shown in Figure 6. Negative BD-rate numbers show a gain compared to the reference. Run-time ratios less than 100% reflect that the run time is lower than the one of the reference.

Figure 6: BD-bit rates and run time ratios of the deblocking filter using adaptive weighting factors compared to the reference.

## Subjective evaluation

Various test sequences have been used for a subjective evaluation. The subjective quality of the proposal has been compared to the reference. The results are shown in the following table. Sequences with substantially improved subjective quality are marked in green.

| Test sequence | Coding configuration | QP | Subjective quality of the proposal compared to HM5.0 |
| --- | --- | --- | --- |
| Kimono | Random access, high efficiency | 37 | Similar |
| BQ_Mall | Random access, high efficiency | 37 | Similar |
| Vidyo3 | Low delay, high efficiency | 37 | Less blocking on board |
| Cactus | Low delay, high efficiency | 37 | Less blocky |
| Cactus | Low delay, high efficiency | 42 | Less blocky, less flickering, more stable |
| BasketballDrill | Random access, high efficiency | 42 | Less blocky, less flickering |
| RaceHorses (832x480) | Low delay, low complexity | 42 | Less blocky, less flickering |
| Vidyo3 | Low delay, high efficiency | 42 | Less blocking, more details |
| BasketballDrill | Random Access, high efficiency | 42 | Less blocky, less flickering |
| RaceHorses (832x480) | Low delay, high efficiency | 42 | Less blocky |
| Vidyo3 | Low delay, low complexity | 42 | Less blocking, more details |

In Figure 7, a cropped part of the decoded frame 269 of the test sequence Cactus, which was coded by the reference in the configuration Low delay, High Efficiency, QP42, is shown. Figure 8 shows the same cropped part of the same decoded frame 269, which was coded by the proposal in the same configuration.

Figure 7: Cropped part of the decoded frame 269 of the test sequence Cactus in the case of the reference. Test case: Low delay, High Efficiency, QP42.

Figure 8: Cropped part of the decoded frame 269 of the test sequence Cactus in the case of the proposal. Test case: Low delay, High Efficiency, QP42.

In Figure 9, a cropped part of the decoded frame 107 of the test sequence BasketballDill, which was coded by the reference in the configuration Random Access, High Efficiency, QP42, is shown. Figure 10 shows the same cropped part of the same decoded frame 107, which was coded by the proposal in the same configuration.

Figure 9: Cropped part of the decoded frame 107 of the test sequence BasketballDrill in the case of the reference. Test case: Random Access, High Efficiency, QP42.

Figure 10: Cropped part of the decoded frame 107 of the test sequence BasketballDrill in the case of the proposal. Test case: Random Access, High Efficiency, QP42.

In Figure 11, a cropped part of the decoded frame 218 of the test sequence RaceHorses, which was coded by the reference in the configuration Low Delay, High Efficiency, QP42, is shown. Figure 12 shows the same cropped part of the same decoded frame 218, which was coded by the proposal in the same configuration.

Figure 11: Cropped part of the decoded frame 218 of the test sequence RaceHorses in the case of the reference. Test case: Low delay, High Efficiency, QP42.

Figure 12: Cropped part of the decoded frame 218 of the test sequence RaceHorses in the case of the proposal. Test case: Low delay, High Efficiency, QP42.

From the figures above, it can be seen that the annoying blocking artifacts are significantly reduced.

# Conclusion

This contribution is part of CE10. It presents the results for a Deblocking filter using adaptive weighting factors which is based on JCTVC-G574. Five adaptive weighting factors are introduced into the deblocking filter of the luminance signal. Each weighting factor weights the value, by which the HM5.0 deblocking filter would modify a sample. The weighting can be interpreted as an adjustment of the frequency responses of the deblocking filter. In this contribution, the weighting factors are estimated at encoder side by minimizing the mean squared error between the deblocked signal and the original input signal considering the subjective importance of samples. However, any other estimation method is also possible since the encoded weighting factors are coded and transmitted to the receiver side. It is also possible to use default weighting factors, which lead to deblocking filter operations equivalent to the ones of HM5.0. For the presented estimation method, the measured BD-bit rates are: I-HE: -0.1%, I-LC: -0.1%, RA-HE: 0.0%, RA-LC: 0.0%, RA-HE10: -0.2%, LD-HE(B): 0.0%, LD-LC(B): 0.1%, LD-HE(P): 0.1%, LD-LC(P): 0.2%. The measured encoder/decoder run time ratios are around 100%. Blocking artifacts are visibly removed, especially for high QP coder configurations. It is proposed to include this proposal into the next version of the HM.

# References

- HM5.0 software: http://hevc.kw.bbc.co.uk/trac/browser/jctvc-hm/tags/HM-5.0.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, High Efficiency Video Coding (HEVC) text specification Working Draft 5, JCTVC-G1103, Geneva, CH, November 2011.

- A. Norkin, Xun Guo, B. Jeon, M. Narroschke, Description of Core Experiment 10 (CE10): Deblocking Filtering, JCTVC-G1210, Geneva, CH, November 2011.

- F. Bossen, Common HM test conditions and software reference configurations, JCTVC-G1200, Geneva, CH, November 2011.

- M. Narroschke, A-K. Seifert, Deblocking filter using adaptive weighting factors, JCTVC-G574, Geneva, CH, November 2011.

- T. Yamakage, S. Asaka, T. Chujoh, M. Karczewicz, I. S. Chong, CE12: Deblocking filter parameter adjustment in slice level, JCTVC-G174, Geneva, CH, November 2011.

# Working draft changes

Required changes in the working draft text are highlighted using yellow color and strikes. The working draft text for using adaptive weighting factors is as follows:

Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| Else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |

| [Ed. (BB): coded as absolute value slice_qp in HM software, should be implemented together with pic_init_qp_minus26 ] |  |
| --- | --- |
| inherit_dbl_params_from_APS_flag | u(1) |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| [Ed. (BB): Although it was agreed to use AVC-like deblocking control syntax, WD and HM should be changed to make use of the AVC-like syntax.] |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| byte_align( ) |  |
| alf_cu_control_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| 5_minus_max_num_merge_cand | ue(v) |
| for( i = 0; i < num_substreams_minus1 + 1; i++ ){ |  |
| substream_length_mode | u(2) |
| substream_length[i] | u(v) |
| } |  |
| weighting_factors_for_deblocking_enabled_flag | u(1) |
| if(weighting_factors_for_deblocking_enabled_flag){ |  |
| for (i=0; i<3; i++) |  |
| alpha_strong_delta[i] | se(v) |
| for (i=0; i<2; i++) |  |
| alpha_weak_delta[i] | se(v) |
| } |  |
| } |  |

Slice header semantics

weighting_factors_for_deblocking_enabled_flag specifies the decoding of weighting factors for the deblocking filter. When  weighting_factors_for_deblocking_enabled_flag is equal to 0, the weighting factors are set to

αs[0] = 8

αs[1] = 16

αs[2] = 8

αw[0] = 64

αw[1] = 64.

alpha_strong_delta[i] plus predictor_strong[i] specifies the weighting factor αs[i] for the deblocking filter. Each weighting factor αs[i] is computed as

αs[i] = predictor_strong[i] + alpha_strong_delta[i].

Table 7-5 – predictor_strong[i] versus i

| i | predictor_strong[i] |
| --- | --- |
| 0 | 8 |
| 1 | 2*αs[0] |
| 2 | ( (αs[1]>>1)+ αs[0] ) >>1) |

alpha_weak_delta[i] plus predictor_weak[i] specifies the weighting factor αw[i] for the deblocking filter. Each weighting factor αw[i] is computed as

αw[i] = predictor_weak[i] + alpha_weak_delta[i].

Table 7-6 – predictor_weak[i] versus i

| i | predictor_weak[i] |
| --- | --- |
| 0 | 64 |
| 1 | αw[0] |

Filtering process for a luma sample

Depending on dSam, the following applies:

- When the variable dSam is equal to 1, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 		
p0’ = Clip1Y( ( (p0 <<6)+ αs[0]*( p2 + 2*p1 + 2*p0 - 6*q0 + q1 )+32) >> 6 ) 	(8-1)

p1’ = ( p2 + p1 + p0 + q0 + 2 ) >> 2 		
p1’ = Clip1Y( ( (p1 <<6)+ αs[1]*( p2 - 3*p1 + p0 + q0  )+32) >> 6 ) 	(8-2)

p2’ = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 		
p2’ = Clip1Y( ( (p2 <<6)+ αs[2]*( 2*p3 - 5*p2 + p1 + p0 + q0  )+32) >> 6 ) 	(8-3)

q0’ = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 		
q0’ = Clip1Y( ( (q0 <<6)+ αs[0]*( p1 + 2*p0 - 6*q0 + 2*q1 + q2 )+32) >> 6 ) 	(8-4)

q1’ = ( p0 + q0 + q1 + q2 + 2 ) >> 2 		
q1’ = Clip1Y( ( (q1 <<6)+ αs[1]*( p0 + q0 - 3*q1 + q2 )+32) >> 6 ) 	(8-5)

q2’ = ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 		
q2’ = Clip1Y( ( (q2 <<6)+ αs[2]*( p0 + q0 + q1 - 5*q2 + 2*q3 )+32) >> 6 ) 	(8-6)

–	Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4		(8-7)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( -tc, tc,  )		(8-8)

p0’ = Clip1Y( p0 +  )	
p0’ = Clip1Y( ( (p0 <<6)+ αw[0]*+32)>>6 )		(8-9)

q0’ = Clip1Y( q0 -  )
		q0’ = Clip1Y( ( (q0 <<6)- αw[0]*+32)>>6 )		(8-10)

    - If dEp1 is equal to 1, the filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-11)

pi’ = Clip1Y( p1 + p )	
		pi’ = Clip1Y( ( (p1 <<6)+ αw[1]*p+32)>>6 )		 (8-12)

    - If dEq1 is equal to 1, the filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-13)

qi’ = Clip1Y( q1 + q )
		qi’ = Clip1Y( ( (q1 <<6)+ αw[1]*q+32)>>6 )		 (8-14)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).