| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0149 |
| --- | --- |

| Title: | CE10.2: Cross check results for the deblocking filter line buffer reduction proposal JCTVC-H0280 of Sony and MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Anand Kotra Semih Esenlik Matthias Narroschke Panasonic R&D Center Germany Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49 6103 766-154 Anand.Kotra@eu.panasonic.com +49 6103 766-1194 Semih.Esenlik@eu.panasonic.com +49 6103 766-204 Matthias.Narroschke@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This documents reports cross check results for the CE10.2 proposal JCTVC-H0280 “CE10.2.1: Reducing one pixel line buffer by modified deblocking filter for horizontal LCU boundaries” by Sony and Mediatek.   The proposal JCTVC-H0280 consists of two technical changes compared to HM5.0. First, one line buffer is reduced by modifying the deblocking filter. Second, the so called luminance virtual boundary of adaptive loop filtering is shifted to three pixels above the horizontal LCU boundary to match the proposed modified deblocking. The combination of both technical changes is cross checked in this contribution. The BD-bit rate results as well as the run time ratios reported by Sony and MediaTek are confirmed.

# Introduction

Sony and MediaTek have provided source code and results for their CE10.2 proposal on line buffer reduction for deblocking on 6th January 2012.

In the software, both technical changes are enabled by the use of the following macro definitions:

#define MTK_DF_R3W2    1

#define SONY_DF_R3W2   1

#define MTK_DF_R2W1    1

#define MTK_DISABLE_DF_LCU_BOUNDARY_LUMA    1

#define ALF_VB_L3    1

Following the common conditions [2], the performance for high efficiency cases have been evaluated. The executables were compiled on 64-bit Linux with the compiler gcc 4.1.2.

For encoding, the compiled HM5.0 reference software [1] and the compiled software of Sony/Mediatek are executed on a Linux-cluster with computers of the same configuration.

For decoding, the compiled HM5.0 reference software [1] and the compiled software of Sony/ Mediatek are executed on a single Linux-PC, configured with Hyper-Threading and Turbo-Mode turned off. No other processes except for the operating system were running in parallel.

The measured BD-rates and run time ratios are shown below. All details can be found in the attached excel-sheet.

# Comparison of the achieved results to the results reported by Sony/MediaTek

The measured BD-bit rate results match perfectly to those reported by Sony/Mediatek, the run time ratios match approximately.

# Software inspection

The software was reviewed. The software seems to perform the algorithms described in the corresponding contribution JCTVC-H0280.

# Conclusion

The results of Sony/Mediatek for JCTVC-H0280 [3] are confirmed.

# References

- HM-5.0 software, , http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-5.0

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200,         Geneva,       November, 2011.

- C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S. Lei, M. Ikeda, T. Suzuki, “CE10.2.1: Reducing one pixel line buffer by modified deblocking filter for horizontal LCU boundaries”, JCTVC-H0280, San José, February, 2012.