| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0150 |
| --- | --- |

| Title: | CE4 Subtest 2.1: Crosscheck report of Enhancements of AVC quantization matrix in HEVC (JCTVC-H0230) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the crosscheck result of CE4 Subtest 2.1 (JCTVC-H0230), proposal of Enhancements of AVC quantization matrix in HEVC proposed by Sony and TI.

# Cross Check Procedure

The source code corresponds to CE4 Subtest 2.1 was provided by Sony, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  According to the test description in JCTVC-G1204 (CE4 description), the following performance was measured:

- 6 sets symmetry and asymmetry quantization matrices (12 in total) coding performance

  - Results of compression performance of matrix itself with error statistics

The test condition defined in JCTVC-G1204 subtest 2 was used for the crosscheck.

In addition, the source code was carefully studied and the observation of the crosschecker is noted in the later chapter.

# Results

## Quantization Matrices Coding Performance

The proponents of subtest 2.1 presented 2 options to be tested: option1 and option2b.  Accordingly, quantization matrices coding performance for each option was measured and reported.

### Option1

Using the provided software and configuration file for option1, option1 testing was conducted for all 12 quantization matrices (symmetry1 to symmetry6 and asymmetry1 to asymmetry6).  The measured results are as follows:

| (Subtest2.1, option1) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 4 | 4 | 4 | 4 | 4 | 4 |
| Matrix [0][0] Bit | 67 | 55 | 81 | 75 | 67 | 81 |
| Matrix [0][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][3] Bit | 67 | 39 | 73 | 73 | 67 | 73 |
| Matrix [0][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][0] Bit | 209 | 165 | 267 | 257 | 165 | 209 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][3] Bit | 185 | 93 | 245 | 243 | 93 | 185 |
| Matrix [1][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Total Bit | 548 | 372 | 686 | 668 | 412 | 568 |
| MaxError | 30 | 30 | 85 | 47 | 62 | 96 |
| AvgError | 4.11 | 5.23 | 18.23 | 6.32 | 17.35 | 34.11 |

Table 1: Symmetry quantization matrices coding results for proposed subtest2.1 option1

| (Subtest2.1, option1) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 4 | 4 | 4 | 4 | 4 | 4 |
| Matrix [0][0] Bit | 73 | 69 | 91 | 121 | 73 | 91 |
| Matrix [0][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][3] Bit | 93 | 59 | 97 | 139 | 93 | 97 |
| Matrix [0][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][0] Bit | 381 | 283 | 459 | 623 | 285 | 383 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][3] Bit | 331 | 181 | 429 | 567 | 181 | 331 |
| Matrix [1][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Total Bit | 898 | 612 | 1096 | 1470 | 652 | 922 |
| MaxError | 28 | 21 | 103 | 129 | 55 | 143 |
| AvgError | 4.09 | 3.28 | 21.81 | 18.01 | 15.52 | 31.48 |

Table 2: Asymmetry quantization matrices coding results for proposed subtest2.1 option1

### Option2b

Similarly, using the provided software and configuration file for option2b, option2b testing was conducted for all 12 quantization matrices (symmetry1 to symmetry6 and asymmetry1 to asymmetry6).  The measured results are as follows:

| (Subtest2.1, option2b) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 4 | 4 | 4 | 4 | 4 | 4 |
| Matrix [0][0] Bit | 67 | 55 | 81 | 75 | 67 | 81 |
| Matrix [0][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][3] Bit | 67 | 39 | 73 | 73 | 67 | 73 |
| Matrix [0][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][0] Bit | 209 | 165 | 267 | 257 | 165 | 209 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][3] Bit | 185 | 93 | 245 | 243 | 93 | 185 |
| Matrix [1][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [2][0] Bit | 228 | 144 | 262 | 238 | 262 | 144 |
| Matrix [2][1] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [2][2] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [2][3] Bit | 154 | 82 | 168 | 242 | 168 | 82 |
| Matrix [2][4] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [2][5] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [3][0] Bit | 164 | 136 | 292 | 268 | 228 | 292 |
| Matrix [3][1] Bit | 174 | 72 | 260 | 244 | 174 | 260 |
| Total Bit | 1312 | 850 | 1712 | 1704 | 1288 | 1390 |
| MaxError | 39 | 16 | 94 | 40 | 39 | 94 |
| AvgError | 2.39 | 0.67 | 5.30 | 4.94 | 2.62 | 4.48 |

Table 3: Symmetry quantization matrices coding results for proposed subtest2.1 option2b

| (Subtest2.1, option2b) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 4 | 4 | 4 | 4 | 4 | 4 |
| Matrix [0][0] Bit | 73 | 69 | 91 | 121 | 73 | 91 |
| Matrix [0][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][3] Bit | 93 | 59 | 97 | 139 | 93 | 97 |
| Matrix [0][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [0][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][0] Bit | 381 | 283 | 459 | 623 | 285 | 383 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][3] Bit | 331 | 181 | 429 | 567 | 181 | 331 |
| Matrix [1][4] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][5] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [2][0] Bit | 184 | 212 | 340 | 604 | 372 | 234 |
| Matrix [2][1] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [2][2] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [2][3] Bit | 178 | 128 | 336 | 578 | 302 | 170 |
| Matrix [2][4] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [2][5] Bit | 11 | 11 | 11 | 11 | 11 | 11 |
| Matrix [3][0] Bit | 158 | 154 | 318 | 612 | 278 | 412 |
| Matrix [3][1] Bit | 326 | 128 | 444 | 588 | 288 | 458 |
| Total Bit | 1788 | 1278 | 2578 | 3896 | 1936 | 2240 |
| MaxError | 43 | 8 | 48 | 115 | 20 | 48 |
| AvgError | 2.55 | 0.64 | 5.82 | 14.79 | 2.76 | 4.99 |

Table 4: Asymmetry quantization matrices coding results for proposed subtest2.1 option2b

The presented results above matched the results provided by the proponent of subtest 2.1.

# Observation

The crosschecker carefully investigated the provided software for subtest 2.1, and found the following elements are included in the software.  It should be noted that the name of the following syntax and parameter is derived from the source code, and may be different from the preferred naming by the proponents.

## A high level flag: “large_block_size_qm_present_flag”

In APS level, there is a flag to enable/disable the signaling of 16x16 and 32x32 matrices.  If 16x16 and 32x32 matrices signaling is disabled by this flag, 16x16 and 32x32 matrices will be predicted from 8x8 matrices.

## A new syntax: “qscale”

In APS level, there is a syntax element for the parameter, which will be used to actually ‘quantize’ the quantization matrices coefficients.  By setting this “qscale” value greater than 1, the amount of bits to code quantization matrices will be reduced.  However, it should be noted that this value was set equal to 1 for the CE tests, and never used eventually.

## Larger matrices prediction from 8x8 matrices

This item is closely related to “large_block_size_qm_present_flag”, but the larger matrices (i.e. 16x16 and 32x32) will be generated from 8x8 matrices if “large_block_size_qm_present_flag” is set equal to 0.  For the larger matrices generation, upsampling (copying the value without interpolation) is used in the software.

## New signaling mode called “arbitrary mode”

In addition to the existing quantization matrices signaling modes in HM5 (i.e. prediction and DPCM), a new signaling mode called “arbitrary mode” was added in the software.  This mode seems to contain the matrices prediction from other matrices (including other sizes), and its residual coding.

## Down conversion before coding of larger matrices

For 16x16 or larger matrices, there is an option to down convert the larger matrices to smaller matrices (made by sub-sampling).

## Symmetry coding

There is a flag for each quantization matrix to indicate if only the upper right triangle will be coded or entire square will be coded.  This option should be useful to code symmetry matrices.

## Entropy coding switch: “sign_flag”

For DPCM coding mode, there is a flag for each quantization matrix to indicate signed or unsigned Exp-Golomb codes are used to code matrices.

Apparently, the subtest 2.1 software contains quite a few technical elements, and the crosschecker request the group to evaluate each element rather than as a package.

# Conclusion

It has been verified that the results of “Enhancements of AVC quantization matrix in HEVC” provided as subtest 2.1 results (equivalent to JCTVC-H0230) are accurate.  Also, it has been confirmed that the subtest 2.1 software contains at least 7 technical elements, and it is recommended to evaluate each element of subtest 2.1 rather than to evaluate them as a package.