| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0151 |
| --- | --- |

| Title: | CE4 Subtest 2.2: Crosscheck report of up-sampling large size quantization matrices based on JCTVC-G152 (JCTVC-H0384) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the crosscheck result of CE4 Subtest 2.2 (JCTVC-H0384), proposal of up-sampling large size quantization matrices based on JCTVC-G152 proposed by MediaTek.

# Cross Check Procedure

The source code corresponds to CE4 Subtest 2.2 was provided by MediaTek, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  According to the test description in JCTVC-G1204 (CE4 description), the following performance was measured:

- 6 sets symmetry and asymmetry quantization matrices (12 in total) coding performance for test1

  - Results of compression performance of matrix itself with error statistics

- BD-rate performance of proposed up-converted matrix compared to flat matrix (HM5) for test2

The test condition defined in JCTVC-G1204 subtest 2 was used for the crosscheck.

In addition, the source code was carefully studied and the observation of the crosschecker is noted in the later chapter.

# Results

## Quantization Matrices Coding Performance (test1)

The proponents of subtest 2.2 presented 2 items to be tested: test1 and test2.  Test1 tested the coding performance when up-converted matrices (generated from 8x8 matrices) are used as 16x16 and 32x32 quantization matrices.  Using the profiled software and configuration file for test1, test1 was conducted for all 12 quantization matrices (symmetry1 to symmetry6 and asymmetry1 to asymmetry6).  The measured results are as follows:

| (Subtest2.2, Test1) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][0] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][1] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][2] Bit | 79 | 59 | 97 | 91 | 79 | 97 |
| Matrix [0][3] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [0][4] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [0][5] Bit | 81 | 45 | 91 | 89 | 81 | 91 |
| Matrix [1][0] Bit | 335 | 261 | 427 | 385 | 261 | 335 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 335 | 261 | 427 | 385 | 261 | 335 |
| Matrix [1][3] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Matrix [1][4] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Matrix [1][5] Bit | 291 | 141 | 385 | 413 | 141 | 291 |
| Total Bit | 2026 | 1260 | 2576 | 2552 | 1428 | 2110 |
| MaxError | 19 | 14 | 66 | 37 | 62 | 103 |
| AvgError | 2.00 | 3.45 | 19.98 | 4.87 | 18.83 | 33.99 |

Table 1: Symmetry quantization matrices coding results for proposed subtest2.2 test1

| (Subtest2.2, Test1) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Header Bit | 1 | 1 | 1 | 1 | 1 | 1 |
| Matrix [0][0] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][1] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][2] Bit | 89 | 65 | 101 | 117 | 89 | 101 |
| Matrix [0][3] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [0][4] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [0][5] Bit | 107 | 55 | 105 | 135 | 107 | 105 |
| Matrix [1][0] Bit | 381 | 281 | 473 | 621 | 281 | 381 |
| Matrix [1][1] Bit | 2 | 2 | 2 | 2 | 2 | 2 |
| Matrix [1][2] Bit | 381 | 281 | 473 | 621 | 281 | 381 |
| Matrix [1][3] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Matrix [1][4] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Matrix [1][5] Bit | 349 | 177 | 431 | 571 | 177 | 349 |
| Total Bit | 2400 | 1456 | 2860 | 3714 | 1684 | 2430 |
| MaxError | 10 | 11 | 86 | 129 | 55 | 134 |
| AvgError | 2.19 | 2.78 | 17.22 | 15.43 | 15.57 | 28.32 |

Table 2: Asymmetry quantization matrices coding results for proposed subtest2.2 test1

The presented results above matched the results provided by the proponent of subtest 2.2.

## Simulation Results of Proposed Default Quantization Matrices (test2) vs. Flat Matrix

The other test (test2) generates 16x16 and 32x32 default quantization matrices from down-converted 8x8 matrices.  Crosscheck results for test2 are summarized in the following table, where proposed default quantization matrix (test2) is compared with flat matrix (HM5).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 7.0% | 6.3% | 5.0% | 11.5% | -9.9% | -16.4% |
| Class B | 7.7% | 0.7% | 0.2% | 11.6% | -15.1% | -18.4% |
| Class C | 12.5% | 3.5% | 2.4% | 17.4% | -5.2% | -8.5% |
| Class D | 13.3% | 2.0% | 2.8% | 17.5% | -7.9% | -7.3% |
| Class E | 6.3% | 4.1% | 3.9% | 12.6% | -31.8% | -26.9% |
| Overall | 9.7% | 2.8% | 2.4% | 14.3% | -13.5% | -14.9% |
|  | 9.6% | 2.8% | 2.5% | 14.3% | -13.4% | -14.7% |
| Class F | 11.8% | 5.7% | 7.3% | 16.6% | 4.6% | 7.6% |
| Enc Time[%] | 97% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 3.8% | 2.8% | 0.4% | 6.4% | -15.5% | -22.5% |
| Class B | 3.9% | -2.6% | -3.7% | 6.0% | -17.5% | -21.2% |
| Class C | 8.2% | 2.2% | 0.6% | 11.3% | -6.8% | -10.2% |
| Class D | 8.5% | -1.4% | -1.8% | 11.1% | -13.0% | -13.5% |
| Class E |  |  |  |  |  |  |
| Overall | 6.2% | -0.3% | -1.5% | 8.8% | -13.2% | -16.4% |
|  | 6.2% | -0.3% | -1.4% | 8.9% | -13.0% | -16.2% |
| Class F | 10.7% | 7.8% | 9.6% | 16.0% | 7.3% | 11.7% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.2% | -0.9% | -1.4% | 3.6% | -12.1% | -15.3% |
| Class C | 4.7% | 0.8% | 0.2% | 8.6% | -6.3% | -8.8% |
| Class D | 4.1% | -1.8% | -1.8% | 7.0% | -12.1% | -13.0% |
| Class E | 1.5% | -1.9% | -2.1% | 3.6% | -28.4% | -27.4% |
| Overall | 3.2% | -0.9% | -1.2% | 5.7% | -13.7% | -15.3% |
|  | 3.1% | -0.8% | -1.2% | 5.7% | -13.3% | -14.9% |
| Class F | 6.9% | 4.6% | 6.9% | 13.0% | 3.3% | 7.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 2.0% | 2.1% | 4.2% |
| Class B | 4.0% | -2.8% | -3.8% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 3.1% | -0.6% | -0.3% |
|  | 3.1% | -0.6% | -0.2% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 3: BD-rate results for proposed default quantization matrix (test2) vs. flat matrix

# Observation

The crosschecker carefully investigated the provided software for subtest 2.2, and found that the software contains required modifications to perform Test1 and Test2 of subtest 2.2 only as described in CE4 description (JCTVC-G1204).

# Conclusion

It has been verified that the results of “Method and syntax for quantization matrices representation” provided as subtest 2.2 results (equivalent to the matrices coding results presented in JCTVC-H0384) are accurate.  Also, it has been verified that the software for subtest 2.2 only contains the tools presented in CE4 description (JCTVC-G1204).