| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0154 |
| --- | --- |

| Title: | Side information signaling improvement for TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bumshik Lee, Jaeil Kim and Munchurl Kim Guseong-dong, Yuseong-Gu, Daejeon, Korea | Tel: Email: | 82-42-350-7519 lbs@kaist.ac.kr, jaeil1203@kaist.ac.kr, mkim@ee.kaist.ac.kr |
| Source: | KAIST |  |  |

_____________________________

# Abstract

In this contribution, an efficient side information signaling method for TU, which is implemented on top of the platform of JCTVC-G442 is proposed. The proposed method of signaling side information for TU is to alter the coding order of cbf and split_transform_flag for luma. Hence, split_transform_flag is coded only when non-zero coefficients do not exist in the current or lower depth of TU. The proposed method is applied to the inter-coded CU. Unlike the HM 5.0 and JCTVC-G442, the decoding for split_transform_flag is not necessary when cbf = 0, leading to decoding complexity reduction. The proposed method achieves a comparable coding efficiency compared to G442 with reduced decoding time ranging from 4% to 7%.

# Introduction

In JCTVC-G442, an improved scheme to chroma TU specification was proposed in the 7th JCT-VC meeting [1]. The split_trasnform_flag is only used for luma and a split flag for chroma TU is inferred from prediction modes. Although the scheme achieves a coding efficiency improvement, it can further be improved in signaling side information for TU. In this contribution, a side information signalling method for the split_transform_flag and cbf for luma is proposed.

# Proposed method

In this contribution, the split_transform_flag for luma is proposed to be coded only when cbf takes on a non-zero value. In order to do that, the coding order of cbf and split_transform_flag is changed each other. Thus, when the TU is decided to be not in the minimum size defined in the SPS, the required bits for coding and the resulting decoding time can be saved. Unlike the HM 5.0 and JCTVC-G442, the decoding for split_transform_flag is not necessary when cbf = 0, leading to decoding complexity reduction.

# Experimental results

The proposed method is implemented in HM5.0 including JCTVC-G442. In CE2 description [2], three experimental conditions are given. The experimental results in Table 1 are obtained under the condition of 4.4 with QuadtreeTUMaxDepthInterChroma 2, QuadtreeTUMaxDepthInterChroma 1 in cfg file and #define CHROMA_32x32_TU_OFF  0 (in TypeDef.h), respectively.

Table 1. Performance of proposed method against JCTVC-G442 in HM5.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | -0.1% | -0.1% | -0.1% |
| Class E | - | - | - | - | - | - |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | - | - | - | - | - | - |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | 0.2% | 0.0% |
| Class D | 0.0% | -0.2% | 0.7% | 0.0% | 0.4% | 0.3% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | 0.1% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 94% |  |  | 93% |  |  |

# Conclusions

In this contribution, an improved side information signaling method for TU is proposed. In the proposed method, the coding order of cbf and split_transform_flag is changed and split_transform_flag is only coded when cbf is a non-zero value. The proposed method shows comparable coding efficiency with the method in JCTVC-G442, reducing the decoding time from 4% up to 7%.

# References

- A. Minezawa, K. Sugimoto, S. –I. Sekiguchi, A. Ichigaya and S. Sakaida, “Improvement to chroma TU specification”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G442, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- K. Sugimoto, X. Guo and X. Wang, “Description of Core Experiment 2 (CE2): Chroma RQT depth,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1202, 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

KAIST may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).