| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0156 |
| --- | --- |

| Title: | CE10.3: Simplification of Second Delta Calculation in Deblocking by SKT/SKKU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungyoup Yang Jaehwan Kim Kwanghyun Won Byeungwoo Jeon Sungkyunkwan University 300 Chunchun-dong, Jangan-gu, Suwon, Korea Jeongyeon Lim SK Telecom 9-1 Sunae-dong, Bundang-gu, Sungnam, Korea | Tel: Email: | +82-31-290-7186 binbak1@ece.skku.ac.kr +82-31-290-7186 kjh4759@skku.edu +82-31-290-7186 wkh12345@skku.edu +82-31-290-7144 bjeon@skku.edu +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | SK Telecom and Sungkyunkwan University |  |  |

_____________________________

# Abstract

This contribution addresses simplification of the deblocking filter by simpler calculation of the second delta value. Simulation results show that the BD-rate results and subjective quality are almost similar to the HM5.0 anchor with added benefit of decrement in decoding execution time.

# Introduction

The current HM5.0 uses different types of filter according to pixel positions from the filtering boundary.

The following is the calculation of luma weak filter in deblocking in the HM5.0.

- For the first pixel

 = Clip( –tc, tc, (( 9ⅹ(q0 – p0) – 3ⅹ(q1 – p1) + 8) / 16))

p0’ = p0 + 

q0’ = q0 – 

- For the second pixel p1 (If dEp1 is equal to 1)

p = Clip( –(tc >> 1), (tc >> 1), ((( p2 + p0 + 1) >> 1) – p1 + ) >> 1)

p1’ = p1 + p

- For the second pixel q1 (If dEq1 is equal to 1)

q = Clip( –(tc >> 1), (tc >> 1), ((( q2 + q0 + 1) >> 1) – q1 – ) >> 1)

q1’ = q1 + q

# Proposed encoder description

Based on the previous contribution [1], this contribution addresses simplification of the deblocking filter by simpler calculation of the second delta value. The delta values for the second pixels from the filtering boundary are calculated by half of the delta value which is calculated for the first pixel at the filtering boundary without additional filtering processes for the second pixels.

The following is the calculation of luma weak filter in deblocking in this proposal.

- For the first pixel (no change from HM5.0)

 = Clip( –tc, tc, (( 9ⅹ(q0 – p0) – 3ⅹ(q1 – p1) + 8) / 16))

p0’ = p0 + 

q0’ = q0 – 

- For the second pixel p1 (If dEp1 is equal to 1) (changed from HM5.0)

p1’ = p1 + /2

- For the second pixel q1 (If dEq1 is equal to 1) (changed from HM5.0)

q1’ = q1 – /2

# Experimental results

The proposed method is implemented on HM5.0 software [2]. Table 1 shows the summarized results of the proposed method compared to the HM5.0 anchor. The simulations are performed under the common test conditions in JCTVC-G1200 [3]. Detailed results are included in JCTVC-H0156.xls.

Table 1. Experimental results of the proposed method compared to the HM5.0 anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.2% | 0.1% | -0.3% | -0.2% |
| Class C | -0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | -0.1% | -0.6% | 0.1% | 0.0% | 0.4% | 0.2% |
| Class E | -0.1% | 0.6% | 0.1% | 0.2% | -0.7% | 0.0% |
| Overall | -0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% |
|  | -0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class F | -0.1% | 0.1% | -0.3% | -0.1% | -0.6% | -0.5% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.8% | 0.1% | -0.6% |
| Class C | -0.1% | -0.1% | -0.3% | 0.4% | 0.1% | 0.3% |
| Class D | 0.0% | -0.1% | 0.0% | 0.2% | -0.2% | 0.1% |
| Class E | -0.1% | 0.3% | 0.2% | 1.2% | -0.9% | -0.8% |
| Overall | -0.1% | 0.0% | 0.0% | 0.5% | -0.2% | -0.3% |
|  | -0.1% | 0.0% | 0.0% | 0.5% | -0.2% | -0.2% |
| Class F | -0.2% | -0.1% | 0.1% | 0.2% | -0.5% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusion

This contribution addresses simplification of the deblocking filter by simpler calculation of the second delta value. Simulation results show that the BD-rate results and subjective quality are almost similar to the HM5.0 anchor with the decreased decoding execution time.

# Working draft text modifications

– Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:  = ( 9 * ( q0 – p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4 (8-1) - When abs() is less than tc*10, the following ordered steps apply: - The filtered sample values p0’ and q0’ are specified as follows:  = Clip3( -tc, tc,  ) (8-2) p0’ = Clip1Y( p0 +  ) (8-3) q0’ = Clip1Y( q0 -  ) (8-4) - If dEp1 is equal to 1, the filtered sample value pi’ is specified as follows: p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 ) (8-5) pi’ = Clip1Y( p1 + p ) (8-6) pi’ = Clip1Y( p1 + /2 ) (8-7) - If dEq1 is equal to 1, the filtered sample value qi’ is specified as follows: q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 ) (8-8) qi’ = Clip1Y( q1 + q ) (8-9) qi’ = Clip1Y( q1 - /2 ) (8-10) - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

# References

[1] J. Yang, K. Won, B. Jeon, and J. Lim, “CE12: SK Telecom/SKKU Deblocking Filter,” JCTVC-F258, Torino, IT, July 2011.

[2] Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, and Thomas Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCTVC-G1103, Geneva, CH, Nov. 2011.

[3] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-G1200, Geneva, CH, Nov. 2011.

# Patent right declaration(s)

SK Telecom and Sungkyunkwan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).