| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0160 WG11 Number: m23032 |
| --- | --- |

| Title: | CE11:Context reduction for coefficient level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kengo Terada Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 { terada.kengo, sasai.hisao, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This proposal presents a technique for context reduction of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. In this proposal, if context set index is equal to 1, 2, 4 or 5, the contexts for each coeff_abs_level_greater1/2_flag are reduced by changing the threshold of c1 and c2. c1 threshold is changed from 3 to 2. c2 threshold is changed from 2 to 1. This proposal was implemented on HMv5.0 and evaluated in the CE11. Contexts of coeff_abs_level_greater1/2_flag were reportedly reduced from 56 to 46 without major impact on coding efficiency.

# Introduction

We investigated the number of context used on HMv5.0 common condition test [1]. Table.1 shows the investigation result. This proposal is to reduce the context of coeff_abs_level_greater1/2_flag.

Table.1 : The number of context used on HMv5.0 common condition test

# Proposed Solution

## Reducing context

In the current HEVC, There are luma 6 context set and chroma 2 context set for each coeff_abs_level_greater1/2_flag. coeff_abs_level_greater1_flag context set consists of 4 contexts. coeff_abs_level_greater2_flag context set consists of 3 contexts. The index of the context within coeff_abs_level_greater1_flag context set, c1, is determined based on the trailing ones to a maximum of 3. And, the index of the context within coeff_abs_level_greater2_flag context set, c2, is determined based on the number of coeff_abs_level_greater1_flag to a maximum of 2.

In this proposal, if context set index, ctxSet, is equal to 1, 2, 4 or 5, the contexts for coeff_abs_level_greater1/2_flag are each reduced by changing the threshold of c1 and c2. c1 threshold is changed from 3 to 2. c2 threshold is changed from 2 to 1.

Table.2 : Example of context derivation for coeff_abs_level_greater1/2_flag in case of ctxSet 1, 2, 4, or 5

Fig.1 shows the flow chart for coeff_abs_level_greater1_flag decoding. The operation for condition judgment is unnecessary to change c1 threshold because it is set with setting of ctxSet. It is similar about c2 threshold change. Therefore this proposal can be realized by addition of slight operation.

Fig.1 : Flow chart for coeff_abs_level_greater1_flag decoding

## The benefit of proposed solution

This proposed solution can reduce the memory size for coefficient level contexts as shown in Table.3 and 4. In the system pre-loading context to cache memories, this proposed solution can reduce pre-loading data by the number of context within ctxSet decreasing.

Table.3 : The number of context within each ctxSet for coeff_abs_level_greater1/2_flag

Table.4 : The number of overall context used on HMv5.0 + Proposal common condition test

# Results

The proposed method performance relative to the HMv5.0 software was checked under the common test condition described in JCTVC-G1200 [1]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. The cross verification is performed by TI.

Table.5 : BD BR for proposed solution ( Common Conditions )

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | 0.5% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.2% | 0.0% | 0.5% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class F | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.2% | 0.6% |  |  |  |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.2% | -0.5% | 0.0% | 0.7% | 0.2% |  |  |  |
| Class E | 0.2% | 0.7% | -0.3% | -0.1% | -0.2% | -0.7% |  |  |  |
| Overall | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
|  | 0.0% | 0.2% | -0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | -0.2% | -0.4% | -0.6% | 0.1% | -0.8% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table.6 : BD BR for proposed solution ( Low QP (12, 17, 22, 27) )

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class E | 0.0% | 0.1% | -0.1% | -0.1% | -0.6% | -0.5% |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class F | -0.1% | 0.0% | -0.2% | 0.2% | 0.3% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

# Conclusion

In this contribution, a method for context reduction is proposed. Contexts for coeff_abs_level_greater1/2_flag were reportedly reduced from 56 to 46, which is about 5% reduction relative to the total number of contexts. This proposed solution can reduce the memory size for coefficient level contexts without major impact on coding efficiency. It is suggested to consider the inclusion of this proposal in the HM/CD of HEVC.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC 7th Meeting, Geneva, November, 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text for Working Draft

Table 9-7 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| …. |  |  |  |  |  |
| residual_coding() | last_significant_coeff_x_prefix | Table 9-40 | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Table 9-26 | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Table 9-41 | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Table 9-44 | 0..31 0..26 | 32..63 27..53 | 64..95 54..80 |
|  | coeff_abs_level_greater2_flag | Table 9-45 | 0..23 0..18 | 24..47 19..37 | 48..71 38..56 |

Table 9-29 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 7 | 9 8 | 10 9 | 11 | 12 10 | 13 11 | 14 12 | 15 13 |
| initValue | 104 | 68 | 116 | 86 | 104 | 132 | 86 | 87 | 105 | 134 | 87 | 103 | 102 | 66 | 114 | 68 |
|  | 16 14 | 17 15 | 18 16 | 19 | 20 17 | 21 18 | 22 19 | 23 | 24 20 | 25 21 | 26 22 | 27 23 | 28 24 | 29 25 | 30 26 | 31 |
| initValue | 87 | 84 | 100 | 101 | 72 | 69 | 101 | 86 | 104 | 130 | 147 | 149 | 104 | 196 | 100 | 165 |
|  | 32 27 | 33 28 | 34 29 | 35 30 | 36 31 | 37 32 | 38 33 | 39 | 40 34 | 41 35 | 42 36 | 43 | 44 37 | 45 38 | 46 39 | 47 40 |
| initValue | 119 | 179 | 179 | 164 | 119 | 85 | 117 | 149 | 136 | 103 | 103 | 103 | 133 | 98 | 114 | 115 |
|  | 48 41 | 49 42 | 50 43 | 51 | 52 44 | 53 45 | 54 46 | 55 | 56 47 | 57 48 | 58 49 | 59 50 | 60 51 | 61 52 | 62 53 | 63 |
| initValue | 118 | 99 | 115 | 116 | 87 | 100 | 85 | 117 | 135 | 146 | 147 | 164 | 119 | 148 | 116 | 133 |
|  | 64 54 | 65 55 | 66 56 | 67 57 | 68 58 | 69 59 | 70 60 | 71 | 72 61 | 73 62 | 74 63 | 75 | 76 64 | 77 65 | 78 66 | 79 67 |
| initValue | 119 | 179 | 148 | 164 | 119 | 85 | 117 | 149 | 136 | 87 | 103 | 103 | 133 | 98 | 114 | 115 |
|  | 80 68 | 81 69 | 82 70 | 83 | 84 71 | 85 72 | 86 73 | 87 | 88 74 | 89 75 | 90 76 | 91 77 | 92 78 | 93 79 | 94 80 | 95 |
| initValue | 118 | 99 | 115 | 100 | 87 | 84 | 85 | 85 | 135 | 177 | 147 | 164 | 119 | 132 | 148 | 149 |

Table 9-30 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater2_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 5 | 7 6 | 8 | 9 7 | 10 8 | 11 9 |
| initValue | 86 | 103 | 73 | 102 | 103 | 73 | 103 | 88 | 89 | 115 | 117 | 103 |
|  | 12 10 | 13 11 | 14 | 15 12 | 16 13 | 17 | 18 14 | 19 15 | 20 16 | 21 17 | 22 18 | 23 |
| initValue | 117 | 118 | 103 | 102 | 103 | 72 | 101 | 103 | 104 | 101 | 167 | 121 |
|  | 24 19 | 25 20 | 26 21 | 27 22 | 28 23 | 29 | 30 24 | 31 25 | 32 | 33 26 | 34 27 | 35 28 |
| initValue | 84 | 102 | 88 | 117 | 118 | 104 | 103 | 119 | 136 | 83 | 116 | 118 |
|  | 36 29 | 37 30 | 38 | 39 31 | 40 32 | 41 | 42 33 | 43 34 | 44 35 | 45 36 | 46 37 | 47 |
| initValue | 100 | 117 | 87 | 85 | 86 | 103 | 84 | 118 | 120 | 117 | 150 | 120 |
|  | 48 38 | 49 39 | 50 40 | 51 41 | 52 42 | 53 | 54 43 | 55 44 | 56 | 57 45 | 58 46 | 59 47 |
| initValue | 84 | 102 | 88 | 117 | 118 | 104 | 87 | 119 | 136 | 83 | 116 | 118 |
|  | 60 48 | 61 49 | 62 | 63 50 | 64 51 | 65 | 66 52 | 67 53 | 68 54 | 69 55 | 70 56 | 71 |
| initValue | 84 | 117 | 87 | 69 | 86 | 87 | 84 | 118 | 120 | 117 | 150 | 120 |

| Table 9-31 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element |  |  | Type of binarization |  |  |  | maxBinIdxCtx | ctxIdxTable |  | ctxIdxOffset |  |  |  |
| .... |  |  |  |  |  |  |  |  |  |  |  |  |  |
| coeff_abs_level_greater1_flag |  | I | FL, cMax = 1 |  |  |  | 0 | Table 9-44 |  | 0 |  |  |  |
|  |  | P |  |  |  |  | 0 | Table 9-44 |  | 32 27 |  |  |  |
|  |  | B |  |  |  |  | 0 | Table 9-44 |  | 64 54 |  |  |  |
| coeff_abs_level_greater2_flag |  | I | FL, cMax = 1 |  |  |  | 0 | Table 9-45 |  | 0 |  |  |  |
|  |  | P |  |  |  |  | 0 | Table 9-45 |  | 24 19 |  |  |  |
|  |  | B |  |  |  |  | 0 | Table 9-45 |  | 48 38 |  |  |  |
| Table 9-37 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Syntax element | ctxIdxTable, ctxIdxOffset |  |  |  | binIdx |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 1 |  |  | 2 |  | 3 | >=4 |  |
| …. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| coeff_abs_level_greater1_flag | Table 9-44 |  |  | 0 | 0..31 0..26 (subclause 9.3.3.1.1.5) | na |  |  | na |  | na | na |  |
|  |  |  |  | 32 27 | 0..31 0..26 (subclause 9.3.3.1.1.5) | na |  |  | na |  | na | na |  |
|  |  |  |  | 64 54 | 0..31 0..26 (subclause 9.3.3.1.1.5) | na |  |  | na |  | na | na |  |
| coeff_abs_level_greater2_flag | Table 9-45 |  |  | 0 | 0..23 0..18 (subclause 9.3.3.1.1.6) | na |  |  | na |  | na | na |  |
|  |  |  |  | 24 19 | 0..23 0..18 (subclause 9.3.3.1.1.6) | na |  |  | na |  | na | na |  |
|  |  |  |  | 48 38 | 0..23 0..18 (subclause 9.3.3.1.1.6) | na |  |  | na |  | na | na |  |

###### 9.3.3.1.1.6 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the color component index cIdx, the 16 coefficient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

Table 9-42 – Specification of OneCtxSetMap[ i ]

| I | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| OneCtxSetMap | 0 | 4 | 7 | 10 | 14 | 17 |

….

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )	(9-1)

- The variable ctxSet is equal to 0 or 3, the following applies.

ctxIdxInc  =  OneCtxSetMap[ctxSet]  +  Min( 3, greater1Ctx )	(9-2)

- Otherwise (the variable ctxSet is equal to 1, 2, 4 or 5), the following applies.

ctxIdxInc  =  OneCtxSetMap[ctxSet]  +  Min( 2, greater1Ctx )	(9-3)

###### 9.3.3.1.1.7 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

Inputs to this process are the color component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

Table 9-43 – Specification of AbsCtxSetMap[ i ]

| I | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| AbsCtxSetMap | 0 | 3 | 5 | 7 | 10 | 12 |

….

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater2Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  3 )  +  Min( 2, greater2Ctx )	(9-47)

- The variable ctxSet is equal to 0 or 3, the following applies.

ctxIdxInc  =  AbsCtxSetMap[ctxSet]  +  Min( 2, greater2Ctx )	(9-57)

- Otherwise (the variable ctxSet is equal to 1, 2, 4 or 5), the following applies.

ctxIdxInc  =  AbsCtxSetMap[ctxSet]  +  Min( 1, greater2Ctx )	(9-68)