| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0161 |
| --- | --- |

| Title: | CE1: verification of C3 sub-test from HHI (JCTVC-H0266) by Samsung and Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Rajan Joshi, Joel Sole Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com +1-858-658-5673 {rajanj, joels}@qualcomm.com |
| Source: | Samsung Electronics Co. Ltd., Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, the performance of CE1 sub-test C3 is verified. Tests 4, 8, 9 and 10 were tested according to proponent’s request. Average Luma BD-rate gain in verified tests is 0.3% (test 4), 0.3% (test 8), 0.3% (test 9), and 0.3% (test 10). Increment of encoding and decoding times compared to the anchor is no more than 2%.

# Introduction

Code for the verified tests was provided by the proponent in time (according to [1]). Implementation in general matches with the description provided in [1] and [2].

In verified tests, the proponent modifies the probability estimation process by using exponentially decaying memory probability estimation model. Each model requires 12 bits, as opposed to 7 bits in HM5.0, to store the states. Also, compared to HM5.0 CABAC, longer distance prediction is used. Exponential memory decay can be characterized by number M of previously encoded bins which have an effect on the current bin probability estimation. In tests 9 and 10 proponent uses M=32 (which is longer than M19 in HM5.0 CABAC) and in tests 4 and 8, M=64 is used.

Probability is represented with 12 bits accuracy and initialized directly as 12 bits value. Entropy coder initialization was modified to output 12 bits value according to [1].

For interval sub-division, look-up table is used. Table shape and size is the same as in HM5.0 for tests 9 and 10 (64x4). The table content is modified because the probability grid is uniform in contrast to HM5.0 CABAC. Tests 4 and 8 use a table size of 16x8, which is half of HM5.0 table size. Accessing the look-up table requires mapping the 12 bit probability to the 64 or 16 state entries. For test 4 and 8, this mapping implies the following calculation

( ( m_state - 2048 ) ^ ( ( m_state >> 11 ) - 1 ) ) >> 7

Similar operations are required for the other tests. Probability up-date uses direct calculation instead of LUT in HM5.0 and AVC CABAC.

Test 8 and 10 are the same as tests 4 and 9, respectively, except that the initial values for probabilities are modified.

In all tests m_entropyBits table size used in the RD process is 4 times larger compared to HM5.0.

# Simulation results

We used s/w provided by proponent and ran it according to [3]. Summary of our verification tests is presented in Table 1. Luma BD-rate in Table 1 is averaged across mandatory tests only [3]. No performance drop in average for Chroma was observed in all the tests. More details about test data can be found in appendix (Tables 2-5) and excel spreadsheets attached to this contribution.

Table 1. Summary of verified tests (average Luma BD-rate gain is reported).

| Verified test | Test 4 | Test 8 | Test 9 | Test 10 |
| --- | --- | --- | --- | --- |
| M | 64 | 64 | 32 | 32 |
| Initialization Adjustment | no | yes | no | yes |
| sm_aucLPSTable size | 16x8 | 16x8 | 64x4 | 64x4 |
| m_entropyBits Size | 512 | 512 | 512 | 512 |
| AI | -0.5% | -0.5% | -0.4% | -0.4% |
| RA | -0.2% | -0.3% | -0.3% | -0.3% |
| LB | -0.1% | -0.1% | -0.3% | -0.2% |
| All | -0.3% | -0.3% | -0.3% | -0.3% |
| Enc. Time. | 101% | 101% | 101% | 102% |
| Dec. Time | 101% | 101% | 101% | 101% |

Our observations are:

- Longer distance prediction for probability estimation (M=64) is beneficial for Intra test.

- For Inter test cases, when number of bins to be encoded is significantly lower, shorter distance prediction (M=32) in probability estimation works better.

- Probability initialization adjustment doesn’t help much (sometimes causes performance degradation).

- Optional class F has losses of 0.4% for low delay B configuration

Overall performance compared to HM 5.0 was improved in all tests by about 0.3%. There is some reduction of LUT tables used in the decoder since m_aucNextStateMPS, m_aucNextStateLPS are not needed any more. As probability is represented with 12 bits precision, there is increase in memory usage for each context model (1 byte in HM5.0, 2 bytes in the proposed design).

# Conclusions

Samsung and Qualcomm confirm performance reported by HHI in tests 4, 8, 9 and 10, subtest C3 in CE1. On average tested tool provides 0.3% Luma BD-rate gain.

# References

- R. Joshi, E. Alshina, H. Sasai, H. Kirchhoffer, J. Lainema, “Description of Core Experiment 1: Entropy Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011

- J. Stegemann, H. Kirchhoffer, D. Marpe, T. Wiegand (Fraunhofer HHI), “Non-CE1: Counter-based probability model update with adapted arithmetic coding engine,” Document of Joint Collaborative Team on Video Coding, JCTVC-G547, November 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Appendix.

Table 2. Test 4 results (data provided by Qualcomm).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.68% | -1.38% | -1.05% | -0.66% | -1.00% | -0.99% |  |  |  |
| Class B | -0.47% | -0.95% | -0.83% | -0.43% | -0.88% | -0.94% |  |  |  |
| Class C | -0.45% | -0.83% | -0.63% | -0.37% | -0.68% | -0.67% |  |  |  |
| Class D | -0.59% | -0.34% | -0.27% | -0.52% | -0.73% | -0.68% |  |  |  |
| Class E | -0.40% | -1.29% | -1.03% | -0.39% | -1.20% | -1.15% |  |  |  |
| Overall | -0.50% | -0.89% | -0.72% | -0.45% | -0.87% | -0.86% |  |  |  |
|  | -0.50% | -0.87% | -0.69% | -0.45% | -0.85% | -0.86% |  |  |  |
| Class F | -0.17% | -0.27% | -0.04% | -0.14% | -0.01% | -0.34% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.62% | -1.09% | -1.02% | -0.66% | -0.99% | -0.99% | -0.57% | 0.62% | 0.75% |
| Class B | -0.23% | -0.63% | -0.41% | -0.28% | -0.69% | -0.81% | -0.18% | -0.65% | -0.33% |
| Class C | -0.06% | -0.22% | 0.13% | -0.09% | -0.54% | -0.53% |  |  |  |
| Class D | 0.06% | 0.49% | 0.73% | 0.03% | -0.15% | -0.25% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.16% | -0.28% | -0.04% | -0.20% | -0.55% | -0.61% | -0.35% | -0.08% | 0.15% |
|  | -0.15% | -0.27% | -0.03% | -0.20% | -0.54% | -0.63% | -0.05% | 0.01% | 0.25% |
| Class F | 0.17% | 0.36% | 0.57% | 0.13% | -0.17% | 0.00% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.23% | -0.22% | 0.09% | -0.47% | -0.68% | -0.04% |  |  |  |
| Class C | -0.12% | 0.09% | 0.47% | -0.23% | 0.10% | 0.23% |  |  |  |
| Class D | 0.10% | 1.08% | 1.20% | 0.10% | 0.81% | 0.94% |  |  |  |
| Class E | 0.13% | -0.52% | -0.71% | 0.13% | -1.17% | -0.77% |  |  |  |
| Overall | -0.05% | 0.13% | 0.31% | -0.15% | -0.20% | 0.14% |  |  |  |
|  | -0.04% | 0.17% | 0.29% | -0.15% | -0.24% | 0.10% |  |  |  |
| Class F | 0.43% | 0.55% | 0.86% | 0.16% | -0.27% | 0.32% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

Table 3. Test 8 results (data provided by Qualcomm).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.7% | -1.4% | -1.1% | -0.7% | -1.0% | -1.0% |  |  |  |
| Class B | -0.5% | -1.0% | -0.9% | -0.4% | -0.9% | -0.9% |  |  |  |
| Class C | -0.5% | -0.9% | -0.7% | -0.4% | -0.7% | -0.7% |  |  |  |
| Class D | -0.6% | -0.5% | -0.4% | -0.5% | -0.8% | -0.8% |  |  |  |
| Class E | -0.4% | -1.3% | -1.2% | -0.4% | -1.2% | -1.2% |  |  |  |
| Overall | -0.5% | -1.0% | -0.8% | -0.5% | -0.9% | -0.9% |  |  |  |
| Class F | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | -0.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.6% | -1.1% | -1.1% | -0.6% | -1.0% | -0.8% | -0.6% | 0.8% | 0.9% |
| Class B | -0.3% | -0.7% | -0.5% | -0.3% | -0.7% | -0.8% | -0.2% | -0.5% | -0.5% |
| Class C | -0.2% | -0.4% | -0.1% | -0.2% | -0.5% | -0.5% |  |  |  |
| Class D | -0.1% | 0.2% | 0.4% | -0.1% | -0.1% | -0.6% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.4% | -0.2% | -0.3% | -0.6% | -0.7% | -0.4% | 0.0% | 0.1% |
| Class F | 0.1% | 0.2% | 0.2% | 0.1% | -0.3% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 102% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | 0.0% | -0.4% | -0.4% | 0.5% |  |  |  |
| Class C | -0.1% | -0.1% | 0.3% | -0.2% | 0.0% | 0.2% |  |  |  |
| Class D | -0.1% | 0.8% | 0.2% | 0.0% | 1.1% | -0.1% |  |  |  |
| Class E | 0.3% | 0.7% | -1.8% | 0.3% | -1.3% | -1.0% |  |  |  |
| Overall | -0.1% | 0.2% | -0.2% | -0.1% | -0.1% | 0.0% |  |  |  |
| Class F | 0.4% | 0.6% | 0.5% | 0.5% | 0.7% | 0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

Table 4. Test 9 results (data provided by Samsung).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.7% | -1.4% | -1.0% | -0.7% | -1.0% | -1.0% |  |  |  |
| Class B | -0.5% | -1.0% | -0.8% | -0.4% | -0.9% | -0.9% |  |  |  |
| Class C | -0.5% | -0.8% | -0.6% | -0.4% | -0.7% | -0.7% |  |  |  |
| Class D | -0.6% | -0.3% | -0.3% | -0.5% | -0.7% | -0.7% |  |  |  |
| Class E | -0.4% | -1.3% | -1.0% | -0.4% | -1.2% | -1.2% |  |  |  |
| Overall | -0.5% | -0.9% | -0.7% | -0.5% | -0.9% | -0.9% |  |  |  |
| Class F | -0.2% | -0.3% | 0.0% | -0.1% | 0.0% | -0.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.6% | -1.1% | -1.0% | -0.7% | -1.0% | -1.0% | -0.6% | 0.6% | 0.7% |
| Class B | -0.2% | -0.6% | -0.4% | -0.3% | -0.7% | -0.8% | -0.2% | -0.6% | -0.3% |
| Class C | -0.1% | -0.2% | 0.1% | -0.1% | -0.5% | -0.5% |  |  |  |
| Class D | 0.1% | 0.5% | 0.7% | 0.0% | -0.1% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.3% | 0.0% | -0.2% | -0.5% | -0.6% | -0.4% | -0.1% | 0.1% |
| Class F | 0.2% | 0.4% | 0.6% | 0.1% | -0.2% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | 0.1% | -0.5% | -0.7% | 0.0% |  |  |  |
| Class C | -0.1% | 0.1% | 0.5% | -0.2% | 0.1% | 0.2% |  |  |  |
| Class D | 0.1% | 1.1% | 1.2% | 0.1% | 0.8% | 0.9% |  |  |  |
| Class E | 0.1% | -0.5% | -0.7% | 0.1% | -1.2% | -0.8% |  |  |  |
| Overall | -0.1% | 0.1% | 0.3% | -0.2% | -0.2% | 0.1% |  |  |  |
| Class F | 0.4% | 0.6% | 0.9% | 0.2% | -0.3% | 0.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

Table 5. Test 10 results (data provided by Samsung).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.5% | -0.7% | -0.5% | -0.5% | -0.5% | -0.5% |  |  |  |
| Class B | -0.4% | -0.5% | -0.4% | -0.4% | -0.5% | -0.5% |  |  |  |
| Class C | -0.4% | -0.6% | -0.4% | -0.3% | -0.4% | -0.4% |  |  |  |
| Class D | -0.4% | -0.2% | -0.2% | -0.4% | -0.5% | -0.5% |  |  |  |
| Class E | -0.4% | -0.4% | -0.3% | -0.4% | -0.5% | -0.5% |  |  |  |
| Overall | -0.4% | -0.5% | -0.4% | -0.4% | -0.5% | -0.5% |  |  |  |
| Class F | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% | -0.3% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% | -0.6% | -0.5% | 0.6% | 0.6% |
| Class B | -0.3% | -0.2% | -0.1% | -0.3% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| Class C | -0.2% | -0.4% | -0.1% | -0.2% | -0.3% | -0.3% |  |  |  |
| Class D | -0.2% | -0.3% | 0.2% | -0.2% | 0.2% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.1% | -0.3% | -0.1% | -0.2% | -0.3% | 0.2% | 0.1% |
| Class F | -0.1% | 0.0% | 0.1% | -0.2% | -0.2% | -0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.3% | 0.2% | 0.3% | -0.4% | 0.3% | 0.7% |  |  |  |
| Class C | -0.2% | -0.2% | 0.2% | -0.3% | 0.2% | 0.2% |  |  |  |
| Class D | -0.2% | 0.4% | 0.4% | -0.2% | 1.3% | -0.1% |  |  |  |
| Class E | 0.0% | 0.5% | -0.5% | -0.1% | -0.1% | 0.1% |  |  |  |
| Overall | -0.2% | 0.2% | 0.1% | -0.3% | 0.4% | 0.3% |  |  |  |
| Class F | -0.1% | 0.2% | 0.1% | 0.0% | -0.6% | 0.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |