| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0163 WG11 Number: m23035 |
| --- | --- |

| Title: | Non-CE8: Cross check report of JCTVC-H0136 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Shunsuke Iwamura NHK Science & Technology Research Laboratories 1-10-11 Kinuta, Setagaya-ku, Tokyo 157-8510, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This report describes the cross check results of JCTVC-H0136. JCTVC-H0136 reported subjective video quality degradation from the ALF virtual boundary introduced in HM 5.0. The subjective quality degradation was observed around virtual boundary at flat area in particular. The degradation was confirmed with an additional test sequence, which was not a common test sequences. Additionally, JCTVC-H0136 proposed two types of modified filtering. This report also performs as the cross check of the proposed filtering methods. The experiment results were identical to the proponents’ results and the subjective quality improvement was confirmed.

# Introduction

Sony tested the performance of HM5.0 with additional test sequences. The additional sequences are distributed from Institute of Image Information and Television Engineers [1]. These were made for evaluating video systems, which includes the video coding system. Therefore, it is easy to find subjective quality degradation with the sequences. They found the distortion around virtual boundary of JCTVC-G212 [2] in the sequence of “Fountain_Chromakey”. The distortion was analyzed in JCTVC-H0136 [3]. In this document, they proposed two solutions for improvement the subjective quality. We confirmed the distortion with the same sequence and conducted the cross verification of their proposals with common test sequences.

# Subjective quality evaluation

## Coding conditions and results

We conducted the encoding test with “Fountain_Chromakey” [4] [5]. The sequence is provided as 10 bit/ 4:2:2 format. Since a 4:2:2 sequence cannot be encoded with the current HM, we converted the sequence to 8 bit / 4:2:0 by the converter, which was provided from the proponents, in order to compare the results. The specification of the sequence is shown in table 1-1. The sequence was encoded under the conditions as shown in table 1-2. The experiment results are shown in table 1-3. The results were identical to the results of the proponents. We confirmed that the proponents’ experiments were conducted correctly.

Table 1-1 Specification of “Fountain_Chromakey”

| Resolution [pixel] | 1920x1080 |
| --- | --- |
| Color space | YCbCr 4:2:0 |
| Frame rate | 59.94p |
| Frame number | 180 |
| Bit depth | 8 |

Table 1-2 Encode condition of “Fountain_Chromakey”

| Test condition | LP-HE (Low delay, high efficiency, P slices only) |
| --- | --- |
| Frame number to be encoded | 180 |
| Frame rate | 60 |
| QP | 37 |
| SAO | ON and OFF |

Table 1-3 Encode result of “Fountain_Chromakey”

| Condition | SAO:ON |  |  |  | SAO:OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | Bitrate [Kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] | Bitrate [Kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| 22 | 72164.15 | 40.32 | 43.25 | 43.84 | 72417.33 | 40.30 | 43.18 | 43.73 |
| 27 | 38227.31 | 37.62 | 42.89 | 42.44 | 38240.67 | 37.57 | 42.81 | 42.32 |
| 32 | 18075.75 | 34.40 | 42.51 | 41.51 | 18061.35 | 34.33 | 42.26 | 41.28 |
| 37 | 6611.67 | 31.10 | 42.36 | 40.89 | 6600.07 | 31.01 | 42.00 | 40.63 |

## Subjective quality

The distortion was observed in the low luminance and flat area. The degradation was observed as luminance un-uniformity due to the filtering process around virtual boundary was switched according to the processing pixel position. The switching filtering process caused the signal un-uniformity. The distortion was observed as line-like noise (see figure 2-1). The distortion emphasized the block boundary. The degradation could be observed easier in the case of SAO off and high QP (see figure 2-2).

Figure 2-1 A cropped decoded picture of “Fountain_Chromakey” Frame #74

HM5.0, LowDelayP_HE, QP=37 case SAO ON

Figure 2-2 The cropped decoded picture of “Fountain_Chromakey” Frame #74

HM5.0, LowDelayP_HE, QP=37, SAO OFF

# Proposed method of JCTVC-H0136

The two methods were proposed as the solutions to this quality degradation in JCTVC-H0136. These proposals are the modification of star5x5 shape filtering. The cross9x9 shape filtering does not have any modification.

## Method1

Basically, this method is similar as current implementation of HM5.0. The outside of virtual boundary is padded with the zero-order hold data around the border. And the filtering uses the padding data for filtering instead of the pre-filtered pixel. However, the proposed process does not average the filtered pixel value and the pre-filtered pixel value. And when the processed pixel is around virtual boundary, though the filtering is skipped in HM5.0, this method applies the filtering to the pixels with the same way. From this modification, the procedure is integrated and the luminance un-uniformity is decreased. The source code is also cleaned up. The filtering process, which has three if-statement in HM5.0, was integrated.

## Method2

This method introduced the same concept as the cross9x9 shape filtering. In the case of cross9x9 shape filtering, when the reference pixel of filtering is over virtual boundary, the tap length are shorten within the available area length. The star5x5 shape filter is also shorten the tap length like that. Therefore, this method does not need padding data over virtual boundary. The concept of the star5x5 shape is integrated to the cross9x9 shape filtering.

# Objective evaluation

## Coding condition

All experiments were conducted on Linux 64 bit OS with gcc-4.1.2 with the same computer systems. Default conditions were according to JCEVC-G1200 [6].

## Experiment results

### The experiment results of method1

Table 4-1 BD result of Method 1

|  | All Intra HE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |  |  |  |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
|  | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |  |  |  |
|  | Y | U | V | Y | U |  | V |  |  |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% |  | -0.1% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  |  |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class D | 0.0% | 0.0% | -0.2% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  | -0.1% |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  | 0.0% |  |  |
| Class F | -0.1% | 0.0% | 0.0% |  |  |  |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |  |  |  |
|  | Y | U | V | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | -0.1% |  | 0.1% |  | 0.1% |  |
| Class C | 0.0% | -0.2% | 0.0% | 0.1% |  | 0.1% |  | 0.0% |  |
| Class D | 0.0% | -0.4% | 0.3% | 0.0% |  | 0.0% |  | -0.4% |  |
| Class E | -0.1% | 0.2% | -0.2% | -0.2% |  | -0.5% |  | -0.1% |  |
| Overall | 0.0% | -0.1% | 0.1% | -0.1% |  | 0.0% |  | -0.1% |  |
|  | 0.0% | -0.1% | 0.1% | -0.1% |  | -0.1% |  | -0.1% |  |
| Class F | -0.3% | -0.2% | -0.4% | 0.0% |  | 0.4% |  | 0.4% |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

### The experiment results of method2

Table 4-2 BD result of Method 2

|  | All Intra HE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |  |  |  |
| Class A (8bit) | 0.1% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class E | 0.0% | 0.1% | 0.1% |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
|  | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |  |  |  |
|  | Y | U | V | Y | U |  | V |  |  |
| Class A (8bit) | 0.3% | 0.0% | 0.0% | 0.3% | -0.1% |  | 0.0% |  |  |
| Class B | 0.1% | 0.0% | 0.1% | 0.2% | 0.0% |  | 0.0% |  |  |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class D | 0.0% | -0.2% | -0.1% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |  | 0.0% |  |  |
|  | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |  | 0.0% |  |  |
| Class F | -0.1% | -0.1% | -0.1% |  |  |  |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |  |  |  |
|  | Y | U | V | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% |  | 0.1% |  | 0.0% |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.1% |  | 0.0% |  | -0.2% |  |
| Class D | 0.1% | -0.4% | 0.3% | 0.0% |  | 0.5% |  | 0.0% |  |
| Class E | 0.6% | -0.1% | -0.1% | 0.8% |  | -0.1% |  | 0.1% |  |
| Overall | 0.2% | -0.1% | 0.2% | 0.2% |  | 0.1% |  | 0.0% |  |
|  | 0.2% | -0.1% | 0.1% | 0.2% |  | 0.1% |  | -0.1% |  |
| Class F | -0.2% | 0.0% | -0.5% | 0.1% |  | 0.3% |  | 0.3% |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |

## Summary of objective evaluation

The experiment results are identical to the results of the proponents. The results of method1 shows a small gain without a significant change in process time. The error of process time is within the HDD I/O speed fluctuation. It may be negligible. Theoretically, the number of if-statement is reduced and the complexity is also decreased. The results of medthod2 shows small losses. Because the method2 used the shorter tap filter around virtual boundary, it might cause some losses. The process time is almost the same.

# Subjective evaluation

The subjective evaluation was conducted with the encode results of “Fountain_Chromakey”. The coding condition is same as section 2.

## Coding results

The coding results are shown in table 5-1, 2, 3, 4. The results are identical to proponent’s results.

Table 5-1 Encode result of “Fountain_Chromakey” by proposed method 1 (SAO : ON)

| QP | Bitrate [kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| --- | --- | --- | --- | --- |
| 22 | 72174.00 | 40.30 | 43.25 | 43.84 |
| 27 | 38235.52 | 37.57 | 42.89 | 42.45 |
| 32 | 18077.34 | 34.37 | 42.50 | 41.50 |
| 37 | 6601.25 | 31.09 | 42.35 | 40.88 |

Table 5-2 Encode result of “Fountain_Chromakey” by proposed method 1 (SAO : OFF)

| QP | Bitrate [kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| --- | --- | --- | --- | --- |
| 22 | 72417.25 | 40.27 | 43.18 | 43.73 |
| 27 | 38240.35 | 37.52 | 42.82 | 42.31 |
| 32 | 18059.31 | 34.31 | 42.24 | 41.26 |
| 37 | 6591.26 | 31.00 | 42.01 | 40.63 |

Table 5-3 Encode result of “Fountain_Chromakey” by proposed method 2 (SAO : ON)

| QP | Bitrate [kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| --- | --- | --- | --- | --- |
| 22 | 72176.20 | 40.31 | 43.25 | 43.83 |
| 27 | 38228.57 | 37.60 | 42.89 | 42.45 |
| 32 | 18075.22 | 34.39 | 42.50 | 41.51 |
| 37 | 6606.48 | 31.09 | 42.34 | 40.86 |

Table 5-4 Encode result of “Fountain_Chromakey” by proposed method 2 (SAO : OFF)

| QP | Bitrate [kbps] | PSNR Y [dB] | PSNR U [dB] | PSNR V [dB] |
| --- | --- | --- | --- | --- |
| 22 | 72414.15 | 40.30 | 43.18 | 43.73 |
| 27 | 38236.49 | 37.56 | 42.82 | 42.31 |
| 32 | 18064.62 | 34.32 | 42.26 | 41.29 |
| 37 | 6583.71 | 31.00 | 42.01 | 40.62 |

## Subjective quality of the test sequence, “Fountain_Chromakey”

The degradation can not be observed in the same frame. The luminance un-uniformity from switching filtering is not almost observed.

Figure 5-3 The cropped decoded picture of “Fountain_Chromakey” Frame #74

HM5.0 with Method 1, LowDelayP_HE, QP=37, SAO ON

Figure 5-4 The cropped decoded picture of “Fountain_Chromakey” Frame #74

HM5.0 with Method 1, LowDelayP_HE, QP=37, SAO OFF

Figure 5-3 The cropped decoded picture of “Fountain_Chromakey” Frame #74

HM5.0 with Method 2, LowDelayP_HE, QP=37, SAO ON

Figure 5-4 The cropped decoded picture of “Fountain_Chromakey” Frame #74

HM5.0 with Method 2, LowDelayP_HE, QP=37, SAO OFF

## Subjective quality of the test sequence, “BQSquare”

Figure 5-5, 6, 7 respectively illustrate the parts of decoded picture of “BQSquare” of ClassD with HM5.0, HM5.0 with method1 and HM5.0 with method2. In this case, the distortion around virtual boundary is not almost observed because the sequence does not have a large flat area against the LCU size. However, the subjective quality is improved after applying the proposal methods. It is easy to find the difference when we see the top of the parasols of “BQSquare”. The pointed tips are not identifiable in figure 5-5, as they are cleary reconstructed in figure 5-6, 7.

Figure 5-5 The cropped decoded picture of “BQSquare” Frame #341

HM5.0, LowDelay_HE, QP=37, SAO ON

Figure 5-6 The cropped decoded picture of “BQSquare” Frame #341

HM5.0 with Method 1, LowDelay_HE, QP=37, SAO ON

Figure 5-7 The cropped decoded picture of “BQSquare” Frame #341

HM5.0 with Method 2, LowDelay_HE, QP=37, SAO ON

# Summary

The coding results perfectly matched to the proponents’ results. We confirmed the implementation in the source code. These proposals requests small modifications. Though the gains are small, the concepts were simple and consistency. In particular, in the case of method 1, the implementation is cleaner than current HM5.0 and the coding performance is improved.

# References

- The Institute of Image Information and Television Engineers, http://www.ite.or.jp/en/

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, I. S. Chong, M. Karczewicz, “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with Cross9x9,” Document of Joint Collaborative Team on Video Coding, JCTVC-G212 November 2011.

- O. Nakagami, M. Ikeda and T. Suzuki, “Non-CE8: Report on visual artifacts by ALF virtual boundary processing and proposal on the filtering process modification,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0136, February 2012.

- http://www.ite.or.jp/data/p_t/dl/emanual.pdf

- http://www.ite.or.jp/shuppan/sample/265-60p.mov

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.