| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0165 |
| --- | --- |

| Title: | CE1: verification of C2 sub-test from Canon (JCTVC-H0140) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Co. Ltd.. |  |  |

_____________________________

# Abstract

In this contribution performance of CE1 sub-test C2 is verified. In this test proponent removes look-up-table in multi-parameter probability up-date model. Average Luma BD-rate gain in verified tests is 0.7%. Average increment of encoding and decoding time compare to anchor is no more than 3% and 1% correspondently.

# Introduction

Implementation in general matches with description provided in [1] and [2]. Proponent re-uses multi-parameter probability estimation model from [3]. In interval sub-division replaces look-up table with explicit formula. It saves memory, since sm_aucLPSTable is not needed anymore. But instead multiplication is used (which can be not desirable for some application).

In order to prevent overflow which might happen if uiLPS is replaced by multiplication one condition check is needed per one bin encoding and 2 more condition checks are needed per each LPS up-date.

Code for verified test was provided by proponent 4 days after CE1 dead-line specified in [1] but in time enough to study it and verify performance.

# Simulation results

We used s/w provided by proponent and run it according to [3]. Summary of our verification tests is presented in Table 1. Luma BD-rate in Table 1 is averaged across mandatory tests only [3]. Performance of [2] is reported in Table 1 for comparison. More details about test data can be found in appendix (Tables 2) and excel spreadsheet attached to this contribution.

Table 1. Summary of verified test (average Luma BD-rate gain is reported).

|  | JCTVC-H0113 | Verified test |
| --- | --- | --- |
| AI | -0.8% | -0.8% |
| RA | -0.7% | -0.7% |
| LB | -0.6% | -0.6% |
| All | -0.7% | -0.7% |
| Enc. Time. | 103% | 103% |
| Dec. Time | 101% | 100% |

These test data prove that aucLPSTable [64][64] in [2] can be removed without performance drop.

# Conclusions

Samsung confirm performances reported by Canon for subtest C2 in CE1. In average tested tool provides 0.7% Luma BD-rate gain and proves that look up –table for interval sub-division can be replaced by direct computations without performance drop.

# References

- C. Rosewarne, M. Maeda (Canon), “Modified probability update and table removal for multi-parameter CABAC update,” Document of Joint Collaborative Team on Video Coding, JCTVC-G413, November 2011.

- R. Joshi, E. Alshina, H. Sasai, H. Kirchhoffer, J. Lainema, “Description of Core Experiment 1: Entropy Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011

- A. Alshin, E. Alshina, J.H. Park “CE1, subtest C4: Multi-parameter probability estimation for CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0112, February 2012

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1200, November 2011.

Appendix.

Table 2. Performance test results.

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.9% | -0.9% | -0.8% | -0.9% | -0.9% | -0.9% |  |  |  |
| Class B | -0.8% | -0.8% | -0.7% | -0.8% | -0.9% | -0.9% |  |  |  |
| Class C | -0.8% | -0.8% | -0.7% | -0.7% | -0.8% | -0.8% |  |  |  |
| Class D | -0.8% | -0.5% | -0.5% | -0.8% | -0.8% | -0.8% |  |  |  |
| Class E | -0.7% | -0.8% | -0.6% | -0.8% | -0.9% | -0.9% |  |  |  |
| Overall | -0.8% | -0.7% | -0.6% | -0.8% | -0.9% | -0.9% |  |  |  |
| Class F | -0.6% | -0.6% | -0.3% | -0.6% | -0.4% | -0.6% |  |  |  |
| Enc Time[%] | 105% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| Class A (8bit) | -0.9% | -0.6% | -0.9% | -0.9% | -1.0% | -1.2% | -0.9% | 0.6% | 0.4% |
| Class B | -0.7% | -0.6% | -0.5% | -0.7% | -0.9% | -1.0% | -0.7% | -0.6% | -0.5% |
| Class C | -0.6% | -0.6% | -0.4% | -0.6% | -1.0% | -1.0% |  |  |  |
| Class D | -0.5% | -0.4% | -0.3% | -0.6% | -0.7% | -0.9% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.7% | -0.6% | -0.5% | -0.7% | -0.9% | -1.0% | -0.8% | -0.1% | -0.1% |
| Class F | -0.5% | -0.4% | -0.3% | -0.6% | -0.5% | -0.7% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.7% | -0.1% | -0.2% | -0.8% | -0.9% | -0.5% |  |  |  |
| Class C | -0.5% | -0.2% | -0.2% | -0.6% | -0.5% | -0.7% |  |  |  |
| Class D | -0.4% | -0.4% | 0.3% | -0.4% | 0.2% | -0.2% |  |  |  |
| Class E | -0.3% | 0.3% | -0.3% | -0.6% | -0.4% | -0.3% |  |  |  |
| Overall | -0.5% | -0.1% | -0.1% | -0.6% | -0.4% | -0.4% |  |  |  |
| Class F | -0.5% | -0.5% | -0.6% | -0.5% | -0.8% | -0.5% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |