| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0168 WG11 Number: m23040 |
| --- | --- |

| Title: | Proposal of HEVC profile/level definitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com +81-6-6900-9689 {sasai.hisao,nishi.takahiro}@jp.panasonic.com |
| Source: | Sony Corporation, Panasonic Corporation |  |  |

_____________________________

# Abstract

HEVC will be promoted to CD stage in 8th JCT-VC meeting at San Jose and the AHG on profile/level was established in the last JCT-VC meeting. This contribution proposes the initial idea of HEVC profile / level definition. The points of the proposal are as follows.

- Level definitions

- General constraints

- Level specific constraints

- Others

The profile/level definitions should be included in CD and open issues should be studied further by FDIS.

# Profile

Number of conformance point should be as small as possible. The main profile should support

- 4:2:0 8 bit

- I/P/B slices

In AVC, too much flexibility is allowed in the spec while some of them are not used. And each application forum specifies its own constraints. Then implementer need to check both AVC spec and application spec. Too much flexibility should be avoid in HEVC profile definition.

In addition, only one package of tools should be included for one functionality, e.g. for parallel processing capability.

# Level definition

In AVC, many sub-levels are defined, however it should be clean up. The following is an initial idea of level definition for discussion during San Jose meeting.

| Level | Typical size | Max luma pixel rate (pel/sec) | Max luma frame size (pel) | Max bit rate (1000 bit/sec) | MinCR | Max DPB (number of frames) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TBD |  |  |  |  |  |  |
| 2 | 720x480@24p, 720x480@30p, 720x480@60i, 720x576@50i | 10,368,000 | 414,720 | 7,000 | 2 | 4 | 7,000 |
| 3 | 1280x720@60p, 1440x1088@60i, 1920x1088@24p, 1920x1088@30p, 1920x1088@60i | 62,668,800 | 2,088,960 | 30,000 | 4 | 4 | 30,000 |
| 4 | 1920x1088@60p, 2048x1088@60p | 133,693,440 | 2,228,224 | 40,000 | 4 | 4 | 40,000 |
| 5 | 3840x2160@24p, 3840x2160@30p, 4096x2160@24p, 4096x2160@30p | 265,420,800 | 8,847,360 | 100,000 | 8 | 4 | 100,000 |
| 6 | 3840x2160@60p, 4096x2160@60p | 530,841,600 | 8,847,360 | 150,000 | 8 | 4 | 150,000 |
| 7 | 7680x4320@30p | 995,328,000 | 33,177,600 | 200,000 | ? | 4 | 200,000 |
| 8 | 7680x4320@60p | 1,990,656,000 | 33,177,600 | 300,000 | ? | 4 | 300,000 |

Table 1: Level definitions

# General constraints

The following constraints should be included in main profile.

LCU size:

- Max LCU size is equal to 64x64

- Min LCU size is equal to 16x16

Slice constraints:

- Min slice granularity is equal to LCU size

- Each slice starts from LCU boundary

Max number of reference frames:

Max value of num_ref_frame is equal to 4 (may be 5, if necessary)
Number of PS:

- Max number of SPS is equal to 8

- Max number of PPS is equal to 16

- Max number of APS is equal to 8

CABAC constraints:

Some of the CABAC constraints in AVC increase encoder complexity, while decoder is not critical for such cases. The example is the max number of bits for each MB. Such unnecessary constraints should not be defined.

Parallel processing tool:

Considering the decoder implementation, too many parallel processing tools are not helpful.

- Entropy slice is simple tool to support parallel processing of entropy coding.

- If tile is supported, each tile should be completely independent. For example, in-loop filtering should be turned off at tile boundary, any slice should not continue beyond tile boundary, and so on.

Potential candidate constraint:

- Implementation is simpler if picture size is multiple of LCU. It could be beneficial such constraint in future.

# Level specific constraints

Memory bandwidth restrictions:

To reduce MC memory bandwidth, MC block size and/or MV numbers should be restricted.

- For 8x4, 4x8 block size, only uni-pred is allowed at least for HD and larger size.

- For 4K and larger, more constraint is necessary. Further study is necessary. JCTVC-H0221 proposes such constraints.

Vertical MV range:

- Restrictions of vertical MV range is not necessary.

# Conclusion

This contribution proposes HEVC profile/level definition. This should be discussed during San Jose meeting and profile/level definitions should be included in CD.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).