| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0169 |
| --- | --- |

| Title: | CE5.b: Transform skipping choices based on block parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Glenn Van Wallendael Sebastiaan Van Leuven, Jan De Cock, Rik Van de Walle Gaston Crommenlaan 8 P.O. Box 201 9050 Ghent-Ledeberg, Belgium | Tel: Email: | +32 93314957 glenn.vanwallendael@ugent.be |
| Source: | Ghent University - IBBT |  |  |

_____________________________

# Abstract

In this contribution a mechanism for selection of transform skip mode (TSM) depending on TU size is presented. TSM was introduced in JCTVC-F077 and it enables skipping transform on a row and/or a column of motion compensated residuals. The transform skip mode choice is signalled to the decoder where inverse transforms of rows/columns are performed or skipped.

In this contribution, TSM is enabled or disabled depending on the size of the TU. Results are presented when applying TSM on TU sizes smaller than or equal to 4x4, 8x8, or 16x16. For example, when TSM options are evaluated for 4x4 TU sizes, encoding time increases with 5% to 8%. Under this reduced encoding search, BD-rate can be reduced by 0.2% (RA_HE), 0.5% (RA_LC), 0.0% (RA_HE10), 0.2% (LB_HE), and 0.5%(LB_LC). Largest compressions gains can be found in class F sequences: 1.7% (RA_HE), 2.4% (RA_LC), 3.8% (LB_HE), and 4.8%(LB_LC).

# Introduction

Transform Skip Mode (TSM) defines transform skip in one or both directions on which a transform would be applied under common conditions. As illustrated in Figure 1.a), TSM covers the traditional approach where the transform is applied on both rows and columns of a block.

| a) 2D transform | b) transform on rows only | c) transform on columns only | d) no transform |
| --- | --- | --- | --- |

Figure 1. Transform choices enabled by the proposed transform skip mode

# Transform skipping based on TU size

In this contribution, TSM is enabled or disabled depending on the size of the TU. 
In three tests, the four different TSM possibilities (2D, row transform, column transform, and no transform) are evaluated in the encoder on the following sizes:

- TU width <= 4 and TU height <= 4

- TU width <= 8 and TU height <= 8

- TU width <= 16 and TU height <= 16

For other sizes than indicated, only the 2D transform option is evaluated and signalled.

# Results

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0,0% | -0,2% | -0,3% | 0,0% | -0,1% | 0,0% | 0,1% | 0,0% | -0,1% |
| Class B | -0,2% | 0,0% | 0,1% | -0,5% | 0,3% | 0,5% | -0,1% | 0,0% | 0,1% |
| Class C | -0,4% | -0,3% | -0,2% | -0,9% | -0,1% | 0,2% |  |  |  |
| Class D | -0,8% | -0,1% | -0,2% | -1,4% | 0,3% | 0,2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,4% | -0,1% | -0,1% | -0,8% | 0,1% | 0,3% | 0,0% | 0,0% | 0,0% |
|  | -0,4% | -0,1% | -0,1% | -0,8% | 0,1% | 0,2% | 0,0% | 0,0% | 0,0% |
| Class F | -1,9% | -1,0% | -1,0% | -2,6% | -1,4% | -1,5% |  |  |  |
| Enc Time[%] | 121% |  |  | 116% |  |  | 121% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0,2% | 0,4% | 0,5% | -0,8% | 0,0% | 0,8% |  |  |  |
| Class C | -0,7% | -0,3% | -0,2% | -1,4% | 0,1% | 0,5% |  |  |  |
| Class D | -1,2% | 0,4% | 0,3% | -2,0% | 1,6% | 1,5% |  |  |  |
| Class E | -0,1% | 0,3% | -0,1% | -0,9% | 1,3% | 1,9% |  |  |  |
| Overall | -0,5% | 0,2% | 0,2% | -1,3% | 0,7% | 1,1% |  |  |  |
|  | -0,5% | 0,2% | 0,1% | -1,2% | 0,7% | 1,0% |  |  |  |
| Class F | -4,3% | -2,9% | -2,9% | -5,7% | -4,0% | -3,5% |  |  |  |
| Enc Time[%] | 118% |  |  | 113% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |

Results from TSM when only applied to 16x16 TU size or smaller.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0,0% | 0,0% | -0,1% | 0,0% | 0,1% | 0,1% | 0,1% | -0,1% | 0,0% |
| Class B | -0,1% | 0,0% | 0,1% | -0,3% | 0,6% | 0,7% | -0,1% | 0,0% | 0,1% |
| Class C | -0,3% | -0,2% | -0,2% | -0,8% | 0,0% | 0,2% |  |  |  |
| Class D | -0,7% | -0,2% | -0,3% | -1,3% | 0,4% | 0,2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,3% | -0,1% | -0,1% | -0,7% | 0,3% | 0,4% | 0,0% | -0,1% | 0,0% |
|  | -0,3% | -0,1% | -0,1% | -0,7% | 0,3% | 0,3% | 0,0% | -0,1% | 0,0% |
| Class F | -1,8% | -1,0% | -1,0% | -2,5% | -1,3% | -1,4% |  |  |  |
| Enc Time[%] | 113% |  |  | 111% |  |  | 112% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0,1% | 0,2% | 0,3% | -0,4% | 0,4% | 1,0% |  |  |  |
| Class C | -0,5% | -0,4% | -0,3% | -1,1% | -0,1% | 0,5% |  |  |  |
| Class D | -0,9% | 0,1% | 0,2% | -1,8% | 2,2% | 2,0% |  |  |  |
| Class E | 0,0% | 0,4% | 0,4% | -0,2% | 0,7% | 1,0% |  |  |  |
| Overall | -0,4% | 0,1% | 0,2% | -0,9% | 0,8% | 1,1% |  |  |  |
|  | -0,4% | 0,0% | 0,1% | -0,9% | 0,8% | 1,1% |  |  |  |
| Class F | -4,0% | -2,7% | -2,5% | -5,1% | -2,8% | -3,0% |  |  |  |
| Enc Time[%] | 111% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |

Results from TSM when only applied to 8x8 TU size or smaller.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0,1% | 0,0% | 0,0% | 0,1% | 0,2% | 0,1% | 0,1% | 0,1% | 0,1% |
| Class B | -0,1% | -0,1% | 0,0% | -0,2% | 0,5% | 0,5% | 0,0% | 0,0% | 0,0% |
| Class C | -0,3% | -0,3% | -0,2% | -0,6% | 0,0% | 0,1% |  |  |  |
| Class D | -0,5% | -0,1% | -0,1% | -1,0% | 0,5% | 0,5% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,2% | -0,1% | -0,1% | -0,5% | 0,3% | 0,4% | 0,0% | 0,0% | 0,0% |
|  | -0,2% | -0,1% | -0,1% | -0,5% | 0,3% | 0,3% | 0,0% | 0,0% | 0,0% |
| Class F | -1,7% | -0,8% | -0,8% | -2,4% | -1,1% | -1,3% |  |  |  |
| Enc Time[%] | 108% |  |  | 106% |  |  | 108% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | -0,1% | -0,1% | 0,2% | 0,7% |  |  |  |
| Class C | -0,4% | -0,4% | -0,3% | -0,7% | 0,3% | 0,4% |  |  |  |
| Class D | -0,7% | -0,2% | 0,2% | -1,2% | 1,3% | 1,2% |  |  |  |
| Class E | 0,1% | 0,6% | -0,2% | 0,1% | 0,5% | 0,8% |  |  |  |
| Overall | -0,2% | -0,1% | -0,1% | -0,5% | 0,5% | 0,8% |  |  |  |
|  | -0,2% | -0,1% | -0,1% | -0,5% | 0,5% | 0,7% |  |  |  |
| Class F | -3,8% | -2,1% | -2,4% | -4,8% | -2,4% | -2,4% |  |  |  |
| Enc Time[%] | 106% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |

Results from TSM when only applied to 4x4 TU size.

# Test conditions

Our computing platform used for generating the results is shown below.

## Encoding

Super computer with 167 computing nodes (IBM iDataPlex dx360M2), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5520 (Intel Nehalem microarchitecture, 2.27 GHz, 8 MB L3 cache per quad-core chip), thus 8 cores / node

- memory: 12 GB RAM (DDR3 PC3-8500S 1067 MHz)

- storage (local): one 156 GB 3.5" 15k RPM SAS disk

and 194 computing nodes (IBM HS 21 XM blade), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5420 (Intel Core microarchitecture, 2.5 GHz, 6 MB L2 cache per quad-core chip), thus 8 cores / node

- memory: 16 GB RAM (DDR2 FB-DIMM PC-5300 CL5)

- storage (local): one 146 GB 2.5" 10k RPM SAS disk

The encoder executables were generated with g++ 4.1.2.

## Decoding

- CPU: quad-core Intel Core 2 Q6600 (Intel Core microarchitecture, 2.40GHz, 8MB L2 cache)

- Memory: 4 GB RAM (DDR2 PC2-6400 400MHz)

# Acknowledgement

The research activities described in this proposal were funded by Ghent University, the Interdisciplinary Institute for Broadband Technology (IBBT), the Institute for the Promotion of Innovation by Science and Technology in Flanders (IWT), the Fund for Scientific Research-Flanders (FWO-Flanders), and the European Union. Furthermore, this work was carried out using the Stevin Supercomputer Infrastructure at Ghent University.

# Patent rights declaration(s)

Ghent University - IBBT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).