| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H172 |
| --- | --- |

| Title: | CE6: Cross verification of results related to section CE6.A |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Christophe GISQUET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a summary of the cross-checks performed by Canon on CE6a subtests. The cross-check performance results reportedly match those provided by the proponents. The implemented algorithms are also reportedly in line with the proponents’ proposals.

# Source code analysis of proponents contributions to CE6a

## Cross-channel intra chroma residual prediction, JCTVC-G173

This method, presented in [2], V residuals are predicted by U residuals using a -1/2 ratio and taking into account pixel value range. The implementation was modified in the spirit of the original proposal by deactivating it when the chroma prediction mode is LM, LMA or LML.

## Chroma prediction based on residual luma samples, JCTVC-G346

This method, presented in [3], predicts chroma residuals with a linear model applied to (potentially) recomputed luma residuals.

According to the guidelines of CE6a, and compared to the original contribution, a simplification was added, in the shape of a fast estimation mode, and activated. The proponents declare it as less efficient, but no tests were performed to verify this claim. It was noted that it slightly changes the bitstream syntax compared to the original contribution.

## New modes for Chroma prediction (LML, LMA), JCTVC-G358

Two new chroma prediction modes, presented in [4], similar to the luma-base chroma prediction mode are introduced, using different sets of pixels. The configuration selected by the proponents is the one with in total 8 chroma modes. The implementation is strictly identical to the original proposal, except for the HM5.0-optimized mode codewords.

# Tests performed

Here is the list of cross-checks performed by Canon:

| Contribution | Tools | Proponent | Cross-check |
| --- | --- | --- | --- |
| JCTVC-G173 | Cross-channel intra chroma residual pred | Intel | HKUST Canon |
| JCTVC-G346 | Chroma pred based on residual luma samples | KDDI | HKUST Canon |
| Combination of G173, G346 | Cross-channel intra chroma residual pred Chroma pred based on residual luma | Intel, KDDI | Canon HKUST |
| Combination of G173, G358 | Cross-channel intra chroma residual pred New modes for Chroma pred (LML,LMA) | Intel, HKUST | Canon KDDI |
| Combination of G346, G358 | Chroma pred based on residual luma New modes for Chroma pred (LML,LMA) | KDDI, HKUST | Canon Intel |
| Combination of G173, G346, G358 | Cross-channel intra chroma residual pred; Chroma pred based on residual luma New modes for Chroma pred (LML,LMA) | Intel, KDDI, HKUST | Canon Huawei |

As dictated by common test conditions [1], tests where JCTVC-G.358 is active are not involving AILC, as the LM mode is deactivated in that case.

# Results

## JCTVC-G173

Results for AIHE and AILC are:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.4% | 0.1% | -3.9% | 0.0% | 0.1% | -1.6% |
| Class B | 0.0% | 0.1% | -2.7% | 0.4% | 0.8% | 0.5% |
| Class C | -0.5% | -1.3% | -3.2% | -0.6% | -1.4% | -3.1% |
| Class D | -0.5% | -1.4% | -3.3% | -0.6% | -1.3% | -3.6% |
| Class E | -0.2% | -0.2% | -2.6% | -0.3% | -0.7% | -2.3% |
| Overall | -0.3% | -0.6% | -3.1% | -0.2% | -0.5% | -1.9% |
|  | -0.3% | -0.6% | -3.0% | -0.2% | -0.5% | -1.9% |
| Class F | -0.5% | -1.4% | -2.0% | -0.6% | -1.1% | -3.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The results when encoding all sequences are:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.4% | -2.2% | 0.2% | 0.3% | -1.0% | -0.2% | 0.5% | -2.6% |
| Class B | 0.0% | 0.1% | -2.7% | 0.4% | 0.8% | 0.5% | -0.1% | 0.0% | -3.1% |
| Class C | -0.5% | -1.3% | -3.2% | -0.6% | -1.4% | -3.1% | -0.5% | -1.3% | -3.3% |
| Class D | -0.5% | -1.4% | -3.3% | -0.6% | -1.3% | -3.6% | -0.5% | -1.4% | -3.3% |
| Class E | -0.2% | -0.2% | -2.6% | -0.3% | -0.7% | -2.3% | -0.2% | -0.4% | -3.1% |
| Class F | -0.5% | -1.4% | -2.0% | -0.6% | -1.1% | -3.1% | -0.5% | -1.3% | -2.2% |
| Overall | -0.3% | -0.6% | -2.7% | -0.1% | -0.4% | -1.8% | -0.3% | -0.6% | -2.9% |
|  | -0.3% | -0.6% | -2.7% | -0.1% | -0.4% | -1.7% | -0.3% | -0.6% | -2.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

## JCTVC-G346

It was decided among the CE6a participants to evaluate this contribution on AILC also.

Results are thus:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -2.3% | -1.6% | -0.3% | -4.3% | -2.7% |
| Class B | 0.0% | -3.2% | -2.6% | -0.3% | -6.0% | -3.8% |
| Class C | -0.2% | -2.9% | -3.4% | -0.5% | -5.0% | -5.7% |
| Class D | -0.1% | -2.6% | -2.8% | -0.3% | -4.2% | -4.5% |
| Class E | 0.0% | -1.3% | -1.2% | 0.0% | -2.1% | -2.0% |
| Overall | -0.1% | -2.6% | -2.5% | -0.3% | -4.5% | -3.9% |
|  | -0.1% | -2.6% | -2.4% | -0.3% | -4.5% | -3.9% |
| Class F | -0.5% | -3.4% | -3.9% | -1.2% | -6.3% | -7.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

For all sequence, they are:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -11.9% | -13.3% | -1.3% | -20.3% | -17.7% | -0.2% | -12.1% | -13.7% |
| Class B | 0.0% | -3.2% | -2.6% | -0.3% | -6.0% | -3.8% | 0.0% | -3.5% | -2.9% |
| Class C | -0.2% | -2.9% | -3.4% | -0.5% | -5.0% | -5.7% | -0.2% | -3.1% | -3.5% |
| Class D | -0.1% | -2.6% | -2.8% | -0.3% | -4.2% | -4.5% | -0.1% | -2.8% | -2.9% |
| Class E | 0.0% | -1.3% | -1.2% | 0.0% | -2.1% | -2.0% | 0.0% | -1.6% | -1.5% |
| Class F | -0.5% | -3.4% | -3.9% | -1.2% | -6.3% | -7.3% | -0.5% | -3.3% | -4.0% |
| Overall | -0.2% | -4.3% | -4.6% | -0.5% | -7.7% | -6.8% | -0.2% | -4.5% | -4.8% |
|  | -0.2% | -4.2% | -4.5% | -0.5% | -7.7% | -6.8% | -0.2% | -4.4% | -4.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

## JCTVC-G173 + JCTVC-G346

For AILC and AIHE:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.4% | -2.2% | -5.0% | -0.5% | -4.2% | -5.4% |
| Class B | -0.1% | -3.1% | -4.9% | -0.1% | -5.6% | -4.4% |
| Class C | -0.6% | -4.1% | -6.1% | -0.9% | -5.9% | -8.1% |
| Class D | -0.6% | -3.9% | -5.6% | -0.8% | -5.3% | -7.5% |
| Class E | -0.3% | -1.7% | -3.7% | -0.3% | -2.7% | -4.3% |
| Overall | -0.4% | -3.2% | -5.2% | -0.5% | -5.0% | -6.0% |
|  | -0.4% | -3.1% | -5.1% | -0.5% | -4.9% | -5.9% |
| Class F | -0.9% | -4.4% | -5.6% | -1.7% | -6.9% | -9.6% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 105% |  |  | 103% |  |  |

For all sequences:

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -11.5% | -15.2% | -1.4% | -20.2% | -18.8% | -0.4% | -11.7% | -16.1% |
| Class B | -0.1% | -3.1% | -4.9% | -0.1% | -5.6% | -4.4% | -0.1% | -3.5% | -5.6% |
| Class C | -0.6% | -4.1% | -6.1% | -0.9% | -5.9% | -8.1% | -0.6% | -4.2% | -6.4% |
| Class D | -0.6% | -3.9% | -5.6% | -0.8% | -5.3% | -7.5% | -0.5% | -4.1% | -5.8% |
| Class E | -0.3% | -1.7% | -3.7% | -0.3% | -2.7% | -4.3% | -0.3% | -1.9% | -4.3% |
| Class F | -0.9% | -4.4% | -5.6% | -1.7% | -6.9% | -9.6% | -0.9% | -4.5% | -5.8% |
| Overall | -0.5% | -4.8% | -6.9% | -0.7% | -8.1% | -8.6% | -0.5% | -5.1% | -7.4% |
|  | -0.5% | -4.8% | -6.8% | -0.7% | -8.0% | -8.5% | -0.5% | -5.0% | -7.3% |
| Enc Time[%] | 101% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 105% |  |  | 103% |  |  | 101% |  |  |

## JCTVC-G173 + JCTVC-G358

For AIHE only, as LM mode is only active in AIHE:

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.4% | -2.1% | -5.3% |
| Class B | -0.2% | -2.7% | -4.8% |
| Class C | -0.6% | -3.4% | -5.7% |
| Class D | -0.5% | -3.3% | -5.2% |
| Class E | -0.3% | -1.9% | -4.3% |
| Overall | -0.4% | -2.8% | -5.1% |
|  | -0.4% | -2.8% | -5.0% |
| Class F | -0.8% | -3.6% | -4.7% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 100% |  |  |

For all sequences:

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -8.6% | -11.4% | -0.4% | -8.7% | -12.0% |
| Class B | -0.2% | -2.7% | -4.8% | -0.2% | -3.0% | -5.2% |
| Class C | -0.6% | -3.4% | -5.7% | -0.6% | -3.5% | -5.8% |
| Class D | -0.5% | -3.3% | -5.2% | -0.6% | -3.4% | -5.3% |
| Class E | -0.3% | -1.9% | -4.3% | -0.3% | -2.0% | -4.7% |
| Class F | -0.8% | -3.6% | -4.7% | -0.8% | -3.6% | -5.0% |
| Overall | -0.5% | -4.0% | -6.0% | -0.5% | -4.1% | -6.4% |
|  | -0.5% | -3.9% | -6.0% | -0.5% | -4.0% | -6.3% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## JCTVC-G358 + JCTVC-G346

For AIHE only, as LM mode is only active in AIHE:

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.1% | -3.5% | -2.8% |
| Class B | -0.1% | -4.6% | -3.9% |
| Class C | -0.3% | -4.4% | -5.3% |
| Class D | -0.2% | -3.9% | -4.3% |
| Class E | -0.1% | -2.7% | -2.6% |
| Overall | -0.2% | -4.0% | -4.0% |
|  | -0.2% | -3.9% | -3.9% |
| Class F | -0.7% | -4.8% | -5.6% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 101% |  |  |

For all sequences:

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -14.1% | -15.9% | -0.3% | -14.2% | -16.2% |
| Class B | -0.1% | -4.6% | -3.9% | -0.1% | -4.8% | -4.1% |
| Class C | -0.3% | -4.4% | -5.3% | -0.3% | -4.5% | -5.4% |
| Class D | -0.2% | -3.9% | -4.3% | -0.2% | -4.1% | -4.4% |
| Class E | -0.1% | -2.7% | -2.6% | -0.1% | -2.7% | -2.8% |
| Class F | -0.7% | -4.8% | -5.6% | -0.7% | -4.8% | -5.7% |
| Overall | -0.3% | -5.8% | -6.3% | -0.3% | -5.9% | -6.5% |
|  | -0.3% | -5.7% | -6.2% | -0.3% | -5.9% | -6.4% |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

## JCTVC-G173 + JCTVC-G358+ JCTVC-G346

For AIHE only, as LM mode is only active in AIHE:

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.4% | -3.5% | -6.2% |
| Class B | -0.2% | -4.7% | -6.3% |
| Class C | -0.7% | -5.5% | -7.7% |
| Class D | -0.6% | -5.1% | -6.9% |
| Class E | -0.3% | -2.8% | -5.0% |
| Overall | -0.4% | -4.5% | -6.5% |
|  | -0.4% | -4.4% | -6.4% |
| Class F | -1.0% | -5.8% | -7.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 100% |  |  |

For all sequences:

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -13.9% | -17.7% | -0.5% | -14.0% | -18.5% |
| Class B | -0.2% | -4.7% | -6.3% | -0.2% | -4.9% | -6.7% |
| Class C | -0.7% | -5.5% | -7.7% | -0.6% | -5.6% | -8.0% |
| Class D | -0.6% | -5.1% | -6.9% | -0.6% | -5.3% | -7.0% |
| Class E | -0.3% | -2.8% | -5.0% | -0.3% | -3.1% | -5.5% |
| Class F | -1.0% | -5.8% | -7.1% | -1.0% | -5.9% | -7.4% |
| Overall | -0.5% | -6.4% | -8.5% | -0.5% | -6.6% | -8.9% |
|  | -0.5% | -6.3% | -8.4% | -0.5% | -6.5% | -8.8% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusion

The results we measured match those provided by the proponents.

# References

- F. Bossen, “Common conditions and software reference configurations,” in JCTVC-G1200, Geneva, Switzerland, Nov., 2011.

- Y. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel), “Cross-channel intra chroma residual prediction” in JCTVC-G173, Geneva, Switzerland, Nov., 2011

- K. Kawamura, T. Yoshino, H. Kato, S. Naito (KDDI), “Chroma intra prediction based on residual luma samples” in JCTVC-G346, Geneva, Switzerland, Nov., 2011

- X. Zhang, O. C. Au, J. Dai, F. Zou, C. Pang, X. Wen (HKUST), “New modes for chroma intra prediction” in JCTVC-G346, Geneva, Switzerland, Nov., 2011