| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H174 |
| --- | --- |

| Title: | Results on subset e.2 of CE8 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Tangi POIRIER Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents the results of the section CE8.e.2 regarding the modification of the Sample Adaptive Offsets (SAO) presented in JCTVC-G246. In this document, two experiments have been conducted on the Band Offset tool of SAO. The first experiment A consists in using 3 different SAO band offset classes and in reducing the number of offsets from 16 to 8. The second experiment B consists in using only one SAO band offset class with 8 offsets. For each proposed classification the center of the intensity range is coded. Results of these two core experiments show respectively around 2% and 0.4% average BDR gain for Chroma component without any impact on the Luma component. Moreover, it is reported that the memory usage of SAO parameters can be reduced to 43 % compared to the current HM-5.0.

# Introduction

The current HEVC classification uses 2 kinds of classifications: the Edge Offset (EO) and the Band Offset (BO). For Edge Offset, 4 classes are defined. For Band Offset, 2 classes are defined, corresponding to two groups of pixel values as depicted in Figure 1 (cf. JCTVC-D122 [2]). The intensity range is divided into 32 ranges. Each SAO Band Offset group uses half of the intensity range. The first group is the central range of pixel values containing 16 ranges and the second group is composed of the other 16 ranges corresponding to the extremities of the whole intensity interval.

Figure 1: Band Offset classification of current HM5.0.

This classification is useful when the signal of a frame area covers all the intensity of pixel values. But when the signal is compacted to a restricted range of pixel values and when this range is not centered on the middle of the intensity interval, this classification is not adapted.

For the current band offset classifications in HM5.0, 16 offset values need to be stored for each SAO partition of the frame. The buffer size needed to store all SAO parameters for a frame is proportional to the number of SAO partitions and to the sizes of SAO parameters. The worst case is reached when the number of partitions is maximum and when all these partitions are classified in band offset where 16 offsets need to be stored. In Experiments A and B the number of offsets for the band offset classification is reduced from 16 to 8.

# Proposed Band Offset classifications

## 3 Band Offset classifications

In this subset “e” of this Core Experiment, different SAO Band Offset types are proposed in order to be more adaptive to the pixel values repartition. These 3 proposed types of Band Offsets are replacing the current Band Offset classifications illustrated in Figure 1. The classification BO_8o_1, BO_8o_2 and BO_8o_3 correspond, respectively to the classifications BO_NB_8_SUB_1, BO_NB_4_SUB_2 and BO_NB_2_SUB_4 as described in JCTVC-G246. Compared to HM5.0, the number of offsets is changed and is reduced from 16 to 8. So these classifications need always 8 offsets. The number of bits checked for the classification BO_8o_2 and BO_8o_3 are modified compared to the HM5.0 band offsets where 5 bits are checked for the classification. So, for BO_8o_2 and BO_8o_3, 6 and 7 bits are respectively checked to classify pixels of the partition. Yet, these additional 2 bits checking for hardware implementation does not affect the worst case complexity of the SAO loop filtering. Indeed, the worst case complexity is reached with the Edge Offset classification.

| Name | Class | Nb pixel values per class |
| --- | --- | --- |
| BO_8o_ 1 |  | 8 |
| BO_8o_2 |  | 4 |
| BO_8o_3 |  | 2 |

Figure 2: Classification used for Band Offset.

## Center of the Band Offset

For the proposed Band Offset types, the center of each classification is transmitted for each SAO partition. This center is coded by considering a subdivision of the intensity interval into 32 ranges. The center of a class is predicted by the center of the intensity range and a residual is then coded with a unary code.

# Proposed Experiments

For this section “e.2” of this core experiment, we propose two distinct experiments: Exp A and Exp B. For both experiments, the classical band offset classification is removed and changed by the proposed classification in order to reduce the number of offset to 8. In the first experiment Exp A, 3 classifications are used. For the second experiment, Exp B, only one classification is used (BO_8o_1). This classification needs the same number of bits checking as HM5.0. Table 1 summarizes the classification used in each experiment.

|  | HM5.0 | Exp A | Exp B |
| --- | --- | --- | --- |
| BO_0 | On |  |  |
| BO_1 | On |  |  |
| BO_8o_ 1 |  | on | on |
| BO_8o_2 |  | on |  |
| BO_8o_3 |  | on |  |

Table 1: Summary of the Band Offset classification used for HM5.0 and for the experiments A and B.

# Memory size reduction for the storage of SAO parameters

Compared to the current HM5.0, the proposed band offset classifications reduces the memory size needed to store the SAO parameters in the worst case. Indeed, the maximum number of offsets for a partition is reduced from 16 to 8. Table 2 gives the memory size of SAO parameters needed for one frame with the current HM5.0 for both proposed experiments.

In this table, we considered that 256 blocks of SAO can be used for each frame. For each block, the SAO type needs to be stored with its related offsets values. Moreover, for the proposed experiments the center of the Band Offset is also stored. In the current HM5.0, 7 SAO types are used which need 3 bits. For the proposed experiments Exp A and Exp B, 6 and 8 SAO types are respectively used. So, 3 bits are also needed to store this SAO type. For the coding of the center of the band, 32 positions are considered, which needs 5 bits in memory.

The number of bits for a frame is equal to Nb blocks*( Bits per offset * Max nb of classes + bits for SAO type + bits for center). The maximum memory needed for both experiments is the same and this memory size is reduced by 43% compared to HM5.0 as shown in Table 2.

|  | HM5.0 | Exp A | Exp B |
| --- | --- | --- | --- |
| Number of blocks | 256 | 256 | 256 |
| Bits for SAO type | 3 | 3 | 3 |
| Bits per offset | 6 | 6 | 6 |
| Max number of offsets | 16 | 8 | 8 |
| Bits for center of BO | N. A. | 5 | 5 |
| Total bits | 25344 | 14336 | 14336 |
| % memory size reduction | -- | 43% | 43% |

Table 2: Memory size for SAO parameters in the worst case for one frame containing 256 SAO blocks.

# Results

Simulations results were performed on a Linux platform according to the common test conditions as defined in JCTVC-G1200 [3]. The results on low complexity configurations are also reported in comparison to the LC anchor where SAO have been enabled. .

## Experiment A

### Objective results

Table 3 shows the results of experiment Exp A. On average, the proposed modification has no impact on the luma component with 0% BDR. Yet 0.1% BDR gain is obtained in LDBHE which compensates the 0.1% BDR loss in Intra configuration. The results are obtained by removing the HM5.0 SAO band offset classification. The major improvement is obtained for Chroma with respectively 1.8% and 1.9% average BDR gain for U and V components. The best average gain is obtained for LDBHE configuration with 2.5% and 2.8% BDR gain. The proposed scheme does not affect the encoding and the decoding run time. A decrease of decoding run time is observed for LDPHE configuration (see related Excel file). In this experiment the number of offset which needs to be stored in the worst case is divided by 2.

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.6% | -1.3% |  |  |  |
| Class B | 0.1% | -1.5% | -1.8% |  |  |  |
| Class C | 0.1% | -0.9% | -1.0% |  |  |  |
| Class D | 0.1% | -0.7% | -0.8% |  |  |  |
| Class E | 0.1% | -2.4% | -2.4% |  |  |  |
| Overall | 0.1% | -1.2% | -1.4% |  |  |  |
|  | 0.1% | -1.2% | -1.5% |  |  |  |
| Class F | 0.1% | -1.3% | -0.9% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | -0.7% | -1.4% | 0.0% | -1.7% | -1.9% |
| Class B | 0.0% | -2.2% | -2.4% | 0.0% | -1.7% | -1.9% |
| Class C | 0.0% | -1.4% | -1.1% |  |  |  |
| Class D | 0.0% | -1.3% | -1.2% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -1.5% | -1.6% | 0.0% | -1.7% | -1.9% |
|  | 0.0% | -1.5% | -1.6% | 0.0% | -1.7% | -1.9% |
| Class F | 0.0% | -1.8% | -1.6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -3.1% | -3.3% |  |  |  |
| Class C | 0.0% | -2.2% | -2.2% |  |  |  |
| Class D | 0.0% | -1.6% | -2.1% |  |  |  |
| Class E | -0.2% | -3.5% | -3.9% |  |  |  |
| Overall | -0.1% | -2.6% | -2.8% |  |  |  |
|  | -0.1% | -2.6% | -2.8% |  |  |  |
| Class F | -0.3% | -2.5% | -2.8% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |

Table 3: Simulation results of Experiment Exp A (3 band offsets) of the proposed scheme compared to the HM5.0 anchors.

Table 4 shows the results of Exp A for low complexity configurations. In these experiments SAO have been enabled in the Anchor. In average, for these 4 configurations 0.1% gain is obtained for Luma component and 2.1% and 1.9% respectively for Chroma U and V. These results are little bit better than those obtained for High efficiency cases. So, the efficiency of the proposed SAO modifications in Exp A is robust to the set of tools used in High efficiency cases like ALF.

|  | All Intra LC + SAO |  |  | Random Access LC+ SAO |  |  | Low delay B LC+ SAO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | U | V | Y | U | V | U | V |
| Class A | -0.1% | -0.5% | -1.0% | 0.0% | -1.8% | -2.4% | -0.1% | -4.0% | -4.3% |
| Class B | 0.1% | -1.0% | -1.6% | 0.0% | -2.9% | -3.3% | -0.1% | -3.0% | -2.9% |
| Class C | 0.1% | -0.8% | -0.8% | 0.0% | -2.0% | -2.2% | 0.0% | -2.2% | -2.6% |
| Class D | 0.1% | -0.5% | -0.7% | 0.0% | -2.2% | -2.5% | -0.5% | -4.7% | -5.7% |
| Class E | 0.1% | -1.8% | -1.5% |  |  |  | -0.1% | -2.7% | -2.5% |
| Class F | 0.2% | -1.0% | -0.8% | 0.1% | -1.8% | -1.6% | -0.1% | -3.3% | -3.5% |
| Overall | 0.0% | -0.9% | -1.1% | 0.0% | -2.3% | -2.6% | -0.1% | -3.3% | -3.5% |
|  | 0.0% | -0.9% | -1.2% | 0.0% | -2.3% | -2.6% | -0.1% | -3.3% | -3.5% |
| Enc Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |

|  | Low delay P LC+ SAO |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | -4.0% | -4.0% |
| Class C | 0.0% | -2.8% | -2.4% |
| Class D | 0.0% | -2.1% | -3.4% |
| Class E | -0.2% | -3.9% | -5.7% |
| Class F | -0.4% | -2.9% | -2.7% |
| Overall | -0.1% | -3.1% | -3.6% |
|  | -0.1% | -3.1% | -3.5% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4: Simulation results for Low Complexity configuration of experiment Exp A compared to HM5.0 anchors with SAO enabled.

### Subjective viewing

In terms of subjective evaluation, the visual quality of Exp A is similar to HM5.0 anchors and we noticed that Exp A can sometimes improve the visual quality compared to the current HM5.0 anchors. Table 5 shows some examples of improvements observed.

In example 1, the “text” has been better reconstructed compared to the HM5.0 anchors. In example 2, the basketball net has been correctly reconstructed in Exp A compared to the HM5.0 anchors. In example 3, some artifacts were removed in the Exp A. Example 4 shows an improvement of Chroma V component where the gear bike has a better reconstruction in Exp A than in HM5.0.

| HM5.0 | Exp A |
| --- | --- |
| Example 1: SlideShow_1280x720 QP 37 YUV frame 108 RAHE |  |
| Example 2: BasketballDrill_832x480 QP 37 YUV frame 7 RAHE |  |
| Example 2: BasketballDrill_832x480 QP 37 YUV frame 31 LDHE |  |
| Example 3: SlideShow_1280x720 QP 37 V frame 399 LDHE |  |

Table 5 Examples of visual improvements of the proposed Exp A.

## Experiment B

### Objective results

Table 6 shows the results of experiment Exp B. In average the proposed modification has no impact on the Luma component with 0% BDR. 0.4% average BDR gain is obtained for Chroma component. The proposed scheme does not affect the encoding and the decoding run time. As for the first experiment, in this experiment the number of offset which need to be stored in the worst case is divided by 2.

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | -0.3% | -0.3% |  |  |  |
| Class C | 0.0% | -0.3% | -0.2% |  |  |  |
| Class D | 0.0% | -0.1% | -0.1% |  |  |  |
| Class E | 0.0% | -0.4% | -0.5% |  |  |  |
| Overall | 0.0% | -0.2% | -0.3% |  |  |  |
|  | 0.0% | -0.3% | -0.3% |  |  |  |
| Class F | 0.3% | -0.2% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.2% | 0.1% | -0.7% | -0.6% |
| Class B | 0.0% | -0.4% | -0.4% | 0.0% | -0.4% | -0.4% |
| Class C | 0.0% | -0.5% | -0.2% |  |  |  |
| Class D | 0.0% | -0.3% | -0.3% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.4% | -0.3% | 0.0% | -0.5% | -0.5% |
|  | 0.0% | -0.4% | -0.3% | 0.1% | -0.5% | -0.5% |
| Class F | 0.2% | -0.4% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.5% | -0.7% |  |  |  |
| Class C | 0.0% | -0.6% | -0.6% |  |  |  |
| Class D | 0.1% | -0.3% | -0.1% |  |  |  |
| Class E | 0.1% | -0.8% | -0.1% |  |  |  |
| Overall | 0.0% | -0.5% | -0.4% |  |  |  |
|  | 0.0% | -0.5% | -0.4% |  |  |  |
| Class F | 0.1% | -0.6% | -1.0% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |

Table 6: Simulation results of Experiment Exp B (3 band offsets) of the proposed scheme compared to the HM5.0 anchors.

Table 7 shows the results of Exp B for low complexity configurations. In these experiments SAO have been enabled in the Anchor. In average, for these 4 configurations 0.5% gain is obtained for Chroma component. This result is little bit better than those obtained for High efficiency cases. As Exp A, the efficiency of Exp b is robust to the set of tools used in High efficiency cases.

|  | All Intra LC + SAO |  |  | Random Access LC+ SAO |  |  | Low delay B LC+ SAO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | U | V | Y | U | V | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | -0.5% | -0.6% | 0.0% | -0.7% | -0.7% |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.6% | -0.6% | 0.0% | -1.0% | -0.8% |
| Class C | 0.0% | -0.2% | -0.2% | 0.0% | -0.7% | -0.5% | 0.0% | -0.6% | -0.9% |
| Class D | 0.0% | -0.1% | -0.1% | 0.1% | -0.6% | -0.6% | 0.0% | -0.8% | -0.5% |
| Class E | 0.0% | -0.3% | -0.3% |  |  |  | 0.3% | -0.6% | -0.1% |
| Class F | 0.4% | -0.2% | 0.0% | 0.3% | -0.4% | -0.1% | 0.0% | -0.7% | -0.6% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.6% | -0.6% | 0.0% | -0.7% | -0.6% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.6% | -0.6% | 0.0% | -0.7% | -0.7% |
| Enc Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 99% |  |  |

|  | Low delay P LC+ SAO |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.9% | -0.9% |
| Class C | 0.0% | -0.6% | -0.5% |
| Class D | 0.0% | -0.1% | -1.1% |
| Class E | 0.1% | 0.0% | -1.3% |
| Class F | 0.2% | -0.2% | 0.0% |
| Overall | 0.0% | -0.4% | -0.7% |
|  | 0.0% | -0.4% | -0.7% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 7: Simulation results for Low Complexity configuration of experiment Exp B compared to HM5.0 anchors with SAO enabled.

### Subjective viewing

In term of subjective evaluation, Exp B is quite similar to the current HM5.0 anchors and no degradation was observed.

# Conclusion

This contribution reports results of the replacement of the HM5.0 SAO band offset classifications by 3 classifications with a reduced number of offsets. For the 3 proposed SAO band offset types the center of the intensity range is transmitted in order to fit to a signal which is represented by a restricted range of pixel values as Chroma signal. Two experiments were conducted, one with 3 SAO band offset groups and one with only 1 SAO band offset group. Both experiments do not affect the coding efficiency of the Luma component objectively and subjectively. In addition, both experiments provide gains for Chroma component with around 2% average BDR gain when 3 SAO band offsets are used and 0.4% when only one classification is used. Moreover, examples of visual improvements are provided. In terms of memory, the proposed modification reduces the buffer needed to store the SAO parameters. The memory reduction is about 43% compared to HM5.0.

# Reference

- G. Laroche, T. Poirier, P. Onno, “On additional SAO Band Offset classifications”, Joint Collaborative Team on Video Coding, JCTVC-G246, Nov. 2011.

- C.-M. Fu, C.-Y. Chen, Y.-W. Huang, and S. Lei, “TE8 Subtest3: Picture Quadtree-based Adaptive Offset”, Joint Collaborative Team on Video Coding, JCTVC-D122, Jan. 2011.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).