| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0175 |
| --- | --- |

| Title: | Non-CE6c: adaptations of Intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Stephane PAUTET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes three modifications to current intra prediction mode coding in HM. The first is a modification of the MPM derivation process, avoiding the systematic access to the top and left neighboring PUs. The second is a modification of the remaining mode coding, enabling direct decoding of Planar mode and also FLC remaining bins decoding.  The third consists in replacing the current HM intra mode numbering by a numbering based on angular index, resulting in removing one look-up table from the WD and from the HM. These modifications purportedly simplify the intra mode coding process and HEVC specification.

In addition, the proposed modifications are reported to bring coding gains. When using two CABAC contexts for the 2 flags used in intra mode coding, an average BD-Rate gain of 0.3% and 0.4% for AI-HE and AI-LC configurations respectively is reported. When all bins are bypass coded, an average BD-Rate gain of 0.2% and 0.3% for AI-HE and AI-LC configurations respectively is reported.

# Introduction

In the current HEVC design, intra mode coding makes use of the two neighboring Top and Left PUs to derive two Most Probable Modes (MPMs) values. The mode of the current PU is then compared to the MPMs. If it is equal to one of the MPMs, 2 bins are used to signal that the mode is one of the MPMs and to signal which MPM is used. If the intra mode is not equal to one of the MPMs, the remaining mode value, possibly decremented according to the MPMs values, is coded.

The intra mode decoding process can be summarized as follows:

Derivation of the 2 MPMs:

- Non-availability check:

  - If top / left PU is not available/intra, topMode / leftMode is set to Planar

- Mapping:

  - If topMode / leftMode is not supported by current PU, topMode / leftMode is set to Planar

- MPMs computation:

  - If topMode != leftMode		MPM0 = min(leftMode, topMode)

MPM1 = max(leftMode, topMode)

  - otherwise 				MPM0 = Planar

    - if topMode ! = Planar	MPM1 = topMode

Intra mode decoding:

- If (prev_intra_luma_pred_flag = 1)

  - If (mpm_idx = 0) 					mode = MPM0

  - Otherwise						mode = MPM1

- Otherwise

  - mode = remaining_mode

  - When MPM0 <= mode				mode = mode+1

  - When MPM1 <= mode				mode = mode+1

This design presents several issues:

- non-FLC are needed to code remaining modes, since for PUs larger than 4x4 supporting 35 modes, there are 33 possible remaining modes.

- Hor+6 mode is not supported in 4x4 PUs; however this mode would logically be included (4x4 PU support all other Ver+2x and Hor+2x modes).

- Full MPM derivation is always required, implying a systematic access to left and top PUs.

- A specific mode numbering is used, requiring a look-up table specifying the mapping between the intra mode and an intra angular index (Table 8-7 in WD and table g_aucAngIntraModeOrder[mode] in HM).

JCTVC-G153 has proposed a solution to solve issues 1 and 2. This proposal proposes one further simplification to JCTVC-G153, consisting in inserting the planar mode as one preferred mode. It also proposes a simplification of the MPM derivation process. This avoids the decoder to systematically fully derive the 2 MPMs, which addresses issue 3. Finally, it is proposed to replace the intra mode numbering by an angular-index based numbering, which addresses issue 4.

# Proposal description

## Simplification of the MPMs derivation

In the current design, it is necessary to systematically access the top and left PUs to derive the 2 MPMs. We propose one simple simplification which consists in setting MPM0 to left Mode. Then MPM1 is derived from the value of topMode and MPM0.

The new MPM computation process is:

  - MPM0 = leftMode

  - If topMode != MPM0		MPM1 = topMode

  - otherwise if MPM0 ! = Planar	MPM1 = Planar

  - otherwise				MPM1 = DC

This avoids for the decoder to systematically access both Top and Left PUs: when prev_intra_luma_pred_flag = 1 and mpm_idx = 0, mode is equal to MPM0, derived from left PU only.

## Use of Planar as a preferred mode

Planar mode is statistically more frequent than the other intra modes (around 25% of the PUs of any size use this mode). Therefore making this mode more favored in the coding process looks logical. As in JCTVC-G153, we add a flag intra_mode_planar_flag in the remaining mode decoding tree, indicating if the mode is planar or not. MPM derivation process is modified to avoid MPMs to be equal to Planar mode. In addition we add mode Hor+6 intra to the set of 4x4 PU modes. Thanks to these modifications, FLC can be used to code the remaining mode. The decoding tree is depicted in Figure 1.

Figure 1: proposed intra mode decoding tree.

This gives the following advantages:

- Hor+6 intra prediction mode is included in the set of 4x4 PU modes (19 possible modes)

- Simple FLC are used to binarize rem_intra_luma_pred_mode_minus1, since there are 16 (19-3) possible remaining modes for 4x4 PUs and 32 (35-3) for PUs of other size

- Coding gains are obtained (see test 3 in annex)

## Use of angular indexes for the mode numbering

In the current design, all bins, except the flag prev_intra_luma_pred_flag, are bypass coded. This means that the mode numbering has no impact on the coding performance (no mode is favored compared to the others since no context is used). It is therefore preferable to choose a simple mode numbering that may avoid additional text in the specification and look-up tables in the implementation.

We propose to replace the current intra mode numbering by angle index-based numbering (cf Figure 2):

- The angular modes are numbered from 0 to 32, from down left diagonal to top right diagonal.

- Planar is numbered 33,

- DC is numbered 34.

One important property of the new mode numbering is that modes of 4x4 PUs, except planar mode, have only even values (in Figure 2 modes in grey are not supported by 4x4 PUs). Therefore a 4x4 mode is divided by 2 before being coded (and inversely multiplied by 2 after being decoded).

This enables two possible very simple side improvements bringing added coding efficiency:

- New Mapping:

  - If topMode / leftMode is not supported by current PU, topMode / leftMode is divided by 2, instead of being enforced to Planar as in current design

- Improved MPM1 derivation

  - If topMode = leftMode, MPM1 = (leftMode + 1)  instead of DC as in current design

    - Using an adjacent mode has been proven to bring gains.

This gives the following advantages:

- simplifications of  text specification and HM design: mapping table mode-angular index is removed (8-7 in WD and g_aucAngIntraModeOrder in HM), MDIS table can be simplified (8-6 in WD), DCT/DST tables can be replaced by simple logic (8-14 in WD and g_aucDCTDSTMode_xx in HM)

- Coding gains are obtained from improved mapping and MPM1 derivation, with negligible added complexity

Figure 2: intra mode numbering in current design, and proposed new mode numbering.

## Combination of the 3 modifications

When proposals 2.1, 2.2 and 2.3 are combined, the intra mode decoding process works as follows:

Derivation of the 2 MPMs:

- Non-availability check:

  - If top / left PU is not available/intra, 		topMode / leftMode is set to DC

  - Otherwise if top / left mode is equal to Planar, 	topMode / leftMode is set to DC

- Mapping:

  - If (intraPredModeNum = 19)			topMode = topMode >> 1

leftMode = leftMode >> 1

- MPMs computation:

  - MPM0 = leftMode

  - If topMode != MPM0				MPM1 = topMode

  - Otherwise if MPM0 < ( intraPredModeNum – 3 )	MPM1 = MPM0+1

  - Otherwise if intraPredModeNum = 19		MPM1 = Vert >> 1

  - Otherwise 						MPM1 = Vert

Intra mode decoding:

- If (prev_intra_luma_pred_flag = 1)

  - If (mpm_idx = 0) 					mode = MPM0

  - Otherwise						mode = MPM1

- Otherwise if (intra_mode_planar = 0)			mode = Planar

- Otherwise

  - mode = remaining_mode

  - When Min( MPM0, MPM1 ) <= mode		mode = mode+1

  - When Max( MPM0, MPM1) <= mode		mode = mode+1

- When ( intraPredModeNum = 19 && mode != Planar )	mode = mode << 1

# Results

Tests were performed using the tests conditions specified in JCTVC-G1200, on AI-HE and AI-LC configurations. Detailed results are reported in Annex and in attached xls files.

We report below the results combining the simplified MPM derivation, the insertion of Planar as preferred mode, the use of angular-index based numbering. Two cases are considered: usage of 2 CABAC contexts (one for prev_intra_luma_pred_flag, one for intra_mode_planar_flag), usage of no CABAC context at all (all bins are bypass coded). These 2 cases correspond to files JCTVC-H0175_IMC_Test5a.xls and JCTVC-H0175_IMC_Test5b.xls, respectively. In both cases gains are observed. The encoder run time is slightly decreased (2%). The decoder run time is not impacted. Due to its simpler design, the solution with no context is recommended.

|  | AI-HE |  |  |  |  | AI-LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 2 Ctx | -0.3% | -0.1% | -0.1% | 98% | 100% | -0.4% | -0.2% | -0.2% | 98% | 100% |
| No Ctx | -0.2% | 0.0% | 0.0% | 98% | 100% | -0.3% | -0.2% | -0.1% | 98% | 100% |

# Conclusion

In this contribution, we have proposed the following simplifications:

- Modified remaining mode coding, enabling a binary+FLC binarization,

- Modified MPM derivation, avoiding for the decoder to systematically access the left and top PUs,

- Modified mode numbering, based on angular indexes, resulting in removing one look-up table,

- Removal of all contexts for intra mode coding, resulting in a simpler entropy coding.

The combination of these modifications gives a Y-BDR gain of 0.2% for AI-HE and 0.3% Y-BDR gain. We recommend adopting the proposed modifications in the next HEVC test model.

# References

- F. Bossen, “Common conditions and software reference configurations,” in JCTVC-G1200, Geneva, Switzerland, Nov., 2011.

- C. Yeo, H. L. Tan, Y. H. Tan, Z. Li., “Non-CE6: On intra prediction mode coding”, in JCTVC-G153, Geneva, Switzerland, Nov., 2011.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex – more complete results

Tests were performed using the tests conditions specified in JCTVC-G1200, on AI-HE and AI-LC configurations. The combinations listed in Error: Reference source not found have been evaluated. In the current HM, one CABAC context is used for the syntax element prev_intra_luma_pred_flag. A second context is added for the new syntax element intra_mode_planar_flag. A configuration without any context (all bins are bypass coded) is also tested (test 5b of Error: Reference source not found).

Table 1: description of evaluation tests.

|  | Simplified MPMs | planar | Angular numbering | Angular numbering + new mapping | Angular numbering + new MPM1 derivation | Nb of CABAC ctx |
| --- | --- | --- | --- | --- | --- | --- |
| Test1 | X |  |  |  |  | 1 |
| Test2 | X | X |  |  |  | 2 |
| Test3 | X | X | X |  |  | 2 |
| Test4 | X | X | X | X |  | 2 |
| Test5a | X | X | X | X | X | 2 |
| Test5b | X | X | X | X | X | 0 |

Error: Reference source not found summarizes the results for these different tests. Full results can be found in Annex and in attached files. Configuration 5b, that combines the 3 proposals, and that use only bypass coding for all bins, is recommended.

Table 2: results summarizing.

|  | AI-HE |  |  |  | AI-LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc/Dec runtime | Y | U | V | Enc/Dec runtime |
| Test1 | 0.00% | 0.00% | 0.00% | 98/100 | 0.00% | 0.00% | 0.00% | 98/100 |
| Test2 | -0.15% | -0.09% | -0.08% | 99/100 | -0.23% | -0.18% | -0.15% | 98/101 |
| Test3 | -0.15% | -0.08% | -0.08% | 99/100 | -0.22% | -0.17% | -0.15% | 99/100 |
| Test4 | -0.21% | -0.14% | -0.11% | 99/99 | -0.28% | -0.20% | -0.19% | 98/101 |
| Test5a | -0.28% | -0.12% | -0.10% | 98/100 | -0.36% | -0.20% | -0.16% | 98/100 |
| Test5b | -0.20% | -0.04% | 0.01% | 98/100 | -0.27% | -0.15% | -0.10% | 98/100 |

## Test1: Simplified MPMs

As observed in the following table, the proposed MPM derivation simplification has no impact on the coding efficiency, while decreasing the encoder run time by 2%.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.00% | -0.01% | -0.02% | 0.00% | 0.01% | -0.01% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.01% | 0.01% | 0.00% | -0.01% | -0.02% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.01% | -0.01% | -0.01% | 0.00% | -0.01% | 0.01% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Test2 : test1 + planar as preferred mode

Using planar mode as a preferred mode gives around 0.15% Y-BDR gain.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.20% | 0.15% | 0.21% | -0.34% | -0.05% | 0.07% |
| Class B | -0.21% | -0.16% | -0.16% | -0.30% | -0.25% | -0.17% |
| Class C | -0.13% | -0.14% | -0.13% | -0.19% | -0.16% | -0.17% |
| Class D | -0.13% | -0.03% | -0.06% | -0.15% | -0.10% | -0.13% |
| Class E | -0.08% | -0.14% | -0.10% | -0.17% | -0.27% | -0.26% |
| Overall | -0.15% | -0.09% | -0.08% | -0.23% | -0.18% | -0.15% |
|  | -0.15% | -0.09% | -0.07% | -0.23% | -0.17% | -0.15% |
| Class F | -0.13% | -0.29% | -0.24% | -0.15% | -0.36% | -0.34% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Test3 : test2 + angular-based mode numbering

Numbering simplification using angular-index based numbering does not cause any change in performance.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.19% | 0.16% | 0.22% | -0.34% | -0.04% | 0.05% |
| Class B | -0.21% | -0.16% | -0.18% | -0.30% | -0.24% | -0.17% |
| Class C | -0.13% | -0.13% | -0.11% | -0.19% | -0.14% | -0.18% |
| Class D | -0.13% | -0.01% | -0.07% | -0.15% | -0.09% | -0.13% |
| Class E | -0.08% | -0.14% | -0.09% | -0.17% | -0.29% | -0.26% |
| Overall | -0.15% | -0.08% | -0.08% | -0.22% | -0.17% | -0.15% |
|  | -0.15% | -0.08% | -0.07% | -0.23% | -0.16% | -0.15% |
| Class F | -0.13% | -0.27% | -0.19% | -0.14% | -0.34% | -0.31% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Test4 : test3 + new mapping

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.26% | 0.13% | 0.21% | -0.39% | -0.05% | 0.04% |
| Class B | -0.25% | -0.19% | -0.19% | -0.35% | -0.25% | -0.20% |
| Class C | -0.21% | -0.22% | -0.20% | -0.26% | -0.22% | -0.23% |
| Class D | -0.18% | -0.07% | -0.09% | -0.19% | -0.14% | -0.17% |
| Class E | -0.14% | -0.21% | -0.13% | -0.24% | -0.27% | -0.28% |
| Overall | -0.21% | -0.14% | -0.11% | -0.28% | -0.20% | -0.19% |
|  | -0.21% | -0.14% | -0.11% | -0.28% | -0.19% | -0.18% |
| Class F | -0.19% | -0.32% | -0.26% | -0.21% | -0.38% | -0.37% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

## Test5 : test4 + MPM0++

Intra mode coding is performed as described in section 2.4.

### Test5a: 2 contexts

New MPM1 derivation, with MPM1=MPM0+1 when both MPMs are equal, brings around 0.05% Y_BDR gain.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.33% | 0.10% | 0.15% | -0.46% | -0.08% | 0.05% |
| Class B | -0.31% | -0.13% | -0.14% | -0.41% | -0.18% | -0.13% |
| Class C | -0.27% | -0.20% | -0.20% | -0.33% | -0.25% | -0.25% |
| Class D | -0.20% | -0.08% | -0.11% | -0.23% | -0.18% | -0.17% |
| Class E | -0.29% | -0.18% | -0.08% | -0.41% | -0.27% | -0.23% |
| Overall | -0.28% | -0.12% | -0.10% | -0.36% | -0.20% | -0.16% |
|  | -0.28% | -0.12% | -0.10% | -0.36% | -0.19% | -0.16% |
| Class F | -0.25% | -0.39% | -0.26% | -0.25% | -0.41% | -0.47% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | 0.3% | 0.1% | -0.2% | 0.2% | 0.3% | -0.1% | 0.3% | 0.6% |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.3% | 0.0% | 0.2% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | -0.1% | 0.1% | 0.3% |
|  | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | -0.1% | 0.1% | 0.3% |
| Class F | -0.2% | -0.2% | -0.4% | -0.1% | -0.3% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  | 101% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | -0.2% | 0.1% |
| Class C | 0.1% | -0.1% | 0.2% | 0.0% | 0.1% | 0.0% |
| Class D | 0.2% | 0.1% | 0.2% | 0.0% | 0.1% | 0.4% |
| Class E | 0.1% | -0.1% | -1.2% | 0.0% | -0.4% | -0.3% |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
|  | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class F | -0.3% | -0.4% | -0.5% | 0.2% | -0.4% | -0.6% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

### Test5b: no context

A large part of the gain is saved when using no context instead of 2 contexts.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.34% | 0.11% | 0.15% | -0.45% | -0.10% | -0.01% |
| Class B | -0.19% | -0.06% | -0.02% | -0.31% | -0.14% | -0.07% |
| Class C | -0.19% | -0.13% | -0.11% | -0.22% | -0.18% | -0.18% |
| Class D | -0.15% | 0.00% | -0.06% | -0.12% | -0.17% | -0.20% |
| Class E | -0.21% | -0.01% | 0.21% | -0.34% | -0.14% | 0.01% |
| Overall | -0.20% | -0.04% | 0.01% | -0.27% | -0.15% | -0.10% |
|  | -0.20% | -0.03% | 0.01% | -0.27% | -0.15% | -0.11% |
| Class F | 0.01% | 0.09% | 0.08% | 0.01% | 0.25% | 0.01% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | 0.2% | 0.3% | -0.2% | 0.2% | 0.2% | 0.0% | 0.1% | 0.5% |
| Class B | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.2% | 0.1% | 0.2% | -0.2% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.3% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.4% | 0.0% | -0.4% | 0.2% |
| Class C | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.3% | 0.0% | 0.0% | 0.2% | -0.4% |
| Class E | 0.1% | 1.0% | 0.0% | -0.1% | 0.0% | 0.6% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.1% |
| Class F | 0.0% | 0.0% | -0.1% | 0.1% | -0.7% | 0.2% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |