| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0177 |
| --- | --- |

| Title: | Non-CE6a: Use of chroma phase in LM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Christophe GISQUET Stephane PAUTET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This proposal provides additional results related the chroma phase issue in the luma-based chroma prediction mode (LM mode), already addressed in contribution JCTVC-G245. The issue consists in a possible misalignment of interpolated luma samples with the chroma samples. In JCTVC-G245, it has been proposed to signal the chroma phase parameters and to adapt the luma interpolation process based on this chroma phase information.

This proposal reports further experiments and recommends one set of filters to handle four possible basic chroma phase settings. The evaluation has been made on different versions of 4:2:0 sequences, generated from 4:4:4 sequences, whose chroma components have been downsampled with specific chroma phases. Using these recommended filters, average BDR gains up to* 0.2% Y, 4.2% U and 3.5% V are reported.

* the set of 4:4:4 sequences have been converted into several sets of 4:2:0 sequences with different chroma phases. One average BDR gain is measured for each set of 4:2:0 sequences.

# Introduction

The issue of chroma phase in the luma-based chroma prediction mode (LM mode) has been raised in JCTVC-G245 [1]. In its current design, LM mode only considers 4:2:0 content with so-called ‘nominal’ luma and chroma samples position [2]. In JCTVC-G245 a generalized design has been proposed to handle more generic cases, that is, with non-nominal luma and chroma samples position. It consists in taking into account the chroma samples position (chroma phases) in the luma interpolation process in case of 4:2:0 content. This information is explicitly signaled in the sequence parameter set (insertion in PPS or APS could also been considered). The interpolation process of luma samples is modified so that interpolated luma samples are better positioned related to the chroma samples to be predicted, whatever the considered chroma phase.

In this proposal, a set of filters is recommended for three other chroma phase settings than the nominal one.

# Chroma phase in current HM design

## Chroma phase definition

The chroma phase corresponds to the location of the chroma samples related to the luma samples. In Figure 1 different possible positions of the chroma samples related to the luma samples in a 4:2:0 content are considered. For each position, a couple of horizontal and vertical chroma phases (Cx,Cy), in unit of luma samples of a frame, is specified.

Figure 1: chroma phases in 4:2:0 format.

Chroma phase is already implicitly specified in the current HM WD [2], in the same way as in H.264/MPEG-4 AVC. The syntax element chroma_format_idc defines the chroma format sampling structure. In case of 4:2:0 content, corresponding to chroma_format_idc equal to 1, the nominal vertical and horizontal relative locations of luma and chroma samples in pictures correspond to (Cx=0, Cy=0.5).

In addition, in H.264/MPEG-4 AVC, alternative chroma sample relative locations may be indicated in case of 4:2:0 content in Video Usability Information.

Chroma phases are also specified in the SVC extension of H.264/MPEG-4 AVC, in order to accurately control the inter-layer chroma prediction.

## Chroma phase in current LM mode design

In the LM mode, the nominal position is also explicitly considered. The interpolated luma samples RecL’[x,y] used to derive the linear model parameters and to predict the chroma signal are virtually located at the nominal position, as illustrated in Figure 2. The filters used to generate the collocated interpolated luma samples are build based on this assumption:

- To generate the left border and inner luma samples, vertical filter [1,1]/2 is used

  - RecL’[-1,y] = 	( RecL[-2,2y] + RecL[-2,2y+1] ) >> 1	      for the left border samples**

  - RecL’[x,y] = 	( RecL[2x,2y] + RecL[2x,2y+1] ) >> 1	      for the inner samples

- To generate the top border luma samples, horizontal filter [1,2,1]/4 is used

  - RecL’[x,-1] = 	( RecL[2x-1,-1] + 2 RecL[2x,-1] + RecL[2x+1,-1] + 2 ) >> 2

Figure 2: position of luma-chroma samples in LM mode.

**for the LCU border, the immediately left border samples are considered (this enables reducing the storage needs).

# Proposal: LM mode generalization with chroma phase

As already described in JCTVC-G245, it is suggested to generalize the LM mode to other cases than the nominal chroma samples positioning. The 4 cases depicted in Figure 1 are considered. In case of 4:2:0 content, the chroma phase relative to the luma samples position is signaled in the Sequence Parameter Set, as described in section 3.1. Depending on the chroma phase, different filters are used to generate the interpolated luma samples used to predict the chroma samples.

## Syntax changes

It is first proposed to insert syntax elements related to chroma phases into the Sequence Parameter Set.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 1 ¦ ¦ chroma_format_idc = = 2 ) { |  |
| chroma_phase_x | u(1) |
| } |  |
| if( chroma_format_idc = = 1 ) { |  |
| chroma_phase_y | u(1) |
| } |  |
| max_temporal_layers_minus1 | u(3) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

chroma_phase_x specifies the horizontal phase shift of the chroma components in units of half luma samples of a frame (possible values 0,1). If chroma_phase_x is not present, it shall be inferred to be equal to 0.

chroma_phase_y specifies the vertical phase shift of the chroma components in units of half luma samples of a frame (possible values 0,1). If chroma_phase_y is not present, it shall be inferred to be equal to 0.

## Modification of luma signal interpolation process

Based on the chroma phase configuration, the set of luma interpolation filters to generate top, left and inner interpolated luma samples is selected. Several sets of filters have been evaluated (see annex A.4 for detailed results obtained with different considered filters).

Based on the evaluation, the following sets of filters are recommended.

- chroma_phase_x=0 and chroma_phase_y=0 (Cx=0, Cy=0)

  - Top : [1,2,1]/4 horizontal	Left : [1,2,1]/4 vertical	Inner : [1,2,1]/4 horizontal

  - This setting looks useful to filter luma samples to reduce noise.

- chroma_phase_x=1 and chroma_phase_y=0 (Cx=0.5, Cy=0)

  - Top : [1,1]/2 horizontal	Left : [1,2,1]/4 vertical	Inner : [1,1]/2 horizontal

  - This filter is the symmetrical of the current HM filter.

- chroma_phase_x=1 and chroma_phase_y=1 (Cx=0.5, Cy=0.5)

  - Top : [1,1,1,1]/4 horizontal	Left : [1,1,1,1]/4 vertical	Inner : 1,11,14

  - This filter gives around 1% U, 0.4% V better results than [1,1]/2.

# Results

## Test conditions

To evaluate the proposed solution, new test sequences with specific chroma phases have been used. A set of 8 native RGB 4:4:4 sequences, grouped into 2 classes, are used:

- Class P: 1080p sequences used for the AVC professional extension (man_in_restaurant, playing_cards, rolling_tomatoes, table_setting), [3]. Each sequence is made of 60 frames.

- Class S: 2 1080p sequences, (ducks_takeoff, 251 frames, in_to_tree, 224 frames), 2 2160p (3940x2160) sequences, (crowd_run, 61 frames, park_joy, 61 frames) from SVT [4].

Each sequence has then been converted into 8 different YUV 4:2:0 versions, corresponding to 8 different horizontal/vertical chroma phases. RGB 4:4:4 to YUV 4:2:0 conversion processes are described in annex). The related conversion tools (RGB 4:4:4 to YUV 4:4:4, YUV 4:4:4 to YUV 4:2:0) are provided in attachment to this contribution (file JCTVC-H0177_conversionTools.zip).

## Results

In our experiments, 4 HM versions are used:

- HM_V0: Default LM, corresponding to nominal chroma phase Cx=0, Cy=0.5

- HM_V1: LM adapted for Cx=0, Cy=0

- HM_V2: LM adapted for Cx=0.5, Cy=0

- HM_V3: LM adapted for Cx=0.5, Cy=0.5

The corresponding modified HM5.0 software is provided in attachment to this contribution (file JCTVC-H0177_HM5.0_LM-with-ChromaPhase_Canon.zip). The following define enables to use chroma phase in LM mode:

LM_CHRPHASE_CXCY

Additionally, 2 defines CHRPHASE_CX and CHRPHASE_CY are used to specify the horizontal and vertical chroma phase of the HM.

The following table summarizes the results (BD-rate variations in %) obtained with the filters provided in section 3.2. It can be observed that using interpolation filters adapted to the chroma phase of the input sequence is always beneficial. Default HM performs better than the other ‘chroma phased’ HM versions only for the sequences corresponding to the nominal chroma phase (Cx=0, Cy=0.5). The observed average gain is up to 0.2% Y, 4.2% U and 3.5% V.

|  |  | Reference HM_V0: Cx=0, Cy=0.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase of input sequences |  | HM_V1 Cx=0, Cy=0 |  |  | HM_V2 Cx=0.5, Cy=0 |  |  | HM_V3 Cx=0.5, Cy=0.5 |  |  |  |
| Cx | Cy | Y | U | V | Y | U | V | Y | U | V | Best version |
| 0 | 0 | -0.1 | -1.1 | -1.5 | 0.0 | 0.3 | 0.3 | 0.1 | 1.5 | 1.0 | HM_V1 |
| 0.25 | 0 | -0.1 | -1.4 | -1.5 | 0.0 | -1.1 | -1.4 | 0.0 | -0.3 | 0.0 | HM_V1 |
| 0.5 | 0 | -0.1 | -1.3 | -1.2 | -0.1 | -2.8 | -2.7 | -0.1 | -2.0 | -1.4 | HM_V2 |
| 0.75 | 0 | -0.1 | -1.2 | -1.0 | -0.2 | -4.2 | -3.5 | -0.2 | -3.4 | -2.3 | HM_V2 |
| 0 | 0.5 | 0.3 | 4.9 | 2.6 | 0.4 | 7.3 | 3.8 | 0.1 | 1.5 | 1.4 | HM_V0 |
| 0.25 | 0.5 | 0.3 | 5.0 | 2.6 | 0.3 | 5.2 | 2.6 | 0.0 | -0.6 | -0.2 | HM_V3 |
| 0.5 | 0.5 | 0.3 | 4.8 | 2.5 | 0.2 | 3.0 | 1.1 | -0.1 | -2.6 | -1.9 | HM_V3 |
| 0.75 | 0.5 | 0.3 | 4.2 | 2.1 | 0.2 | 0.9 | -0.2 | -0.2 | -4.2 | -2.8 | HM_V3 |

We have also evaluated the impact of not using the default LM mode (HM_V0) for the different considered test sequences. The following table shows the very strong impact of the LM mode, even when the phase is not correctly handled. The case where the loss is minimum (Cx=0.75, Cy=0) is also the one for which the highest gain is observed using the right ‘chroma phased’ HM (that is HM_V2).

| Phase of input sequences |  | HM without LM mode vs HM with default LM mode |  |  |
| --- | --- | --- | --- | --- |
| Cx | Cy | Y | U | V |
| 0 | 0 | 2.1 | 17.4 | 15.5 |
| 0.25 | 0 | 2.0 | 17.4 | 15.4 |
| 0.5 | 0 | 1.9 | 15.7 | 14.4 |
| 0.75 | 0 | 1.8 | 13.0 | 12.9 |
| 0 | 0.5 | 2.3 | 23.1 | 19.0 |
| 0.25 | 0.5 | 2.2 | 22.7 | 18.7 |
| 0.5 | 0.5 | 2.1 | 20.2 | 17.2 |
| 0.75 | 0.5 | 2.0 | 16.6 | 15.2 |

# Conclusion

This study has confirmed the trends previously reported in JCTVC-G245. In addition we have evaluated different filters and have selected a recommended set of filters to be applied for 4 possible chroma phase settings. We suggest the JCTVC group to consider these filters for adoption in the HM.

Currently only 4:2:0 format is considered. The concept can be easily generalized to 4:2:2.

# References

- Edouard FRANCOIS , Christophe GISQUET, Stephane PAUTET, “Non-CE6a: Use of chroma phase in LM mode” JCTVC-G245, 7th Meeting: Geneva, CH, 21–30 November, 2011.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand., “WD4: Working Draft 4 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1103, 7th Meeting: Geneva, CH, 21–30 November, 2011.

- Tom McMahon., “Film-Originated Test Sequences”, Joint Video Team (JVT) of ISO/IEC JPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6), JVT-J042, 10th Meeting: Waikoloa, Hawaii, USA, 8-12 December, 2003.

- “The SVT High Definition Multi Format Test Set”, Feb.2006, http://media.xiph.org/svt/SVT_MultiFormat_v10.pdf

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# ANNEX – conversion from RGB 4:4:4 to YUV 4:2:0

## A1.	RGB 4:4:4 to YUV 4:4:4

The following transformation is used:

Y =  0.299R + 0.587G + 0.114B

U = -0.147R - 0.289G + 0.436B

V =  0.615R - 0.515G - 0.100B

The attached conversion tool ‘dpxsgiToYUV’ enables to read SGI or DPX 4:4:4 sequences and to convert them into YUV 4:4:4 according to this transform.

## A2.	YUV 4:4:4 to YUV 4:2:0 with specific chroma phase

Each YUV 4:4:4 sequence is converted into YUV 4:2:0 with a given chroma phase  (Cx, Cy).

The chroma downsampling is achieved using downsampling filters from the JSVM (the corresponding filter is given in section A.3).

Figure 3: schematic illustration of the 4:4:4 to 4:2:0 conversion.

The generic chroma signal downsampling equation is as follows:

Cdown[ x, y ] = Cref[ x + Cx , y + Cy ]

with (Cx,Cy) being the horizontal and vertical chroma phases (using conventions of Figure 1).

## A.3		Downsampling filter used to generate chroma re-phased 4:2:0 sequences

These filters are specified in the downsampling tool of the reference software of the SVC extension of the MPEG-4 AVC/H.264 standard (JSVM). They are use in case of downsampling with a ratio of 2.

const int filter16[16][12] =

{

{   0,  -4,  -7,  11,  38,  52,  38,  11,  -7,  -4,   0,   0 }, //phase 0

{   0,  -4,  -7,   9,  37,  51,  40,  13,  -6,  -7,   2,   0 },

{   0,  -3,  -7,   8,  35,  51,  41,  14,  -5,  -7,   1,   0 },

{   0,  -2,  -8,   6,  33,  51,  42,  16,  -5,  -7,   2,   0 },

{   0,  -2,  -8,   5,  32,  50,  43,  18,  -4,  -8,   2,   0 },//phase 0.25

{   0,  -2,  -8,   4,  30,  50,  45,  19,  -3,  -8,   1,   0 },

{   0,  -1,  -8,   2,  28,  49,  46,  21,  -2,  -8,   1,   0 },

{   0,  -1,  -8,   1,  26,  49,  47,  23,  -1,  -8,   0,   0 },

{   0,   0,  -8,   0,  24,  48,  48,  24,   0,  -8,   0,   0 },//phase 0.5

{   0,   0,  -8,  -1,  23,  47,  49,  26,   1,  -8,  -1,   0 },

{   0,   1,  -8,  -2,  21,  46,  49,  28,   2,  -8,  -1,   0 },

{   0,   1,  -8,  -3,  19,  45,  50,  30,   4,  -8,  -2,   0 },

{   0,   2,  -8,  -4,  18,  43,  50,  32,   5,  -8,  -2,   0 },//phase 0.75

{   0,   2,  -7,  -5,  16,  42,  51,  33,   6,  -8,  -2,   0 },

{   0,   1,  -7,  -5,  14,  41,  51,  35,   8,  -7,  -3,   0 },

{   0,   2,  -7,  -6,  13,  40,  51,  37,   9,  -7,  -4,   0 }

};

## A.4		Additional results

Several different sets of filters have been evaluated. The different results are reported in the tables below. The reference is the default HM5.0.

(Cx=0, Cy=0)

- Test A1:	Top : [1,2,1] horiz	Left : [1] vertic		Inner : [1] horiz

- Test A2:	Top : [1,2,1] horiz	Left : [1,2,1] vertic	Inner : [1,2,1] horiz

| Phase input sequences |  | A1 |  |  | A2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cx | Cy | Y | U | V | Y | U | V |
| 0 | 0 | 0.0 | -1.1 | -1.5 | -0.1 | -1.1 | -1.5 |
| 0.25 | 0 | 0.0 | -0.9 | -1.4 | -0.1 | -1.4 | -1.5 |
| 0.5 | 0 | 0.0 | -0.8 | -1.3 | -0.1 | -1.3 | -1.2 |
| 0.75 | 0 | 0.0 | -0.7 | -1.1 | -0.1 | -1.2 | -1.0 |
| 0 | 0.5 | 0.4 | 6.0 | 2.8 | 0.3 | 4.9 | 2.6 |
| 0.25 | 0.5 | 0.4 | 5.9 | 2.8 | 0.3 | 5.0 | 2.6 |
| 0.5 | 0.5 | 0.4 | 5.6 | 2.4 | 0.3 | 4.8 | 2.5 |
| 0.75 | 0.5 | 0.4 | 4.7 | 1.9 | 0.3 | 4.2 | 2.1 |

A2 is recommended because it enables reducing aliasing effects on all samples (top, left, inner). It gives slightly better performance than A1.

(Cx=0.5, Cy=0)

- Test B1:	Top : [1,3,3,1] horiz	Left : [1] vertic		Inner : [1,1] horiz

- Test B2:	Top : [1,1] horiz	Left : [1,2,1] vertic	Inner : [1,1] horiz

| Phase input sequences |  | B1 |  |  | B2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cx | Cy | Y | U | V | Y | U | V |
| 0 | 0 | 0.0 | 0.3 | 0.3 | 0.0 | 0.3 | 0.3 |
| 0.25 | 0 | -0.1 | -1.3 | -1.6 | 0.0 | -1.1 | -1.4 |
| 0.5 | 0 | -0.2 | -2.9 | -2.8 | -0.1 | -2.8 | -2.7 |
| 0.75 | 0 | -0.2 | -4.3 | -3.5 | -0.2 | -4.2 | -3.5 |
| 0 | 0.5 | 0.4 | 7.2 | 3.8 | 0.4 | 7.3 | 3.8 |
| 0.25 | 0.5 | 0.3 | 5.1 | 2.5 | 0.3 | 5.2 | 2.6 |
| 0.5 | 0.5 | 0.2 | 2.9 | 0.9 | 0.2 | 3.0 | 1.1 |
| 0.75 | 0.5 | 0.1 | 0.9 | -0.2 | 0.2 | 0.9 | -0.2 |

B2 is similar to the filters used in the default HM (with top and left filters being permuted). Performance is slightly lower than B1, but the difference does not justify the usage of B1.

B2 is therefore recommended.

(Cx=0.5, Cy=0.5)

- Test C1:	Top : [1,3,3,1] hor	Left : [1,3,3,1] vert 	Inner : [1,1; 1,1]

- Test C2:	Top : [1,1,1,1] hor	Left : [1,1,1,1] vert 	Inner : [1,1; 1,1]

- Test C3:	Top : [1,1] horiz	Left : [1,1] vertic	Inner : [1,1; 1,1]

- Test C4:	Top : [1,1] hor		Left : [1,1] vert 		Inner : [1,0; 0,1]

| Phase input sequences |  | C1 |  |  | C2 |  |  | C3 |  |  | C4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cx | Cy | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| 0 | 0 | 0.1 | 1.6 | 1.1 | 0.1 | 1.5 | 1.0 | 0.0 | 1.8 | 1.1 | 0.1 | 2.4 | 1.3 |
| 0.25 | 0 | 0.0 | -0.2 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.7 | 0.2 | 0.0 | 0.9 | 0.3 |
| 0.5 | 0 | -0.1 | -2.0 | -1.3 | -0.1 | -2.0 | -1.4 | -0.1 | -1.2 | -1.1 | -0.1 | -0.8 | -0.9 |
| 0.75 | 0 | -0.2 | -3.4 | -2.3 | -0.2 | -3.4 | -2.3 | -0.2 | -2.8 | -2.1 | -0.2 | -2.3 | -1.8 |
| 0 | 0.5 | 0.0 | 1.6 | 1.4 | 0.1 | 1.5 | 1.4 | 0.1 | 2.6 | 1.6 | 0.1 | 3.1 | 1.9 |
| 0.25 | 0.5 | 0.0 | -0.6 | -0.2 | 0.0 | -0.6 | -0.2 | 0.0 | 0.5 | 0.1 | 0.0 | 1.0 | 0.4 |
| 0.5 | 0.5 | -0.1 | -2.7 | -1.9 | -0.1 | -2.6 | -1.9 | -0.1 | -1.6 | -1.5 | -0.1 | -1.1 | -1.3 |
| 0.75 | 0.5 | -0.2 | -4.3 | -2.9 | -0.2 | -4.2 | -2.8 | -0.2 | -3.4 | -2.5 | -0.2 | -2.9 | -2.3 |

C1 and C2 provide the best results. C2 is simpler since it does not involve any multiplication and it is almost as performing as C1. C2 is therefore recommended.

Possibly, inner filter of C4 could be used as a simplified version.