| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0178_r1 |
| --- | --- |

| Title: | Encoder speed-up for the Motion Vector predictor cost estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Tangi POIRIER Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents a non-normative change regarding the introduction of an Early Termination rule. This Early Termination rule deals with the computation of the rate-distortion cost of the motion vector predictors at the encoder side. More precisely, a termination rule is proposed to avoid estimating all the rate-distortion costs of the Merge candidates. It is reported that with this early termination rule, an average decrease of 5% of the encoding time is observed without any impact on the BDR measure (average BDR gain of 0%).

# Introduction

## Encoding time reduction

This section presents an encoder simplification in order to reduce the encoding time induced by the numerous candidates in the AMVP and Merge/Skip list. We have observed that the encoding time dedicated to the rate-distortion cost determination for the Merge/Skip list is quite important. This is mainly due to the rate-distortion cost estimation for the Merge mode for each candidate of the list. Indeed, for each candidate of the Merge mode, the rate distortion cost estimation is computed by coding the residual data and by checking a quad-tree of transforms. This computation is quite costly compared to the Skip mode where no residual data are encoded.

In the following section 1.3, we present an early termination rule when estimating the rate distortion cost of the Merge candidates. This avoids estimating all the rate-distortion cost of the Merge candidates without affecting the coding efficiency.

## HM5.0 rate-distortion estimation for the Skip/Merge candidates.

Figure 1 presents the current encoder algorithm to estimate the rate distortion cost for each Merge/Skip candidate in the HM4.0. This diagram is very simple and shows that all predictors are exhaustively tested through the Skip and the Merge mode. Please note that for the Merge mode, the rate-distortion cost computation is more important since the residual data is encoded by checking different transforms of different size.

Figure 1: HM5.0 rate-distortion cost estimation procedure for the list of Skip/Merge candidates.

## Proposed modification for the Merge mode rate distortion computation (fast encoding mode)

presents the proposed encoder algorithm change to avoid some rate distortion cost evaluations for some Merge candidates. Instead of performing systematically the rate-distortion cost of each candidate corresponding to the right part of the Figure 1, an early termination rule is applied. The diagram in  uses a boolean variable to signal the Early Termination for Merge (ETM). When the condition is reached i.e (ETM==TRUE), the computation of the rate-distortion cost of the Merge mode for a given candidate is not performed.

The basic rule is the following: the Early Termination for Merge variable is set to TRUE when for any candidate of the list, the cost (JSkipi) of the Skip mode is inferior to the minimum of any previous rate-distortion costs already computed for previous candidates.

Then when next predictors are under rate-distortion cost estimation process and when the ETM is equal to TRUE, we do not need to estimate the rate-distortion cost of the Merge mode.

Figure 2: Proposed RD cost estimation procedure for Skip/Merge candidates to save encoding time.

# Source code modification

The modification of the proposed encoding algorithm concerns about 10 lines of source codes in HM5.0. As it is a non-normative change, the decoder remains unchanged.

# Results

Table 1 presents the results of the proposed modification in HM5.0 under the common testing conditions [1]. Results in Table 1 show no impact in terms of coding efficiency since the BDR gain is about 0% on average. The encoding time of HM5.0 is reduced by 5% on average. This encoding time decrease can reach up to 8% for the Low Delay P case.

Table 1: Results on HM5.0 of the early termination rule.

# Conclusion

This contribution presents a non-normative change to introduce an early termination rule to avoid the rate-distortion cost estimation regarding the Merge mode for some motion predictor candidates. Results provided in this contribution show that the encoding time is decreased by 5%. The impact on the coding efficiency is negligible.

Finally, we recommend this simple non-normative modification in the encoder in the next HM6.0 software to speed-up HM encoder.

# Reference

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).