| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0179 |
| --- | --- |

| Title: | Storage reduction in dependent tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Stephane PAUTET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the dependent tiles design to limit the storage issue due to dependencies across vertical tile boundaries. The proposal is similar to the solution used for horizontal LCU borders. Motion data of 4x4 blocks adjacent to right borders of tiles are downsampled. Regarding intra mode, mode of PUs on left borders of a tile is forced to Planar. The proposal has been evaluated using tests conditions used in JCTVC-F335. It is reported that no coding impact is observed for all test configurations, while half of the memory is saved for motion data, and no memory is required for intra mode data.

# Introduction

HEVC specifies two types of tiles.

- ‘independent tiles’ that break any prediction mechanisms across tiles boundaries, which means that Coding Units within a tile cannot depend on any data from other tiles.

- ‘dependent tiles’. In dependent tiles, prediction mechanisms are still active across tile boundaries. In particular, CUs within a tile and located along the left border of the tile can depend from their neighboring left coding units, located outside the tile.

As mentioned in JCTVC-G110, the usage of dependent tiles implies the storage of data from all the CUs located on the right border of each tile which has a tile at its right side. Long-term storage must be available for all the horizontally adjacent neighbors on the boundary. In the worst case, the decoder will have to store neighbor data for a whole vertical column of LCUs.

This proposal aims at addressing this issue by using a similar solution as the one used for motion data and intra modes along horizontal borders of LCU.

# Proposal description

## Reducing column buffer for motion data

As in JCTVC-F060, it is proposed to store a downsampled version of the motion data. Motion data of one 4x4 block over two is stored. When a PU on the right side of the vertical tile border needs to access motion data of a left side non-stored block, the motion data of its neighboring upper 4x4 block are actually used, as depicted in Figure 1.

Figure 1: proposed vertical downsampling pattern for 4x4 motion data blocks.

## Reducing column buffer for intra mode

As in JCTVC-G145, it is proposed to remove the whole tile column buffer for intra mode. When a PU on the right side of the vertical tile border needs to access the intra mode of a left non-stored block, the intra mode of left neighbor is assigned to the planar mode.

# Results

Tests were performed using the tests conditions specified in JCTVC-F335. In particular, the following tile partitioning has been used.

Table 1: tile partitioning for proposal evaluation.

|  | Reference |  | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Number of slices | Slice sizes (in units of LCUs) | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
|  |  |  | horizontally | vertically | horizontally | vertically |
| A | 25 | 40 | 5 | 5 | 8 | 5 |
| B | 17 | 30 | 6 | 3 | 5 | 6 |
| C | 4 | 26 | 2 | 2 | 7 | 4 |
| D | 1 | 28 | 1 | 1 | 7 | 4 |
| E | 12 | 20 | 4 | 3 | 5 | 4 |

Full results are provided in the attached xls file. It is observed that the proposed solution has a negligible impact on the coding efficiency.

# Conclusion

It is proposed to reduce the memory needs for dependent tiles by subsampling the stored motion data at the right border of tiles, and fully removing intra mode storage. The proposal has no impact on coding efficiency nor on visual quality. We recommend adopting the two simplifications in the next HEVC test model.

# References

- Wade K. Wan, Tim Hellman, “Implication of parallelized bitstreams on single core decoder architectures,” in JCTVC-G110, Geneva, Switzerland, Nov., 2011.

- Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, and Shawmin Lei, “Reducing Line Buffers for Motion Data and CABAC,” in JCTVC-F060, Torino, Italy, July, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).