| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0180_r2 WG11 Number: m23052 |
| --- | --- |

| Title: | Non-CE3: Phase adjustment of interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

New interpolation filter coefficients for quarter pixel position for luminance, eighth pixel and three-eighth pixels position for chrominance have been introduced at previous Geneva meeting.  However, the phase shift of those filter coefficients of quarter pixel position for luminance and three-eighth pixels position for chrominance are not accurate fractional pixel positions.  Therefore, in this contribution, two filter coefficients are designed to adjust accurate fractional pixel position and their experimental results are reported.  As experimental results, all average of coding gains are almost zero; however, an average of 1.2% gain has been achieved in the case of low delay P on low complexity condition.  The computational complexities have been reduced about 2% on average and the results of memory bandwidth have been reduced about 1% on average.

# Introduction

According to past results of CE3 [1], it is understood that the phase shift of interpolation filter does not need to be identical to accurate fractional pixel position due to improve coding efficiency.  However, considering robust design under various conditions, accurate phase shift of interpolation filter must be important.  Therefore, in this contribution, filter coefficients, the phase shift of which are not identical to accurate fractional pixel position, are redesigned.

# Phase shift of fractional pixel position

Table 1 indicates interpolation filter coefficients for luminance and chrominance that are introduced to current HM5 [2].

Those phase shift of coefficients for quarter pixel position for luminance and three-eighth pixel position for chrominance are not identical to fractional pixel position.  Therefore, we improve filter coefficients in order to adjust phase shift as shown in Table 2.

Table 1	 Interpolation filter coefficients on HM5

|  | Fractional position | Filter coefficients | Phase shift (DC) |
| --- | --- | --- | --- |
| Luma | 1/4 | -1, 4, -10, 58, 17, -5, 1, 0 | 15/64 |
|  | 1/2 | -1, 4, -11, 40, 40, -11, 4, -1 | 32/64 |
| Chroma | 1/8 | -2, 58, 10, -2 | 8/64 |
|  | 1/4 | -4, 54, 16, -2 | 16/64 |
|  | 3/8 | -6, 46, 28, -4 | 26/64 |
|  | 1/2 | -4, 36, 36, -4 | 32/64 |

Table 2	 Improved interpolation filter coefficients

|  | Fractional position | Filter coefficients | Phase shift (DC) |
| --- | --- | --- | --- |
| Luma | 1/4 | -1, 4, -10, 57, 18, -5, 1, 0 | 16/64 |
|  | 1/2 | -1, 4, -11, 40, 40, -11, 4, -1 | 32/64 |
| Chroma | 1/8 | -2, 58, 10, -2 | 8/64 |
|  | 1/4 | -4, 54, 16, -2 | 16/64 |
|  | 3/8 | -6, 48, 26, -4 | 24/64 |
|  | 1/2 | -4, 36, 36, -4 | 32/64 |

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-G1200 [3].  Table 3 indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-G1200, the anchor is HM5.0.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.

According to previous CE3 [4], the computational complexity and memory bandwidth analyses have been conducted.

Table 3	Summary of results

|  | Y | U | V |
| --- | --- | --- | --- |
| Gain Ave.[%] | 0.29% | 0.05% | 0.09% |
| Gain Best [%] | -0.14% | -0.96% | -0.55% |
| Gain Worst [%] | 3.01% | 2.64% | 2.34% |
| Gain Ave.[%] (Full) | 0.07% | -0.06% | 0.03% |
| Gain Best [%] (Full) | -4.50% | -3.69% | -1.64% |
| Gain Worst [%] (Full) | 3.01% | 2.64% | 4.46% |
| Enc Time[%] | 100.06% |  |  |
| Dec Time[%] | 100.40% |  |  |
| Mult Ave.[%] | -2.09% |  |  |
| Adds Ave.[%] | -2.12% |  |  |
| Access[%] | -1.21% |  |  |
| MemBand(2D) Ave.[%] | -0.65% |  |  |
| MemBand(1D) Ave.[%] | -0.65% |  |  |

Table 4	Results of full condition

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% | -0.1% | -0.5% | -0.3% | 0.0% | -0.1% | -0.1% |
| Class B | 0.1% | -0.1% | -0.1% | 0.2% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.2% | 0.0% | 0.0% | 0.4% | 0.1% | 0.0% | 0.3% | 0.1% | 0.0% |
| Class D | 0.4% | 0.0% | 0.1% | 0.9% | 0.3% | 0.3% | 0.4% | 0.3% | 0.5% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall | 0.1% | -0.1% | -0.1% | 0.3% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
|  | 0.1% | -0.1% | -0.1% | 0.3% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  | 97% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 94% |  |  | 100% |  |  |
| Mults | -2% |  |  | -2% |  |  | -2% |  |  |
| Adds | -2% |  |  | -2% |  |  | -2% |  |  |
| MemAccess | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand2D | 0% |  |  | -1% |  |  | 0% |  |  |
| Avg. MemBand1D | 0% |  |  | -1% |  |  | 0% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.4% | 0.3% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.2% | -0.1% | 0.0% | 0.5% | -0.1% | 0.1% | 0.2% | 0.0% | 0.0% |
| Class D | 0.5% | 0.2% | 0.6% | 1.0% | 0.8% | 0.4% | 0.3% | -0.6% | 0.1% |
| Class E | 0.2% | 0.0% | 0.0% | 0.4% | 0.5% | 0.5% | 0.1% | -0.4% | 0.2% |
| Class F | -0.1% | -0.3% | -0.6% | 0.1% | -1.1% | -0.7% | 0.1% | -0.2% | -0.2% |
| Overall | 0.2% | -0.1% | 0.1% | 0.5% | 0.0% | 0.0% | 0.2% | -0.2% | 0.0% |
|  | 0.2% | -0.1% | 0.1% | 0.5% | 0.1% | 0.1% | 0.2% | -0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Mults | -2% |  |  | -2% |  |  | -2% |  |  |
| Adds | -2% |  |  | -2% |  |  | -2% |  |  |
| MemAccess | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand2D | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand1D | -1% |  |  | -1% |  |  | 0% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | -2.0% | 0.3% | -0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.4% | 0.0% | -0.2% | -0.8% | -0.4% | 0.1% | 0.3% | -0.1% | 0.2% |
| Class D | 0.5% | 0.3% | 0.0% | -0.3% | -0.3% | 1.4% | 0.5% | -0.1% | 0.5% |
| Class E | 0.1% | 0.3% | -0.3% | -3.4% | -0.6% | -0.5% | 0.1% | -0.2% | -0.5% |
| Class F | 0.0% | -0.2% | 0.0% | 0.1% | -0.8% | -0.1% | -0.1% | 0.0% | -0.1% |
| Overall | 0.2% | 0.1% | -0.1% | -1.2% | -0.3% | 0.2% | 0.2% | -0.1% | 0.1% |
|  | 0.2% | 0.1% | -0.2% | -1.2% | -0.3% | 0.1% | 0.2% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Mults | -2% |  |  | -1% |  |  | -2% |  |  |
| Adds | -2% |  |  | -1% |  |  | -2% |  |  |
| MemAccess | -1% |  |  | -1% |  |  | -1% |  |  |
| Avg. MemBand2D | 0% |  |  | 0% |  |  | -1% |  |  |
| Avg. MemBand1D | 0% |  |  | 0% |  |  | 0% |  |  |

# Subjective quality assessment

According to previous CE3 condition, subjective qualities of decoded sequences were assessed.  It seems to be difficult to find the difference of subjective quality between proposals and HM50 anchor.

# Conclusions

In this contribution, some interpolation filter coefficients have been designed to adjust accurate fractional pixel position and their experimental results have been reported.

# References

[1] 	T. Chujoh and E. Alshina, “CE3: Summary report of Core Experiment on Motion Compensation,” Joint Collaborative Team on Video Coding, JCTVC-G033, Geneva, Dec. 2011.

[2] 	B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” Joint Collaborative Team on Video Coding, JCTVC-G1103, Geneva, Dec. 2011.

[3] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-G1200, Geneva, Dec. 2011.

[4] 	T. Chujoh and E. Alshina, “CE3: Motion compensation,” Joint Collaborative Team on Video Coding, JCTVC-F903, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).