| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0181_r3 WG11 Number: m23x053 |
| --- | --- |

| Title: | AHG07: A restriction of motion vector for small PU size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

An experimental result of restriction of motion vector for small PU size is reported.  This is a non-normative technology to reduce memory bandwidth for motion compensation.  The worst cases of memory bandwidth of interpolation process are two-dimensional interpolation positions for both Luma and Chroma of bi-prediction PU.  Therefore, in order to reduce the worst case of memory bandwidth, for example, an encoding method that at least one motion vector of L0 or L1 is restricted to the integer position for both Luma and Chroma is introduced.  As an experimental result, the loss of coding efficiency is an average of 0.45% and this value is smaller than the result of prohibition of both 4x8 and 8x4 bi-prediction.  Their worst memory bandwidths are almost the same.

# A restriction of motion vector for small block size

The interpolation filters on current HM are specified 8 or 7-tap for luminance and 4-tap for chrominance.  Considering block based processing, memory access of smaller block is increased.  Therefore, in order to reduce memory bandwidth, the restriction of small prediction block size is a useful non-normative method.

When color format is 4:2:0, the resolutions of interpolation of HM are quarter for luminance signal and eighth chrominance signal.  As shown Figure 1, when the luminance is the integer position or the one-dimensional interpolation position, there are cases that the chrominance pixel is a two-dimensional interpolation position.  In Figure 1, “A” indicates the integer position for luminance and “B” indicates the integer position for chrominance.  White “A” is the integer position for both luminance and chrominance and gray “A” is the integer position for luminance, however, the two-dimensional interpolation position for chrominance.  White blocks are the one-dimensional interpolation position for both luminance and chrominance and gray blocks are the two-dimensional interpolation position for both luminance and chrominance.  Black blocks are the one-dimensional interpolation position for luminance and the two-dimensional position for chrominance.  The worst cases of memory bandwidth of interpolation process are two dimensional interpolation positions for both luminance and chrominance of bi-prediction block.

Therefore, if we restrict motion vectors in order to reduce memory bandwidth, we should consider the fractional position for chrominance.

Interpolation position for luminance			Interpolation position for chrominance

Figure 1  Relationship of interpolation position for luminance and chrominance (4:2:0)

In this contribution, we test an  example of MV restriction as follows.

- 4x8 bi-prediction is prohibited.

- One motion vector of L0 or L1 shall indicate the integer position for chroma when PU size is 8x4 bi-prediction.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-G1200 [1].  Table 1 indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-G1200, the anchor is HM50.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.

Previous CE3 [2] has conducted computational complexity and memory bandwidth analyses for each proposal.  At this time, computational operations and memory bandwidth for each block size and for each single and bidirectional prediction are measured.  This proposal can reduce worst case and average of computational operations and also average of memory bandwidth.

Table 1	Results of summary

|  | Proposal |  |  | 8x4 and 4x8 bi-prediction prohibition |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Gain Ave.[%] | 0.45% | 0.31% | 0.32% | 0.48% | 0.40% | 0.37% |
| Gain Best [%] | 0.01% | -0.78% | -0.85% | 0.01% | -0.17% | -0.81% |
| Gain Worst [%] | 1.34% | 2.26% | 1. 47% | 1.29% | 1.56% | 1.65% |
| Gain Ave.[%] (Full) | 0.23% | 0.18% | 0.16% | 0.26% | 0.21% | 0.21% |
| Gain Best [%] (Full) | -0.54% | -0.87% | -1.74% | -0.91% | -1.07% | -2.00% |
| Gain Worst [%] (Full) | 1. 34% | 2.26% | 1.47% | 1.34% | 1.56% | 1.91% |
| Enc Time[%] | 95.80% |  |  | 92.27% |  |  |
| Dec Time[%] | 100. 76% |  |  | 99.85% |  |  |
| Mult Ave.[%] | -1.74% |  |  | -0.70% |  |  |
| Mult Worst [%] | -22.81% |  |  | -22.81% |  |  |
| Adds Ave.[%] | -1.75% |  |  | -0.70% |  |  |
| Adds Worst [%] | -22.70% |  |  | -22.70% |  |  |
| Access[%] | -1.98% |  |  | -1.58% |  |  |
| MemBand(2D) Ave.[%] | -2.48% |  |  | -2.52% |  |  |
| MemBand(2D) Worst [%] | -35.23% |  |  | -36.36% |  |  |
| MemBand(1D) Ave.[%] | -2.35% |  |  | -2.39% |  |  |
| MemBand(1D) Worst [%] | -31.25% |  |  | -31.25% |  |  |

# Subjective quality

It seems to be difficult to find the difference of subjective quality between proposals and HM50 anchor.

# Conclusions

The simulation results of restriction of motion vector for small PU size have been shown.  As experimental results the degradation of coding efficiency is small and the worst case of memory bandwidth is improved.  When the worst case of memory bandwidth is reduced, motion vectors should be restricted, considering Chroma fractional positions.

# References

[1] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-G1200, Geneva, Dec. 2011.

[2] 	T. Chujoh and E. Alshina, “CE3: Motion compensation,” Joint Collaborative Team on Video Coding, JCTVC-F903, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).