| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0183 WG11 Number: m23056 |
| --- | --- |

| Title: | Non-CE8: Cross-check report for sample adaptive offset with constrained ranges of edge offset (JCTVC-H0114) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-checking results of sample adaptive offset with constrained ranges of edge offset (JCTVC-H0114) are shown.  A source code package was distributed by proponents and it was run on an independent platform.  All local decoded images by encoder matched decoded images by decoder perfectly.  All bitrate and PSNR were identical with proponent's results perfectly.  Detailed experimental results show attached Excel files.  As a subjective quality assessment, it was found that the salt and pepper noises are improved.  Non-CE8: sample adaptive offset with constrained ranges of edge offset (JCTVC-H0114) has been cross-verified independently.

# Software check

The values of edge offset are restricted and the syntax is changed.  Basically, they are minor modifications.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-G1200.  Error: Reference source not found indicates the average results of coding efficiency compared to each anchor.  The anchor conditions are basically JCTVC-G1200; however the condition of low complexity is included in SAO.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.

Table 1	Summary of results

|  | All Intra HE |  |  | All Intra LC+SAO |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.3% | -0.3% | 0.0% | -0.4% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC+SAO |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.5% | -0.1% | -0.1% | -0.4% | 0.1% | 0.1% | 0.1% |
| Class B | -0.1% | -0.4% | -0.4% | -0.1% | -0.2% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class C | -0.1% | -0.5% | -0.4% | -0.1% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.5% | -0.6% | 0.0% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Overall | -0.1% | -0.4% | -0.5% | -0.1% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% |
|  | -0.1% | -0.4% | -0.4% | -0.1% | -0.2% | -0.3% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC+SAO |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class C | -0.1% | -0.5% | -0.6% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class D | -0.1% | -0.6% | -0.6% | 0.0% | -0.5% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class E | -0.1% | -0.4% | -1.1% | -0.2% | -0.4% | -0.5% | 0.0% | -0.5% | 0.3% |
| Class F | -0.4% | -0.5% | -0.6% | -0.1% | -0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
| Overall | -0.2% | -0.5% | -0.6% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.1% |
|  | -0.2% | -0.4% | -0.6% | -0.1% | -0.3% | -0.2% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC+SAO |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.5% | -0.7% | -0.2% | -0.4% | -0.3% | 0.0% | 0.1% | 0.0% |
| Class C | -0.1% | -0.5% | -0.7% | -0.1% | -0.2% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class D | -0.1% | -0.4% | -0.9% | -0.2% | -0.2% | -0.8% | 0.0% | -0.3% | 0.1% |
| Class E | -0.2% | -0.9% | -1.0% | -0.2% | 0.6% | -1.5% | -0.1% | -0.3% | 0.1% |
| Class F | -0.2% | -0.2% | -0.4% | -0.2% | -0.1% | -0.4% | -0.1% | 0.3% | 0.1% |
| Overall | -0.2% | -0.5% | -0.7% | -0.2% | -0.1% | -0.5% | -0.1% | 0.0% | 0.1% |
|  | -0.2% | -0.5% | -0.7% | -0.2% | -0.1% | -0.4% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |

# Subjective quality assessment

Several sequences have been decoded and assessed their subjective qualities.  As proponents say, it was found that the salt and pepper noises are improved.  Typical decoded still images of both proposal and anchor are attached.

# Conclusions

Non-CE8: sample adaptive offset with constrained ranges of edge offset (JCTVC-H0114) that is proposed by Samsung, Qualcomm and MediaTeK has been cross-verified independently.

# References

[1] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-G1200, Geneva, Dec. 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).