| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0184 |
| --- | --- |

| Title: | CE4 Subtest 3: Cross-verification report for test 3.1.a (JCTVC-H0506) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a study and cross-verification results of TU-level dQP signalling, which is proposed in JCTVC-H0506 and tested as test 3.1.a in CE4 Subtest 3. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-H0506 and results exactly match those provided by the proponent.

# Software analysis

Software was provided by the proponent and it was successfully compiled by Microsoft Visual C++ 2008. It was confirmed that the software implementation conforms to the description in JCTVC-G721 [1] and JCTVC-H0506 [2].

# Remarks

The implementation by the proponent extends the syntax element cu_qp_delta_enabled_flag in SPS of HEVC WD3 [3] for representing availability of the functionality of TU-level dQP signalling. In the latest WD of HEVC [4], the syntax element cu_qp_delta_enabled_flag in SPS is removed and incorporated into the syntax element max_cu_qp_delta_depth in PPS. If the JCTVC-H0506 proposal is adopted into HEVC, the syntax of the proposal should be harmonized with the latest HEVC syntax.

# Simulation results

The experiment to verify simulation results provided by the proponent was conducted under the CE4 Subtest 1 test conditions described in JCTVC-G1204 [5]. The computing platform used for the verification is shown below.

| Computing platform | Usage |
| --- | --- |
| OS: Windows 7 64-bit Professional CPU: Xeon X5680 3.33GHz (12 cores) Memory: 32GiB | Up to 12 processes for encoding and decoding ran at the same time. |

Table 1 shows the summary of results. An attached spreadsheet, JCTVC-H0184.xls, contains the detailed results. It was confirmed that BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent.

Table 1: Summary results against test 1.1 (CE4 anchor).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 0.8% | 2.9% | 3.0% | 10.5% | 1.9% | 0.8% | 0.4% | 24.9% |  |  |  |  |
| Class B | 1.0% | 2.4% | 2.7% | 14.6% | 1.8% | 0.9% | 0.8% | 30.7% |  |  |  |  |
| Class C | 0.5% | 1.6% | 1.7% | 7.1% | 0.9% | 0.7% | 0.6% | 12.5% |  |  |  |  |
| Class D | 0.4% | 1.1% | 1.2% | 6.1% | 0.6% | 0.4% | 0.3% | 9.5% |  |  |  |  |
| Class E | 1.0% | 3.5% | 3.1% | 9.3% | 1.7% | -0.1% | 0.4% | 17.1% |  |  |  |  |
| Overall | 0.7% | 2.2% | 2.2% | 9.7% | 1.3% | 0.5% | 0.5% | 19.0% |  |  |  |  |
|  | 0.7% | 2.2% | 2.2% |  | 1.3% | 0.6% | 0.6% |  |  |  |  |  |
| Class F | 0.5% | 1.6% | 1.7% | 5.5% | 1.4% | 0.2% | 0.4% | 10.7% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 102% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 1.0% | 2.5% | 2.5% | 10.5% | 1.0% | 0.3% | 0.0% | 13.4% | 1.7% | 0.7% | 0.9% | 22.2% |
| Class B | 1.2% | 2.4% | 2.9% | 14.4% | 1.5% | 0.9% | 0.9% | 18.0% | 1.3% | 2.4% | 3.3% | 14.9% |
| Class C | 0.7% | 1.4% | 1.3% | 9.0% | 0.4% | 0.1% | 0.3% | 8.6% |  |  |  |  |
| Class D | 0.5% | 1.0% | 1.0% | 8.5% | 0.1% | -0.3% | 0.0% | 7.3% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.9% | 1.8% | 1.9% | 10.8% | 0.7% | 0.3% | 0.4% | 12.0% | 1.5% | 1.7% | 2.2% | 18.2% |
|  | 0.9% | 1.8% | 1.9% |  | 0.7% | 0.3% | 0.4% |  | 1.4% | 1.7% | 2.2% |  |
| Class F | 0.6% | 1.7% | 1.8% | 7.8% | 0.8% | 0.6% | 0.9% | 9.0% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.8% | 4.1% | 5.0% | 13.7% | 1.6% | 2.4% | 2.9% | 16.0% |  |  |  |  |
| Class C | 1.2% | 2.0% | 2.1% | 9.6% | 0.4% | 1.1% | 1.4% | 8.4% |  |  |  |  |
| Class D | 0.9% | 2.1% | 2.6% | 10.8% | 0.1% | 0.6% | 0.3% | 7.9% |  |  |  |  |
| Class E | 2.2% | 5.4% | 4.9% | 9.0% | 1.8% | 0.9% | 0.7% | 7.6% |  |  |  |  |
| Overall | 1.5% | 3.3% | 3.6% | 11.1% | 1.0% | 1.4% | 1.5% | 10.5% |  |  |  |  |
|  | 1.5% | 3.4% | 3.5% |  | 0.9% | 1.4% | 1.4% |  |  |  |  |  |
| Class F | 1.0% | 1.8% | 1.8% | 12.9% | 1.0% | 1.3% | 1.8% | 12.0% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |  |  |  |  |

# Conclusion

This contribution has presented a study and cross-verification results of TU-level dQP signaling, which is proposed in JCTVC-H0506 and tested as test 3.1.a in CE4 Subtest 3. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-H0506 and results exactly match those provided by the proponent.

# References

[1] X. Fang, et al., “CE4 Subtest 1.1.a: QP adaptation at sub-CU level,” JCT-VC document JCTVC-G721, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.

[2] J.-H. Kim, et al., “CE4 Subtest 3.1.a: QP adaptation at sub-CU level,” JCT-VC document JCTVC-H0506, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th meeting, San Jose, February 2012.

[3] T. Wiegand, et al., “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC document JCTVC-E603, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[4] B. Bross, et al., “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCT-VC document JCTVC-G1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.

[5] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-G1204, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.