| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0185 |
| --- | --- |

| Title: | Cross-check of Adaptive Resolution Coding (ARC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Glenn Van Wallendael Sebastiaan Van Leuven, Jan De Cock, Rik Van de Walle Gaston Crommenlaan 8 P.O. Box 201 9050 Ghent-Ledeberg, Belgium | Tel: Email: | +32 93314957 glenn.vanwallendael@ugent.be |
| Source: | Ghent University - IBBT |  |  |

_____________________________

# Abstract

This contribution provides a cross verification of the Adaptive Resolution Coding proposal of Cisco Systems UK (JCTVC-H0234). The aim of ARC is to allow a stream to change between coding pictures at two (or more) resolutions, without requiring a new SPS and IDR frame and without requiring layering as in SVC. The allowable resolutions proposed are full-size (WxH) and quarter-size pictures (W/2xH/2), although the principle could certainly be extended. Apart from rerunning the software to obtain bit exact results, some further inspection on the code and the used test setup was carried out. The code fully corresponds to the description in the proposal. The obtained test results are generated using conditions different from the common conditions. The tested QP range was 32, 35, 39, 42. A change from high to low resolution or the other way around was initiated on the 17th picture. Finally, PSNR and BD measurements were calculated only on the pictures succeeding the resolution change region. The conclusion was that, in the proposed scenario, these test conditions provide more relevant results than when tested under common conditions.

# Code

The code is conforming the proponants description.

# Test conditions

The used test conditions differentiate from the common conditions in the following ways:

- QPs are changed from 22, 27, 32, 37 to QPs of 32, 35, 39, 42. 
For the proposed method, this change is justifiable because when bandwidth availability on the network reduces, the first step an encoder would take is increasing the QP. When the situation gets more critical, a resolution decrease can be performed. As a result, it is more relevant to do simulations on a lower quality than the common conditions.

- A transition from high to low resolution or from low to high resolution is always made on the 17th picture position. For this point as well, this corresponds with the scenario that is described and is therefore a relevant way to test the scenario.

- PSNR and BD measurements were calculated only on the pictures starting from the 17th picture. For the calculation of encoding and decoding times, the total time spent on all pictures is used.

Although these 3 points are remarked, it must be added that they are well described in the proposal.

Our computing platform used for generating the results is shown below.

## Encoding

Super computer with 167 computing nodes (IBM iDataPlex dx360M2), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5520 (Intel Nehalem microarchitecture, 2.27 GHz, 8 MB L3 cache per quad-core chip), thus 8 cores / node

- memory: 12 GB RAM (DDR3 PC3-8500S 1067 MHz)

- storage (local): one 156 GB 3.5" 15k RPM SAS disk

and 194 computing nodes (IBM HS 21 XM blade), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5420 (Intel Core microarchitecture, 2.5 GHz, 6 MB L2 cache per quad-core chip), thus 8 cores / node

- memory: 16 GB RAM (DDR2 FB-DIMM PC-5300 CL5)

- storage (local): one 146 GB 2.5" 10k RPM SAS disk

The encoder executables were generated with g++ 4.1.2.

## Decoding

- CPU: quad-core Intel Core 2 Q6600 (Intel Core microarchitecture, 2.40GHz, 8MB L2 cache)

- Memory: 4 GB RAM (DDR2 PC2-6400 400MHz)

# Simulation results

Bitrates and PSNR values resulting from the simulation exactly match the proposal. Encoding and decoding times vary in the usual range caused by a computing cluster.

## Linear down-sampling

|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,3% | 5,9% | 3,6% | 1,1% | 7,5% | 9,8% | 0,2% | 5,2% | 2,2% |
| Class B | 0,5% | 1,2% | 2,5% | 1,0% | 2,6% | 3,1% | 0,5% | 0,2% | 1,6% |
| Class C | 1,4% | 2,1% | 2,7% | 1,4% | 2,8% | 3,8% | 1,6% | 3,3% | 2,9% |
| Class D | 0,7% | 2,6% | -0,5% | 0,8% | 2,0% | 2,1% | 0,5% | 0,2% | -0,6% |
| Class E | 3,0% | 1,3% | 1,1% | 4,7% | 0,9% | 4,3% | 2,7% | -1,2% | 1,8% |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |
| Overall | 1,1% | 2,6% | 1,9% | 1,6% | 3,2% | 4,6% | 1,0% | 1,6% | 1,6% |
|  | 1,1% | 2,6% | 1,9% | 1,6% | 3,3% | 4,9% | 1,0% | 1,5% | 1,6% |
| Enc Time[%] | 96% |  |  | 94% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 104% |  |  | 104% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0,4% | 6,7% | 4,6% | -1,3% | 8,6% | 5,0% |  |  |  |
| Class B | 0,9% | 2,3% | 2,5% | 0,4% | 3,5% | 5,6% |  |  |  |
| Class C | 1,8% | 3,3% | 2,2% | 1,5% | 3,3% | 3,7% |  |  |  |
| Class D | 1,0% | 1,7% | 0,4% | 0,6% | 3,7% | 4,1% |  |  |  |
| Class E | 2,9% | -1,6% | 1,6% | 4,8% | 2,5% | 5,7% |  |  |  |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |  |  |  |
| Overall | 1,3% | 2,7% | 2,3% | 1,0% | 4,4% | 4,8% |  |  |  |
|  | 1,3% | 2,6% | 2,3% | 1,0% | 4,3% | 4,7% |  |  |  |
| Enc Time[%] | 99% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |  |  |  |

## 4-tap down-sampling

|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,7% | 6,3% | 3,5% | 1,8% | 3,3% | 3,4% | 0,5% | 5,9% | 4,3% |
| Class B | 0,6% | 1,2% | 2,6% | 1,6% | 1,7% | 1,2% | 0,6% | 1,9% | 2,2% |
| Class C | 1,5% | 1,7% | 2,2% | 2,0% | 3,8% | 4,2% | 1,3% | 3,4% | 3,3% |
| Class D | 0,6% | 1,0% | 0,3% | 1,0% | 0,9% | 2,7% | 0,6% | 0,6% | -0,3% |
| Class E | 2,9% | 2,6% | 3,7% | 6,2% | -0,1% | 1,6% | 2,4% | -0,8% | 1,4% |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |
| Overall | 1,2% | 2,5% | 2,4% | 2,3% | 2,0% | 2,6% | 1,0% | 2,3% | 2,2% |
|  | 1,2% | 2,5% | 2,4% | 2,3% | 2,1% | 1,8% | 1,0% | 2,3% | 2,2% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  | 104% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0,8% | 5,0% | 4,4% | -1,9% | 2,6% | -1,2% |  |  |  |
| Class B | 0,9% | 2,5% | 0,3% | 0,4% | 1,9% | 2,1% |  |  |  |
| Class C | 1,6% | 2,8% | 2,4% | 1,7% | 3,7% | 3,2% |  |  |  |
| Class D | 0,7% | 2,7% | 1,5% | 0,5% | 1,6% | 1,7% |  |  |  |
| Class E | 2,9% | -1,1% | 2,4% | 5,8% | -1,3% | -0,6% |  |  |  |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |  |  |  |
| Overall | 1,3% | 2,5% | 2,1% | 1,0% | 1,9% | 1,2% |  |  |  |
|  | 1,3% | 2,5% | 2,0% | 1,0% | 2,1% | 2,6% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 104% |  |  |  |  |  |

## 3-Lobe Lanczos-windowed filter down-sampling

|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,2% | 1,8% | -0,7% | 0,9% | 3,7% | -0,7% | 0,1% | 1,2% | -0,4% |
| Class B | 0,2% | -0,4% | 1,6% | 0,8% | 0,7% | 0,5% | 0,2% | 0,8% | -0,1% |
| Class C | 0,2% | 0,5% | 0,1% | 0,8% | 1,1% | 1,1% | 0,2% | 0,8% | 0,2% |
| Class D | 0,2% | -0,4% | -1,4% | 0,6% | 1,0% | 1,4% | -0,1% | 1,7% | -0,1% |
| Class E | 0,2% | 0,3% | 0,3% | 1,9% | -1,6% | -0,1% | 0,1% | -0,2% | 0,6% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |
| Overall | 0,2% | 0,3% | 0,0% | 1,0% | 1,1% | 0,5% | 0,1% | 0,9% | 0,0% |
|  | 0,2% | 0,2% | -0,1% | 1,0% | 1,1% | 1,2% | 0,1% | 0,9% | 0,1% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  | 106% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0,2% | 0,4% | 0,6% | -1,3% | 1,6% | -1,9% |  |  |  |
| Class B | 0,3% | 1,3% | -0,9% | 0,1% | 2,0% | 0,6% |  |  |  |
| Class C | 0,3% | 1,3% | -0,3% | 0,6% | 0,8% | 0,6% |  |  |  |
| Class D | 0,5% | 0,5% | 1,0% | 0,4% | 1,7% | 3,3% |  |  |  |
| Class E | 0,4% | -1,9% | -1,5% | 1,6% | -1,8% | -0,5% |  |  |  |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |  |  |  |
| Overall | 0,3% | 0,5% | -0,2% | 0,2% | 1,1% | 0,5% |  |  |  |
|  | 0,3% | 0,3% | -0,2% | 0,2% | 1,0% | 2,8% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 107% |  |  | 106% |  |  |  |  |  |

## Linear interpolated co-sited up-sampling

|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,1% | -0,8% | 0,4% | -0,9% | -3,2% | 0,1% | 0,4% | 0,1% |
| Class B | 0,0% | -0,5% | 0,7% | 0,1% | -1,0% | -0,6% | 0,0% | 1,1% | 0,1% |
| Class C | 0,0% | 0,0% | 0,6% | 0,1% | -0,2% | -0,3% | 0,1% | -0,3% | -0,1% |
| Class D | 0,0% | -0,8% | -0,3% | 0,1% | -0,8% | 0,7% | 0,0% | -1,0% | 1,6% |
| Class E | 0,1% | -0,6% | 1,4% | -0,2% | -1,9% | -2,5% | 0,0% | -0,3% | -0,3% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0,0% | -0,3% | 0,3% | 0,1% | -0,9% | -1,1% | 0,1% | 0,0% | 0,3% |
|  | 0,0% | -0,4% | 0,2% | 0,1% | -1,0% | -1,1% | 0,1% | 0,0% | 0,2% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  | 94% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0,0% | 0,4% | -0,8% | 0,3% | -3,5% | -2,4% |  |  |  |
| Class B | 0,0% | 0,0% | 0,0% | -0,1% | 0,0% | -1,1% |  |  |  |
| Class C | 0,0% | -0,4% | 0,2% | 0,0% | -0,6% | -0,6% |  |  |  |
| Class D | 0,1% | 1,3% | 0,2% | -0,1% | -1,8% | -1,2% |  |  |  |
| Class E | 0,0% | -0,5% | -0,3% | -0,8% | -0,6% | -3,1% |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | 0,0% | 0,2% | -0,1% | -0,1% | -1,3% | -1,6% |  |  |  |
|  | 0,0% | 0,1% | 0,0% | -0,1% | -1,6% | -1,6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 93% |  |  | 91% |  |  |  |  |  |

### DCTIF quarter-phase up-sampling

|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,1% | -0,1% | 0,2% | -5,2% | -2,6% | 0,1% | 0,8% | -0,4% |
| Class B | 0,0% | -0,4% | 0,0% | 0,0% | -0,2% | -0,2% | 0,0% | 0,7% | -0,1% |
| Class C | 0,0% | -0,4% | 0,3% | 0,0% | 0,1% | -0,5% | 0,0% | -0,4% | 0,1% |
| Class D | 0,0% | -0,6% | -0,3% | 0,0% | -0,4% | 0,8% | 0,0% | -0,8% | 0,3% |
| Class E | -0,1% | -2,0% | 1,5% | -0,5% | -2,4% | -1,4% | 0,0% | -0,9% | -0,9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0,0% | -0,6% | 0,2% | 0,0% | -1,5% | -0,7% | 0,0% | -0,1% | -0,1% |
|  | 0,0% | -0,6% | 0,3% | 0,0% | 0,0% | -0,8% | 0,0% | 0,0% | -0,3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  | 94% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0,0% | -0,1% | -1,3% | -0,4% | -4,5% | -2,2% |  |  |  |
| Class B | 0,0% | -0,4% | -0,6% | -0,2% | -0,1% | -0,5% |  |  |  |
| Class C | 0,1% | -0,7% | 0,0% | 0,0% | 0,4% | 0,1% |  |  |  |
| Class D | 0,0% | 0,8% | 1,5% | -0,2% | -2,1% | 0,2% |  |  |  |
| Class E | 0,0% | -1,4% | 2,5% | -0,7% | -1,2% | -2,3% |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| Overall | 0,0% | -0,3% | 0,3% | -0,3% | -1,4% | -0,9% |  |  |  |
|  | 0,0% | -0,3% | 0,4% | -0,3% | -1,1% | -0,9% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 93% |  |  | 91% |  |  |  |  |  |

# Conclusion

The code and results are verified and are conformant to the results stated by Cisco.

# Acknowledgement

The research activities described in this proposal were funded by Ghent University, the Interdisciplinary Institute for Broadband Technology (IBBT), the Institute for the Promotion of Innovation by Science and Technology in Flanders (IWT), the Fund for Scientific Research-Flanders (FWO-Flanders), and the European Union. Furthermore, this work was carried out using the Stevin Supercomputer Infrastructure at Ghent University.