| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0188 |
| --- | --- |

| Title: | On the syntax of ChromaQpOffset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

At the 7th JCT-VC meeting ChromaQpOffset, same as the one in AVC, has been adopted. This document proposes to modifications:

- Moving ChromaQpOffset from PPS to APS

- Change the names as “Cb_QP_Offset” and “Cr_QP_Offset”

# Introduction

ChromaQp in AVC is useful to control image quality of the actual product. At the 7th JCT-VC meeting it is agreed that HEVC also includes this functionality, exactly the same as AVC [1].

This document proposes two modifications:

- APS (Adaptation Parameter Set) has been newly introduced into HEVC. It conveys parameters that will or will not be changed picture-by-picture like quantization matrices. Although ChromaQpOffset requires much less bits than quantization matrices, it is adopted in similar way with adaptation parameters. So it should be transmitted not in PPS but in APS.

- In AVC a single parameter “ChromaQpOffset” is applied both for Cb and Cr with Baseline/Main Profiles, and a parameter “ChromaQpOffset” is applied for Cb and “SecondChromaQpOffset” is applied for Cr with High Profile. However HEVC will inherit High Profile features, “CbQpOffset” and “CrQpOffset” will be a better naming.

# Proposed Syntax

In Table 2.1 proposed syntax is shown. Semantics is also slightly changed:

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-1)

The value of QP’c is derived as

QP’C=QPCtoQPY[ clip(0,51,QP’Y+Shift ) ]

Where

QPCtoQPY= {

0, 1, 2, 3, 4, 5, 6, 7, 8, 9,10,11,

12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27,

28,29,29,30,31,32,32,33,34,34,35,35,36,36,37,37,

37,38,38,38,39,39,39,39}

Shift = CbQPOffset (for U) or CrQPOffset (for V)

Values of CbQPOffset and CrQPOffset are in the range of [-12, 12]

Table 2.1

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| aps_deblocking_filter_flag | u(1) |
| aps_chroma_qp_offset_flag | u(1) |
| if( aps_sample_adapt ive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_data_byte_count | u(8) |
| byte_align( ) |  |
| sao_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_data_byte_count | u(8) |
| byte_align( ) |  |
| alf_param( ) |  |
| } |  |
| if (aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| if (aps_chroma_qp_offset_flag) { |  |
| cb_qp_offset | se(v) |
| cr_qp_offset | se(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# References

[1]” Support of ChromaQPOffset in HEVC”, JCTVC-G509, Nov 2011

[2]” High Efficiency Video Coding (HEVC) text specification Working Draft 5”, JCTVC-G1103, Nov 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).