| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0190r2 |
| --- | --- |

| Title: | Non-CE10: Difference-QP-based Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In common test conditions QP value remains same within a picture, but in real applications it will vary. Performance of HEVC with varying QP should well be studied before it becomes international standard.

One of the tools that should be studied under the varying QP scenario is deblocking filter. In the 7th meeting there were 2 proposals related to this topic. One is JCTVC-G384, which proposes max QP of concatenated blocks is employed for deblocking. Another is JCTVC-G640, which proposes average QP of concatenated blocks is employed. Latter one is same as AVC and has been adopted into HM-5.0.

However, blocking artefacts are visible if QP of one block is small while the one for other is large. Either of the previous proposals can reduce such blocking artefacts.

This document proposes one additional operation for deblocking filter that takes difference of QPs of concatenated blocks into account.

# Introduction

If a lower-QP block and a higher-QP block shares boundary as shown in Fig 1.1, blocking artefacts are visually more noticeable than the case where QPs of both blocks are high.

At the 7th JCT-VC meeting 2 proposals are related to varying-QP deblocking. One is JCTVC-G384, which proposes which proposes max QP of concatenated blocks is employed for deblocking. The other is JCTVC-G640, which proposes average QP of concatenated blocks is employed.

However these proposals cannot always apply strong filtering with block boundaries as shown in Fig 1.1.

Fig 1.1

# Proposed Algorithm

To solve the problem stated above, this document proposes one additional logic into strong/weak filter decision as:

if (Bs >0) && (AVE[QP_P, QP_Q] >= 34) && (Diff[QP_P, QP_Q] >=8)

StrongerFilterApplied;

Fig 2.1 is the first frame of C:RaceHorses. It is encoded as Intra with QP=32 (-aq 1 –adr 12). Fig 2.2 and 2.3 are on the portion with a yellow circle in Fig 2.1.

Fig 2.2 is a QP map. As of the boundary with the yellow arrow, QP of upper block is 43 and the one of lower block is 32. Then the difference is large.

In Fig 2.3, green and red lines means weak and strong filter is applied, respectively. As can be seen weak filter is applied with HM-5.0 and strong filter is applied with the proposed method.

The proposed method can easily be harmonized with the WD method (JCTVC-G0640) or the method proposed by JCTVC-G384.

Fig 2.1

Fig 2.2

Fig 2.3

# Simulation Condition

Software of CE4 subtest 1.1 [anchor of CE4 subtest1] is used as anchor. On top of it the proposed method has been implemented.

The proposed method is enabled with the following macro in “Typedef.h”

#define DBF_TEST 1

IT/IT_Loco/RA/RA_Loco/RA_10/LB/LB_Loco conditions are tested with Class A to E sequences [2].

# Simulation Result

Summary of the result is shown in Table 4.1. Complete result is shown in “JCTVC-H0190.xls”. The author would like to thank NEC for cross-checking [5].

Average losses with the proposed method are 0.2%, 0.2%, 0.1%, 0.1%, 0.2%, 0.1% and 0.1% with IT/IT_Loco/RA/RA_Loco/RA10/RA_Loco/LB/LB_Loco conditions respectively. There is a trade-off between loss in coding efficiency and impact on subjective quality. It can be controlled by adjusting thresholds described in section 2 of this document.

The proposed method is applied not so many block boundaries so generally impact on subjective quality is not so much. This is one reason why coding efficiency loss is relatively small. However it is improved where the condition described in section 2 is applied, as shown in the following figures.

Fig 4.1 is the original image of the #299 frame of D:RaceHorses. Fig 4.2 is the portion with a yellow circle in Fig 4.1 encoded with QP=37±12, LDB condition. Left is the decoded image with the proposed method and right is the one with HM-5.0. As can be seen boundary between background and a horse is smoother with the proposed method.

Table 4.1

D: RaceHorses Frame #299

Fig 4.1

Left: Proposed; Right HM-5.0 [Encoded with LDB; QP=37]

Fig 4.2

# Conclusion

This document proposes difference-QP-based deblocking. The proposed method improves visual quality with varying-QP scenarios. Average loss in coding efficiency is less than 0.2%. It can be controlled by adjusting thresholds of the algorithm. It can easily be harmonized with the WD method (JCTVC-G0640) or the method proposed by JCTVC-G384.

It is recommended that the proposed method be adopted in CD.

# Proposed WD Text Changes

##### 8.6.1.4.1 Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE[0], dE[1], dEp[0], dEp[1], dEq[0], and dEq[1] containing decisions,

This process is invoked only when bS is not equal to 0.

Let s’ represent the luma sample array recPictureL of the current picture.

A variable qPL and qPdiff are derived as follows:

qPL = ( ( QPY + QPP + 1 ) >> 1 )

qPdiff = |QPY – QPP|

where QPP is specified by the QPY value of the coding unit containing the sample p0,0.	(8-1)

A variables β is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 51, qPL + ( beta_offset_div2 << 1 ) ) as input.

A variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3( 0, 55, qPL + 2*( bS – 1) + ( tc_offset_div2 << 1) ) as input.

All elements of a one-dimensional array of size (4), dL are initialized to dL[0] = 0, dL[1] = 3, dL[2] = 4 and dL[3] = 7.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3, 4, 7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-2)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-3)

- The elements of one dimensional arrays of size (),dp, dq and dpq are derived as follows with i=0..3:

dp[i] = | p2,dL[i] – 2*p1,dL[i] + p0,dL[i] |		(8-4)

dq[i] = | q2,dL[i] – 2*q1,dL[i] + q0,dL[i] |		(8-5)

dpq[i] = dp[i] + dq[i]		(8-6)

- The elements of the one dimensional array d are derived as follows with i=0..1:

d[i] = dpq[ 2*i ] + dpq[ 2*i + 1 ]		(8-7)

- All elements of the one dimensional arrays dE, dEp and dEq are set equal to 0.

- For each j=0..1, if d[j] is less than β, the following ordered steps apply:

- For each sample location ( xC + xB, yC + yB + dL[k] ), k = 0..1, the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,dL[k], qi,dL[k] with i = 0..3, the variables 2*dpq[ 2*j + k ], β and tC as inputs and a decision dSam[k] as output.

- The variable dE[j] is set equal to 1.

- If QPL >= 34 and QPdiff >=8, the variable dE[j] is set equal to 2.

- If dSam[0] is equal to 1 and dSam[1] is equal to 1, the variable dE[j] is set equal to 2.

- If dp[ 2*j ] + dp[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp[j] is set equal to 1.

- If dq[ 2*j ] + dq[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq[j] is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0, 3, 4, 7 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-8)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-9)

- The elements of one dimensional array of size (4), dp, dq and dpq are derived as follows with i=0..3:

dp[i] = | p2,dL[i] – 2*p1,dL[i] + p0,dL[i] |		(8-10)

dq[i] = | q2,dL[i] – 2*q1,dL[i] + q0,dL[i] |		(8-11)

dpq[i] = dp[i] + dq[i]		(8-12)

- The elements of the one dimensional array d are derived as follows with i=0..1:

d[i] = dpq[ 2*i ] + dpq[ 2*i + 1 ]		(8-13)

- All elements of the one dimensional arrays dE, dEp and dEq are set equal to 0.

- For each j=0..1, if d[j] is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + dL[k] + 4 * j, yC + yB ), k = 0..1, the decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,dL[k], qi,dL[k] with i = 0..3, the variables 2*dpq[ 2*j + k ], β and tC as inputs and a decision dSam[k] as output.

- The variable dE[j] is set equal to 1.

- If QPL >= 34 and QPdiff >=8, the variable dE[j] is set equal to 2.

- If dSam[0] is equal to 1 and dSam[1] is equal to 1, the variable dE[j] is set equal to 2.

- If dp[ 2*j ] + dp[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEp[j] is set equal to 1.

- If dq[ 2*j ] + dq[ 2*j + 1 ] is less than ( β + ( β >> 1 ) ) >> 3, the variable dEq[j] is set equal to 1.

# References

[1] “Varying QP Deblocking” JCTVC-G384, Nov 2011

[2] “Deblocking buf-fix for CU-Varying QP’s and IPCM blocks”, Nov 2011

[3] “Description of Core Experiment 4: Quantization”, JCTVC-G1204, Nov 2011

[4] “Common HM test conditions and software reference configurations”, JCTVC-G1200, Nov 2011

[5] “Non-CE10: Cross-verification report for difference-QP-based deblocking (JCTVC-H0190)”, JCTVC-H0644, Feb 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).