| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0191 |
| --- | --- |

| Title: | CE4 Subtest 3: Cross-verification report for test 3.1.b (JCTVC-H0508) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents cross-verification results of TU-level dQP signaling with PU-level constraint, which is proposed in JCTVC-H0508 and tested as a part of test 3.1.b in CE4 Subtest 3. It was confirmed that simulation results exactly match those provided by the proponent. Although the results show that the method provides gains in some cases, those gains are not logically accountable. It is recommended that the issue be clarified if JCTVC-H0508 is considered as adopted.

# Software analysis

The software was provided by the proponent and it was successfully compiled by Microsoft Visual C++ 2008. It was confirmed that the software implementation conforms to the description in JCTVC-H0508 [1], there are minor issues on formatting, i.e. indentation and usage of braces, to be corrected if it is integrated into HM.

It should be also noted that the implementation by the proponent extends the syntax element cu_qp_delta_enabled_flag in SPS of HEVC WD3 [4] for representing availability of the functionality of TU-level dQP signalling. In the latest WD of HEVC [5], the syntax element cu_qp_delta_enabled_flag in SPS is removed and incorporated into the syntax element max_cu_qp_delta_depth in PPS. If the JCTVC-H0508 proposal is adopted into HEVC, the syntax of the proposal should be harmonized with the latest HEVC syntax.

# Simulation results

The experiment to verify simulation results provided by the proponent was conducted under the CE4 Subtest 1 test conditions described in JCTVC-G1204 [2]. The computing platform used for the verification is shown below.

| Computing platform | Usage |
| --- | --- |
| OS: Windows 7 64-bit Professional CPU: Xeon X5680 3.33GHz (12 cores) Memory: 32GiB | Up to 12 processes for encoding and decoding ran at the same time. |

Table 1 shows the summary of results. An attached spreadsheet, JCTVC-H0191.xls, contains the detailed results. It was confirmed that BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent.

Table 1: Summary results against test 1.1 (CE4 anchor).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 0.3% | 0.9% | 0.8% | 2.8% | 0.1% | 0.8% | 0.8% | 2.3% |  |  |  |  |
| Class B | 0.2% | 0.6% | 0.5% | 0.2% | 0.1% | 0.4% | 0.4% | 0.9% |  |  |  |  |
| Class C | 0.2% | 0.5% | 0.6% | 2.4% | 0.1% | 0.5% | 0.5% | 2.4% |  |  |  |  |
| Class D | 0.1% | 0.5% | 0.4% | 2.2% | 0.1% | 0.3% | 0.3% | 2.3% |  |  |  |  |
| Class E | 0.3% | 0.9% | 0.9% | 0.6% | 0.2% | 1.2% | 1.1% | 1.6% |  |  |  |  |
| Overall | 0.2% | 0.6% | 0.6% | 1.5% | 0.1% | 0.6% | 0.6% | 1.8% |  |  |  |  |
|  | 0.2% | 0.6% | 0.6% |  | 0.1% | 0.6% | 0.6% |  |  |  |  |  |
| Class F | 0.1% | 0.3% | 0.4% | 0.8% | 0.1% | 0.2% | 0.2% | 1.1% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | 0.3% | 0.8% | 0.6% | 1.9% | 0.0% | 0.6% | 0.6% | 3.0% | 0.0% | 0.4% | 0.1% | -6.0% |
| Class B | 0.3% | 0.3% | 0.4% | -1.9% | -0.3% | 0.4% | 0.1% | -0.8% | 0.3% | 0.0% | 0.2% | -2.2% |
| Class C | 0.3% | 0.4% | 0.3% | 1.4% | -0.1% | 0.0% | 0.1% | 1.1% |  |  |  |  |
| Class D | 0.3% | 0.2% | 0.0% | 3.4% | -0.1% | 0.0% | 0.1% | 2.6% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.3% | 0.4% | 0.3% | 0.9% | -0.2% | 0.2% | 0.2% | 1.1% | 0.2% | 0.2% | 0.2% | -3.9% |
|  | 0.3% | 0.4% | 0.3% |  | -0.2% | 0.2% | 0.2% |  | 0.2% | 0.2% | 0.2% |  |
| Class F | 0.0% | -0.2% | 0.0% | -0.2% | -0.6% | -0.8% | -0.7% | 0.0% |  |  |  |  |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.5% | 0.9% | 1.3% | -4.0% | -1.5% | -0.9% | -0.3% | -2.0% |  |  |  |  |
| Class C | 0.5% | 0.6% | 1.0% | 1.5% | -0.4% | -0.3% | 0.5% | 1.4% |  |  |  |  |
| Class D | 0.6% | 0.4% | 1.4% | 5.0% | -0.3% | 0.5% | 0.3% | 3.8% |  |  |  |  |
| Class E | 0.5% | 1.1% | 1.0% | -6.3% | -1.5% | -1.1% | -0.4% | -3.2% |  |  |  |  |
| Overall | 0.5% | 0.7% | 1.2% | -0.8% | -0.9% | -0.4% | 0.0% | 0.1% |  |  |  |  |
|  | 0.5% | 0.8% | 1.1% |  | -0.9% | -0.4% | 0.0% |  |  |  |  |  |
| Class F | 0.2% | 0.8% | 0.3% | 0.5% | -0.4% | -1.1% | 1.0% | -0.2% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |  |  |  |  |

# Remarks

From results shown in Table 1, the proposed method shows gains in some cases, in particular under the LB-LC test condition. Although defects on the implementation have not found at this time and BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent, the proposal aims to achieve higher dQP controllability at the expense of some loss of R-D based performance. Those gains are not logically accountable. It is recommended that the issue be clarified if JCTVC-H0508 is considered as adopted.

# Conclusion

This contribution has presented cross-verification results of TU-level dQP signaling with PU-level constraint, which is proposed in JCTVC-H0508 and tested as a part of test 3.1.b in CE4 Subtest 3. It was confirmed that simulation results exactly match those provided by the proponent. Although the results show that the method provides gains in some cases, those gains are not logically accountable. It is recommended that the issue be clarified if JCTVC-H0508 is considered as adopted.

# References

[1] J.-H. Kim, et al., “CE4 Subtest 3.1.b: Additional constraint for QP adaptation at sub-CU level,” JCT-VC document JCTVC-H0508, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th meeting, San Jose, February 2012.

[2] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-G1204, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.

[3] X. Fang, et al., “A Unified LCU, sub-LCU and sub-CU (TU) Delta QP Signaling,” JCT-VC document JCTVC-G893, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.

[4] T. Wiegand, et al., “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC document JCTVC-E603, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[5] B. Bross, et al., “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCT-VC document JCTVC-G1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.