| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0192 WG11 number: m23066 |
| --- | --- |

| Title: | CE6: Cross-check report for Subtest CE6b and CE6c on modifications to Intra-frame coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hui Li Tan Chuohao Yeo Yih Han Tan 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

CE6b and CE6c aim to improve the coding performance of Intra mode coding of the HEVC Test Model by modifying the current intra prediction and mode coding design. This contribution presents a summary of the results and observations from the experiments performed by Institute for Infocomm Research. The coding results obtained reportedly match what was provided by the proponents.

# Introduction

In the High-Efficiency Video Coding (HEVC) Working Draft 5 (WD5), there are 35 intra prediction modes available for Luma coding. These are the Planar prediction, DC prediction, Ver+x (-8 ≤ x ≤ 8) and Hor+x (-7 ≤ x ≤ 8) directional predictions. All 35 modes are available for 8x8 and above PUs, and only 18 modes are available for 4x4 PUs. The 18 modes are Planar prediction, DC prediction, Ver+2x (-4 ≤ x ≤ 4), Hor+2x (-3 ≤ x ≤ 2), Hor+8.

Part 3 of JCTVC-G119 [1] and JCTVC-H0057 [2] proposes to remove Hor+8 and replace Ver+8 prediction mode with one bi-prediction mode that averages the values of Hor+8 and Ver+8 diagonal prediction. Hence, Hor+6 can be added for 4x4 PU. The total number of remaining modes is also a power of 2 for all block sizes (16 for 4x4 PUs and 32 for 8x8 and above PUs) and the remaining mode index can be binarized with a FLC.

The testing of part 3 of the proposal is divided into two subtests. In Subtest 6b, the coding performance of the proposed harmonized Intra mode coding tool is tested with the replacement of the existing directional intra prediction mode with the bi-prediction mode. In Subtest 6c, the coding performance of the proposed harmonized Intra mode coding tool is tested without the replacement, i.e. Hor+8 is removed from the existing directional intra prediction modes only.

# Simulation Results

The proposed modifications were implemented on HM5.0. Tests were conducted using the two All-Intra configurations, i.e. high-efficiency and low complexity, following the common testing conditions [2] [3].

Table 1 and 2 show the cross-check results of Subtest CE6b and Subtest CE6c.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.1% | 0.1% | 0.1% | -0.2% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class E | 0.0% | 0.2% | 0.2% | -0.1% | 0.1% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class F | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 1. Cross-check results for CE6b.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. Cross-check results for CE6c.

# Conclusions

These CE6b and CE6c cross-check experiments report results which are consistent with those provided by the proponents.

# References

- Y. Lin (Hisilicon), H. Yang (Huawei), L. Liu, J. Zheng (Hisilicon), H. Yu (Huawei), “Modifications to Intra-frame coding,” JCTVC-G119, Geneva, Switzerland, Jul. 2011.

- Y. Lin, L. Liu, J. Zheng (HiSilicon), H. Yang, H. Yu (Huawei), “CE6 subset 5.2.2 and 6.2.2: Intra coding improvements,” JCTVC-H0057, San José, CA, USA, Feb. 2012.

- F. Bossen, “Common conditions and software reference configurations,” JCTVC-G1200, Geneva, Switzerland, Jul. 2011.

- A. Tabatabai, E. Francois and K. Chono, “CE6: Intra coding improvements,” in JCTVC-G1206, Geneva, Switzerland, Jul. 2011.