| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0193 WG11 number: m23067 |
| --- | --- |

| Title: | Non-CE6: Cross-check report for JCTVC-H0175 on adaptations of Intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hui Li Tan Chuohao Yeo Yih Han Tan 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

JCTVC-H0175 aims to improve the coding performance of Intra mode coding of the HEVC Test Model by modifying the current intra prediction and mode coding design. This contribution presents a summary of the results and observations from the experiments performed by Institute for Infocomm Research. The coding results obtained reportedly match what was provided by the proponents.

# Introduction

In the High-Efficiency Video Coding (HEVC) Working Draft 5 (WD5), there are 35 intra prediction modes available for Luma coding. These are the Planar prediction, DC prediction, Ver+x (-8 ≤ x ≤ 8) and Hor+x (-7 ≤ x ≤ 8). All 35 modes are available for 8x8 and above PUs, and only 18 modes are available for 4x4 PUs. The 18 modes are Planar prediction, DC prediction, Ver+2x (-4 ≤ x ≤ 4), Hor+2x (-3 ≤ x ≤ 2), Hor+8.

JCTVC-G153 [1] / JCTVC-H0075 [2] proposed to include Hor+6 directional prediction mode for 4x4 PUs and an simplification of Intra mode coding. The current proposal, JCTVC-H0175 [3], uses a similar decoding tree as in [1] [2], but with the following changes:

(1) Mode is planar when decoding ‘00’;

(2) MPM derivation: MPM0 is derived from the left PU and MPM1 is derived from the above PU;

(3) Current mode numbering is replaced by an angular index numbering;

(4) Mapping of directional intra prediction modes for 4x4 PUs: For WD5, for 4x4 PUs, directional intra prediction modes from left or above PUs which are not within the 18 available modes are mapped to Planar prediction mode. In the proposal, these are mapped to their nearest available prediction mode to their left or below.

The proposal would require potential function calls to get the prediction modes at different branches of the decoding tree and an additional function that only gets the left prediction mode.

Two tests are being conducted, one with no context and one with two contexts.

# Simulation Results

The proposed modifications were implemented on HM5.0. Tests were conducted using the two All-Intra configurations, i.e. high-efficiency and low complexity, following the common testing conditions [4].

Table 1 and 2 show the cross-check results of using no context and using 2 contexts.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | 0.1% | 0.2% | -0.5% | -0.1% | 0.0% |
| Class B | -0.2% | -0.1% | 0.0% | -0.3% | -0.1% | -0.1% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| Class D | -0.2% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class E | -0.2% | 0.0% | 0.2% | -0.3% | -0.1% | 0.0% |
| Overall | -0.2% | 0.0% | 0.0% | -0.3% | -0.2% | -0.1% |
|  | -0.2% | 0.0% | 0.0% | -0.3% | -0.1% | -0.1% |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 96% |  |  |

Table 1. Cross-check results for using no context.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | 0.1% | 0.2% | -0.5% | -0.1% | 0.0% |
| Class B | -0.3% | -0.1% | -0.1% | -0.4% | -0.2% | -0.1% |
| Class C | -0.3% | -0.2% | -0.2% | -0.3% | -0.2% | -0.3% |
| Class D | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| Class E | -0.3% | -0.2% | -0.1% | -0.4% | -0.3% | -0.2% |
| Overall | -0.3% | -0.1% | -0.1% | -0.4% | -0.2% | -0.2% |
|  | -0.3% | -0.1% | -0.1% | -0.4% | -0.2% | -0.2% |
| Class F | -0.3% | -0.4% | -0.3% | -0.3% | -0.4% | -0.5% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 2. Cross-check results for using two contexts.

# Conclusions

These cross-check experiments report results which are consistent with those provided by the proponents.

# References

- C. Yeo, H. Tan, Y. H. Tan, Z. Li, “Non-CE6: On intra prediction mode coding”, JCTVC-G153, Geneva, Switzerland, Nov. 2011.

- C. Yeo, Y. H. Tan, H. L. Tan, Z. G. Li, “CE6: Subtest 6c (6.2.1) – Intra mode coding simplification,” JCTVC-H0075, San José, CA, USA, Feb. 2012.

- E. François, S. Pautet, C.Gisquet, “Non-CE6c: Adaptations of intra mode coding,” JCTVC-H0175, San José, CA, USA, Feb. 2012.

- F. Bossen, “Common conditions and software reference configurations,” JCTVC-G1200 Geneva, Switzerland, Nov. 2011.