| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0194 |
| --- | --- |

| Title: | Non-CE4: Cross-verification report for modified spatial QP prediction based on intra prediction direction (method 1 of JCTVC-H0204) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a study and cross-verification results of a modified rate-control-friendly spatial QP prediction method based on CE4 test 1.3, which is the first proposal of those presented in JCTVC-H0204. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-H0204 and results exactly match those presented in JCTVC-H0204.

# Software analysis

Software was provided by JVC KENWOOD and it was successfully compiled by Microsoft Visual C++ 2008. It was confirmed that the software implementation conforms to the description in JCTVC-H0204 [1]. The implementation is simple and straightforward, and there have been no problems found in the software.

# Remarks

The method is based on the test 1.3 [2] of CE4 [3], which is a spatial QP prediction method based on intra prediction direction. The JCTVC-H0204 method modifies the CE4 test 1.3 so that unavailable reference QPs are replaced with the previously-coded QP. Thus the average operation is always employed, except directional intra cases where approximate intra direction is above or left. Conditional branches of the CE4 test 1.3 may be a little bit simplified by this modification, though it depends on implementations whether implementation is actually simplified or not.

# Simulation results

The experiment to verify simulation results provided by the proponent was conducted under the CE4 Subtest 1 test conditions described in JCTVC-G1204 [3]. The computing platform used for the verification is shown below. As on MinCUDQPSize, two conditions of 8x8 (mandatory in CE4 Subtest 1) and 16x16 (optional in CE4 Subtest 1) were tested.

| Computing platform | Usage |
| --- | --- |
| OS: Windows 7 64-bit Professional CPU: Xeon X5680 3.33GHz (12 cores) Memory: 32GiB | Up to 12 processes for encoding and decoding ran at the same time. |

Tables 1 and 2 show the summary of results. Attached spreadsheets contain the detailed results. It was confirmed that BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent and that differences between encoding/decoding times measured by the proponent and those measured by us are negligible.

Table 1: Summary results against test 1.1 of CE4 (anchor) with MinCUDQPSize equal to 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.4% | -0.4% | -0.4% | -8.2% | -0.3% | -0.3% | -0.3% | -8.4% |  |  |  |  |
| Class B | -0.4% | -0.4% | -0.4% | -10.4% | -0.3% | -0.3% | -0.3% | -10.7% |  |  |  |  |
| Class C | -0.5% | -0.5% | -0.5% | -12.0% | -0.5% | -0.5% | -0.4% | -12.4% |  |  |  |  |
| Class D | -0.5% | -0.5% | -0.4% | -12.1% | -0.4% | -0.4% | -0.4% | -12.2% |  |  |  |  |
| Class E | -0.7% | -0.8% | -0.7% | -12.0% | -0.7% | -0.7% | -0.7% | -13.9% |  |  |  |  |
| Overall | -0.5% | -0.5% | -0.5% | -11.2% | -0.4% | -0.4% | -0.4% | -11.7% |  |  |  |  |
|  | -0.5% | -0.5% | -0.5% |  | -0.4% | -0.4% | -0.4% |  |  |  |  |  |
| Class F | -0.3% | -0.3% | -0.3% | -7.9% | -0.2% | -0.2% | -0.3% | -8.8% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.3% | -0.3% | -0.3% | -5.3% | -0.4% | -0.2% | -0.3% | -5.3% | -0.2% | -0.3% | -0.3% | -4.3% |
| Class B | -0.3% | -0.2% | -0.3% | -6.2% | -0.3% | -0.1% | -0.1% | -6.0% | -0.3% | -0.4% | -0.3% | -6.2% |
| Class C | -0.4% | -0.3% | -0.3% | -8.1% | -0.4% | -0.3% | -0.3% | -7.9% |  |  |  |  |
| Class D | -0.3% | -0.3% | -0.3% | -7.6% | -0.3% | -0.5% | -0.3% | -7.7% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.3% | -7.0% | -0.3% | -0.3% | -0.2% | -6.9% | -0.2% | -0.3% | -0.3% | -5.3% |
|  | -0.3% | -0.3% | -0.3% |  | -0.3% | -0.3% | -0.2% |  | -0.2% | -0.3% | -0.3% |  |
| Class F | -0.1% | -0.3% | -0.2% | -5.9% | -0.3% | -0.2% | -0.1% | -6.3% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | 0.3% | -4.3% | -0.2% | -0.1% | 0.0% | -3.8% |  |  |  |  |
| Class C | -0.3% | -0.6% | -0.4% | -6.2% | -0.4% | -0.5% | -0.1% | -6.0% |  |  |  |  |
| Class D | -0.3% | -0.3% | 0.0% | -5.9% | -0.4% | -0.6% | -0.6% | -6.1% |  |  |  |  |
| Class E | -0.2% | -0.6% | -0.1% | -3.6% | -0.3% | -1.0% | -0.3% | -4.4% |  |  |  |  |
| Overall | -0.3% | -0.4% | 0.0% | -5.1% | -0.3% | -0.5% | -0.2% | -5.0% |  |  |  |  |
|  | -0.3% | -0.4% | -0.1% |  | -0.3% | -0.5% | -0.3% |  |  |  |  |  |
| Class F | -0.2% | -0.4% | -0.2% | -3.6% | -0.2% | -0.7% | -0.1% | -3.8% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |  |  |  |  |

Table 2: Summary results against test 1.1 of CE4 (anchor) with MinCUDQPSize equal to 16x16.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  | All Intra HE-10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.2% | -0.1% | -5.8% | -0.1% | -0.1% | -0.1% | -5.5% |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.2% | -8.2% | -0.1% | -0.1% | -0.1% | -8.1% |  |  |  |  |
| Class C | -0.2% | -0.1% | -0.2% | -9.5% | -0.2% | -0.2% | -0.2% | -9.4% |  |  |  |  |
| Class D | -0.1% | -0.2% | -0.1% | -8.6% | -0.1% | -0.1% | -0.1% | -8.7% |  |  |  |  |
| Class E | -0.4% | -0.4% | -0.4% | -11.0% | -0.3% | -0.2% | -0.3% | -11.4% |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.2% | -8.8% | -0.2% | -0.1% | -0.1% | -8.8% |  |  |  |  |
|  | -0.2% | -0.2% | -0.2% |  | -0.2% | -0.1% | -0.2% |  |  |  |  |  |
| Class F | -0.1% | 0.0% | -0.1% | -6.6% | -0.1% | -0.2% | -0.1% | -7.1% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |  |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  | Random Access HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.1% | -4.0% | -0.2% | -0.1% | -0.1% | -4.1% | -0.1% | 0.0% | 0.2% | -3.4% |
| Class B | -0.1% | -0.2% | 0.1% | -5.0% | -0.2% | 0.0% | -0.1% | -4.7% | -0.1% | -0.2% | -0.2% | -5.0% |
| Class C | -0.2% | -0.1% | 0.1% | -6.4% | -0.2% | -0.2% | -0.1% | -6.4% |  |  |  |  |
| Class D | -0.2% | 0.0% | -0.2% | -6.1% | -0.2% | -0.2% | -0.2% | -6.2% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.1% | 0.0% | -5.5% | -0.2% | -0.1% | -0.1% | -5.5% | -0.1% | -0.1% | 0.0% | -4.3% |
|  | -0.2% | -0.1% | 0.0% |  | -0.2% | -0.1% | -0.1% |  | -0.1% | -0.1% | 0.0% |  |
| Class F | -0.2% | -0.2% | -0.1% | -4.7% | -0.1% | -0.2% | -0.1% | -4.9% |  |  |  |  |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  | Low delay B HE-10 |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.2% | -3.9% | -0.2% | -0.2% | -0.5% | -3.6% |  |  |  |  |
| Class C | -0.2% | -0.3% | -0.3% | -5.6% | -0.1% | -0.3% | -0.2% | -5.5% |  |  |  |  |
| Class D | -0.1% | 0.0% | -0.2% | -5.4% | -0.2% | 0.3% | -0.3% | -5.5% |  |  |  |  |
| Class E | -0.2% | -1.1% | -0.9% | -3.8% | -0.2% | -0.6% | 0.5% | -3.9% |  |  |  |  |
| Overall | -0.1% | -0.4% | -0.3% | -4.7% | -0.2% | -0.2% | -0.2% | -4.6% |  |  |  |  |
|  | -0.1% | -0.5% | -0.3% |  | -0.2% | -0.2% | -0.1% |  |  |  |  |  |
| Class F | -0.2% | -0.2% | -0.1% | -2.7% | 0.0% | 0.1% | -0.7% | -2.8% |  |  |  |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 98% |  |  |  |  |  |  |  |

# Conclusion

This contribution has presented a study and cross-verification results of a modified rate-control-friendly spatial QP prediction method based on CE4 test 1.3, which is the first proposal of those presented in JCTVC-H0204. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-H0204 and results exactly match those presented in JCTVC-H0204.

# References

[1] H. Nakamura, et al., “Non-CE4: Compatible QP prediction with RC and AQ,” JCT-VC document JCTVC-H0204, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th meeting, San Jose, February 2012.

[2] H. Aoki, et al., “CE4 Subtest 1: Rate control friendly spatial QP prediction based on intra prediction (test 1.3),” JCT-VC document JCTVC-H0053, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 8th meeting, San Jose, February 2012.

[3] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-G1204, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 7th meeting, Geneva, November 2011.