| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0195 WG11 number: m23069 |
| --- | --- |

| Title: | Non-CE6: Cross-check report for JCTVC-H0326 on Chroma Intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hui Li Tan Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

JCTVC-H0326 [1] proposes two schemes to modify the current Chroma Intra coding design. This contribution presents a summary of the results and observations from the experiments performed by Institute for Infocomm Research. The coding results obtained reportedly match what was provided by the proponents.

# Introduction

In the High-Efficiency Video Coding (HEVC) Working Draft 5 (WD5), there are six intra prediction modes available for Chroma coding. These are the Planar, Vertical, Horizontal, DC, DM and LM prediction modes.

The first scheme proposes to use Fix-Length binarization with bypass coding for the four remaining modes, i.e. Vertical, Horizontal, DC and Planar.

The second scheme is a modification of proposal JCTVC-G423 [2], and proposes to keep only one Chroma remaining mode, DC/Planar mode depending on the Luma mode.

# Simulation Results

The proposed modifications were implemented on HM5.0. Tests were conducted using the two All-Intra configurations, i.e. high-efficiency and low complexity, following the common testing conditions [3] [4].

The table below shows the cross check results for the first scheme:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.4% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.4% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | -0.4% |
| Class D | 0.0% | -0.1% | -0.2% | 0.1% | -0.4% | -0.6% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | -0.3% | -0.4% |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | -0.3% | -0.4% |
|  | 0.0% | -0.1% | -0.2% | 0.0% | -0.3% | -0.4% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 104% |  |  | 106% |  |  |

Table 1. Cross-check results for the first scheme.

The tables below show the cross check results relating to the second scheme:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.2% | 0.4% | 0.1% | 0.4% | 0.7% |
| Class B | 0.1% | 0.5% | 0.5% | 0.1% | 0.9% | 1.3% |
| Class C | 0.2% | 0.4% | 0.6% | 0.3% | 1.2% | 1.5% |
| Class D | 0.2% | 0.7% | 0.6% | 0.4% | 1.4% | 1.4% |
| Class E | 0.0% | 0.3% | 0.6% | 0.1% | 1.1% | 1.2% |
| Overall | 0.1% | 0.4% | 0.5% | 0.2% | 1.1% | 1.3% |
|  | 0.1% | 0.4% | 0.6% | 0.2% | 1.1% | 1.3% |
| Class F | 0.1% | 0.4% | 0.3% | 0.4% | 1.3% | 1.5% |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 108% |  |  | 108% |  |  |

Table 2. Cross-check results for the second scheme.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.9% | -1.2% | -0.4% | -1.9% | -2.0% |
| Class B | -0.2% | -0.6% | -1.1% | -0.4% | -1.5% | -1.9% |
| Class C | -0.2% | -0.6% | -0.6% | -0.5% | -2.1% | -2.1% |
| Class D | -0.2% | -0.5% | -0.7% | -0.4% | -1.8% | -2.1% |
| Class E | -0.3% | -1.4% | -1.6% | -0.4% | -2.4% | -2.7% |
| Overall | -0.3% | -0.8% | -1.0% | -0.4% | -1.9% | -2.1% |
|  | -0.3% | -0.8% | -1.0% | -0.4% | -1.9% | -2.2% |
| Class F | 0.1% | -0.9% | -0.8% | -0.3% | -1.8% | -1.9% |
| Enc Time[%] | 118% |  |  | 128% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

Table 3. Cross-check results for intra joint mode decision (JMD).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.9% | -1.1% | -0.4% | -1.9% | -1.8% |
| Class B | -0.2% | -0.5% | -1.0% | -0.4% | -1.3% | -1.5% |
| Class C | -0.2% | -0.5% | -0.4% | -0.5% | -1.8% | -1.8% |
| Class D | -0.2% | -0.4% | -0.6% | -0.4% | -1.5% | -1.8% |
| Class E | -0.3% | -1.5% | -1.5% | -0.4% | -2.0% | -2.2% |
| Overall | -0.3% | -0.7% | -0.9% | -0.4% | -1.6% | -1.8% |
|  | -0.3% | -0.7% | -0.9% | -0.4% | -1.7% | -1.8% |
| Class F | 0.1% | -0.9% | -0.8% | -0.3% | -1.5% | -1.5% |
| Enc Time[%] | 119% |  |  | 122% |  |  |
| Dec Time[%] | 107% |  |  | 106% |  |  |

Table 4. Cross-check results for the second scheme with intra joint mode decision (JMD).

# Conclusions

These cross-check experiments report results which are consistent with those provided by the proponents.

# References

- H. Li, B. Li, L. Li, J. Zhang (USTC), H. Yang, H. Yu, “Non-CE6: Simplification of intra chroma mode coding,” JCTVC-H0326, San José, CA, USA, Feb 2012.

- H. Yang, J. Zhou, H. Yu, “Non-CE6: Remove potential duplicate modes from the candidate mode list for chroma intra prediction,” JCTVC-G423, Geneva, Switzerland, Jul., 2011.

- F. Bossen, “Common conditions and software reference configurations,” JCTVC-G1200, Geneva, Switzerland, Jul., 2011.

- A. Tabatabai, E. Francois and K. Chono, “CE6: Intra coding improvements,” in JCTVC-G1206, Geneva, Switzerland, Jul. 2011.