| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0197 |
| --- | --- |

| Title: | CE4: Results of Subtest 1.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Masayoshi Nishitani Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com nishitani.masayoshi@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports CE4 Subtest 1.4 on QP prediction using spatial QP. In the HM5.0, the predicted QP is always QP of the preceding quantization group of coding unit for delta QP signaling. In this contribution, it is proposed to calculate the predicted QP based on QPs of preceding neighbouring quantization groups of coding unit. The simulation results report that the proposed method provides 0.4% BD-rate gain for intra only, 0.3% BD-rate gain for random access, 0.2% BD-rate gain for random access 10 and 0.2% BD-rate gain for low delay B settings in the case of MaxCUDepthDQP=3(8x8).

# Introduction

Delta QP is calculated as the difference between the QP of current quantization group of coding unit, which is hereafter referred to as QGCU in this contribution, and the predicted QP. In the HM 5.0 [1], QP of the preceding QGCU in scan order is employed as the predicted QP. If the predicted QP is more different from the current QP, the larger magnitude of delta QP is, the more bits will be required. Coded bit size of delta QP is not negligible especially when QP is controlled elaborately for adaptive quantization.

Therefore, the method to calculate the predicted QP is significant to reduce the amount of coded bits.

# Algorithm Description

## Current algorithm (HM5.0)

In the HM5.0, QP of the left neighbouring QGCU is employed as the predicted QP. If the left neighbouring QGCU in the current slice is not available, QP of the preceding QGCU in scan order that is available in the current slice is employed as the predicted QP. However, in CE4 Subtest 1, QP of the preceding QGCU in scan order is always employed as the predicted QP, which is defined as Subtest 1.1.

## Proposed algorithm

Brief algorithm of Subtest 1.4 [2] is described as follows.

In the following condition, each neighbouring QGCU which is not in the current LCU is regarded as not available in the same way as in Subtest 1.3.

- If both the left neighbouring QGCU and the above neighbouring QGCU are not in the current LCU (If the current QGCU is the most top-left in the current LCU), the predicted QP is set equal to the QP of the preceding QGCU in scan order.

- Otherwise if a neighbouring QGCU is not available, the predicted QP is set equal to the QP of the neighbouring QGCU which is available.

- Otherwise, if the QPL* is equal to the QPAL*, the predicted QP is derived as follows:	predictedQP=QPL+3∗QPA+2>>2

- Otherwise, if the QPA* is equal to the QPAL, the predicted QP is derived as follows:	predictedQP=3∗QPL+QPA+2>>2

- Otherwise, the predicted QP is derived as follows:
	predictedQP=QPL+QPA+1>>1

Remarks:

- QPL: QP of the left neighbouring QGCU

- QPA: QP of the above neighbouring QGCU

- QPAL: QP of the above-left neighbouring QGCU

## Implementation

### Software

This proposed method is implemented onto the software for HM 5.0 with the CE4 subtest 1.

# Experiments

## Test conditions

Experiments to evaluate performance of the proposed method are conducted under the CE4 Subtest 1 test conditions that are based on the JCTVC common test condition defined in [3], with MaxCUDQPSize equal to 3(8x8) and 2(16x16).

## Experimental environment

The experimental environment used is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

### MaxCUDepthDQP=3(8x8)

In the case of MaxCUDepthDQP=3(8x8), Table 2, Table 3, Table 4 and Table 5 present the simulation results for intra only, random access, random access 10 and low delay, respectively.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.4% | -0.3% | -0.4% | -7.0% | -0.2% | -0.2% | -0.3% | -6.6% |
| Class B | -0.4% | -0.4% | -0.4% | -9.5% | -0.3% | -0.2% | -0.2% | -9.5% |
| Class C | -0.5% | -0.4% | -0.4% | -10.3% | -0.4% | -0.4% | -0.4% | -10.5% |
| Class D | -0.5% | -0.4% | -0.4% | -11.1% | -0.4% | -0.4% | -0.4% | -11.2% |
| Class E | -0.5% | -0.6% | -0.5% | -9.5% | -0.6% | -0.5% | -0.6% | -10.6% |
| Overall | -0.4% | -0.4% | -0.4% | -9.7% | -0.4% | -0.3% | -0.4% | -9.9% |
|  | -0.4% | -0.4% | -0.4% |  | -0.4% | -0.3% | -0.4% |  |
| Class F | -0.2% | -0.3% | -0.2% | -8.1% | -0.2% | -0.3% | -0.3% | -8.2% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |

Table 3 Simulation results for random access

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.3% | -0.1% | -4.8% | -0.3% | -0.2% | -0.3% | -4.6% |
| Class B | -0.2% | -0.3% | -0.3% | -5.7% | -0.2% | -0.1% | -0.1% | -5.6% |
| Class C | -0.3% | -0.1% | -0.4% | -7.0% | -0.3% | -0.2% | -0.2% | -6.9% |
| Class D | -0.3% | -0.4% | -0.4% | -7.3% | -0.3% | -0.6% | -0.2% | -7.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.3% | -6.4% | -0.3% | -0.2% | -0.2% | -6.2% |
|  | -0.3% | -0.2% | -0.2% |  | -0.3% | -0.2% | -0.2% |  |
| Class F | -0.2% | -0.4% | -0.1% | -6.6% | -0.3% | -0.3% | -0.1% | -6.4% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |

Table 4 Simulation results for random access 10

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | 0.0% | -0.1% | -3.8% |
| Class B | -0.2% | -0.3% | -0.3% | -5.8% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.2% | -4.9% |
|  | -0.2% | -0.2% | -0.2% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 5 Simulation results for low delay B

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.4% | -4.0% | -0.2% | -0.3% | -0.4% | -3.5% |
| Class C | -0.3% | -0.5% | -0.4% | -5.6% | -0.3% | -0.6% | 0.2% | -5.3% |
| Class D | -0.3% | -0.4% | -0.4% | -5.6% | -0.3% | -0.7% | -0.5% | -5.7% |
| Class E | -0.1% | -0.2% | -0.3% | -2.4% | -0.1% | -1.2% | -0.6% | -3.1% |
| Overall | -0.2% | -0.3% | -0.4% | -4.5% | -0.2% | -0.7% | -0.3% | -4.4% |
|  | -0.2% | -0.4% | -0.4% |  | -0.2% | -0.6% | -0.3% |  |
| Class F | -0.3% | -0.2% | -0.4% | -4.5% | -0.1% | -0.2% | 0.9% | -4.3% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

The simulation results report that the proposed method provides 0.4% BD-rate [4] gain for intra only, 0.3% BD-rate gain for random access, 0.2% BD-rate gain for random access 10 and 0.2% BD-rate gain for low delay B settings (See the attached excel sheet as for the more detailed results).

### MaxCUDepthDQP=2(16x16)

In the case of MaxCUDepthDQP=2(16x16), Table 6, Table 7, Table 8 and Table 9 present the simulation results for intra only, random access, random access 10 and low delay, respectively.

Table 6 Simulation results for intra only

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.2% | -7.1% | -0.1% | -0.1% | -0.1% | -6.5% |
| Class B | -0.2% | -0.2% | -0.2% | -9.5% | -0.1% | -0.1% | -0.1% | -9.3% |
| Class C | -0.2% | -0.1% | -0.2% | -9.4% | -0.2% | -0.1% | -0.2% | -9.4% |
| Class D | -0.1% | -0.2% | -0.1% | -9.3% | -0.1% | -0.1% | -0.1% | -9.3% |
| Class E | -0.3% | -0.4% | -0.3% | -10.1% | -0.3% | -0.2% | -0.3% | -10.2% |
| Overall | -0.2% | -0.2% | -0.2% | -9.3% | -0.2% | -0.1% | -0.2% | -9.2% |
|  | -0.2% | -0.2% | -0.2% |  | -0.2% | -0.1% | -0.2% |  |
| Class F | -0.2% | -0.1% | -0.2% | -10.6% | -0.1% | -0.3% | -0.1% | -10.8% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 7 Simulation results for random access

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.2% | -4.6% | -0.2% | -0.3% | -0.1% | -4.5% |
| Class B | -0.2% | -0.2% | 0.0% | -5.3% | -0.1% | -0.1% | 0.0% | -5.1% |
| Class C | -0.2% | -0.1% | -0.1% | -5.7% | -0.2% | -0.1% | -0.2% | -5.8% |
| Class D | -0.1% | 0.0% | -0.1% | -6.0% | -0.2% | -0.3% | -0.5% | -6.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.1% | -0.1% | -5.5% | -0.2% | -0.2% | -0.2% | -5.5% |
|  | -0.2% | -0.1% | -0.1% |  | -0.1% | -0.2% | -0.2% |  |
| Class F | -0.2% | -0.2% | 0.0% | -7.7% | -0.1% | -0.1% | -0.3% | -7.7% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |

Table 8 Simulation results for random access 10

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.1% | 0.0% | -3.6% |
| Class B | -0.1% | -0.1% | -0.1% | -5.2% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -4.5% |
|  | -0.1% | -0.1% | -0.1% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 9 Simulation results for Low delay B

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.1% | -4.0% | -0.1% | -0.3% | -0.9% | -3.6% |
| Class C | -0.2% | -0.2% | -0.3% | -5.0% | -0.1% | 0.0% | -0.1% | -4.8% |
| Class D | -0.1% | 0.3% | 0.0% | -5.3% | -0.1% | 0.4% | 0.2% | -5.2% |
| Class E | -0.3% | -1.3% | -0.6% | -2.5% | -0.1% | -0.8% | -0.5% | -2.5% |
| Overall | -0.2% | -0.3% | -0.2% | -4.3% | -0.1% | -0.2% | -0.3% | -4.1% |
|  | -0.2% | -0.4% | -0.2% |  | -0.1% | -0.2% | -0.4% |  |
| Class F | -0.3% | -0.3% | -0.1% | -5.2% | 0.0% | -0.5% | -0.6% | -4.6% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |

The simulation results report that the proposed method provides 0.2% BD-rate gain for intra only, 0.2% BD-rate gain for random access, 0.1% BD-rate gain for random access 10 and 0.1-0.2% BD-rate gain for low delay B settings (See the attached excel sheet as for the more detailed results).

# Conclusions

In this contribution, the experimental results of Subtest 1.4 on QP prediction using spatial QP are reported. The simulation results report that the proposed method provides 0.4% BD-rate gain for intra only, 0.3% BD-rate gain for random access, 0.2% BD-rate gain for random access 10 and 0.2% BD-rate gain for low delay B settings　in the case of MaxCUDepthDQP=3(8x8).

# References

[1] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G, Geneva, November 2011

[2] “Description of Core Experiment 4: Quantization”,

JCTVC-G1204r3, Geneva, November 2011

[3] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

[4] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Proposed modification of HEVC Working Draft 5.0

The modified or added portions are highlighted with yellow colour.

In the case of Proposal 1, the modification of QP prediction based on intra prediction direction complies with CE4 Subtest1.3.

7.4.9  Transform coefficient semantics

The transform coefficient levels are parsed into the arrays transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes and intra prediction modes, is derived as specified in Table 7-Error: Reference source not found.

Table 7-12 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 1 | 1 | 3 | 3 |
| 1 | 2 | 2 | 3 | 3 |
| 2-3 | 3 | 3 | 3 | 3 |
| 4-5 | 1 | 1 | 3 | 3 |
| 6 | 3 | 3 | 3 | 3 |
| 7-8 | 2 | 2 | 3 | 3 |
| 9-10 | 3 | 3 | 3 | 3 |
| 11-12 | 1 | 1 | 3 | 3 |
| 13-14 | 3 | 3 | 3 | 3 |
| 15-16 | 2 | 2 | 3 | 3 |
| 17-19 | 3 | 3 | 3 | 3 |
| 20-23 | 1 | 1 | 3 | 3 |
| 24-27 | 3 | 3 | 3 | 3 |
| 28-31 | 2 | 2 | 3 | 3 |
| 32-33 | 3 | 3 | 3 | 3 |
| 34 | 3 | 3 | 3 | 3 |
| 35 | 3 | 3 | 3 | 3 |

cu_qp_delta can change the value of QPY for a quantization group of coding units where the quantization group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbour quantization group of coding units in the current slice. If the left neighbour quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order that is available in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice. In addition, for the first set of quantization group of coding units in a tile QPY,PREV is initially set equal to SliceQPY

where QPY,PREV is predictor of the luma quantization parameter, QPY for delta QP coding, and is calculated QP of previously coded neighbouring quantization groups of coding units around the current  quantization group of coding units. If neighbouring quantization groups of coding units are not within the current LCU which is the same as the current quantization group of coding unit, that is, neighbouring quantization groups of coding units are outside of LCU boundary, neighbouring quantization groups of coding unit are not available, and QPs of neighbouring quantization groups of coding unit are considered to be not available in the follow description. QPY,PREV is calculated as the weighted average between the QP of left quantization group of coding units and the QP of above quantization group of coding units. The weighted coefficients of left and above quantization group of coding units are given based on the detection which is equal to the QP of above-left quantization group of coding units.

The predicted QP is derived as following steps.

- If the QP of left quantization group of coding units is equal to the QP of above-left quantization group of coding units,

the predicted QP is calculated as the weighted average between the QP of left quantization group of coding units and the QP of above quantization group of coding units if both left and above quantization group of coding units are available. The value of predicted QP is derived as:

predictedQP = (QPL+3* QPA+3)>>2							(7-22)

(2)  Otherwise, if the QP of above quantization group of coding units is equal to the QP of above-left quantization group of coding units,

the predicted QP is calculated as the weighted average between the QP of left quantization group of coding units and the QP of above quantization group of coding units if both left and above quantization group of coding units are available. The value of predicted QP is derived as:

predictedQP = (3*QPL+ QPA+3)>>2							(7-23)

(3)  Otherwise,

the predicted QP is calculated as the average between the QP of left quantization group of coding units and the QP of above quantization group of coding units if both left and above quantization group of coding units are available. The value of predicted QP is derived as:

predictedQP = (QPL+ QPA+1)>>1							(7-24)

If QPY,PREV is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group of coding units in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

QP of the current quantization group of coding units is calculated to add delta QP to the predictive QP.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

QP’Y = QPY + QpBdOffsetY		(7-25)