| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0198 MPEG Number: m23072 |
| --- | --- |

| Title: | Comments on Slice-independent Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming LI Ping WU No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | +86-25-88014641 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In this document, it is proposed to move the flag namely loop_filter_across_slice_flag from SPS to PPS, which, jointly with tile_boundary_independence_flag adopted in PPS at the 7th meeting, provides flexible tradeoff control at picture level among coding efficiency, parallel decoding and probably error resilience.

# Introduction

Slice-independent deblocking filter (DF) and slice-independent adaptive loop filter (ALF) [1] was adopted to HEVC at Daugu meeting. A flag, namely loop_filter_across_slice_flag, was introduced into SPS to specify whether the in-loop filtering operations are performed across slice boundary [2] in coding a whole sequence. At the 7th meeting, similar concept to slice-independent loop filtering was applied to tiles, and tile_boundary_independence_flag was added in PPS (also in SPS) with Tiles parameters [3].

# Proposal

To facilitate the tradeoff control among coding efficiency, parallel decoding and probably error resilience at picture level, we propose to move the flag loop_filter_across_slice_flag from SPS to PPS, as presented in Table 1. Thereby, the slice related syntax elements, e.g. constrained_intra_pred_flag, slice_ granularity, etc, and Tiles related elements can all be signalled in PPS to make flexible coder control. It is recommended to adopt this change into HEVC design.

Table 1. Modified PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constrained_intra_pred_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| slice_granularity | u(2) |
| … |  |
| } |  |

loop_filter_across_slice_flag equal to 1 specifies the in-loop filtering operations are performed across slice boundary; otherwise, the in-loop operations are slice-independent and not across slice boundary. The in-loop filtering operations include deblocking filter, sample adaptive offset, and adaptive loop filter.

# References

- Chia-Yang Tsai, Chih-Wei Hsu, Yu-Wen Huang, Ching-Yeh Chen, Chih-Ming Fu, Shawmin Lei, “Slice Boundary Processing and Picture Layer Raw Byte Sequence Payload”, JCT-VC Document, JCTVC-D128, Daegu, Korea, January 2011.

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan and Thomas Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding”, JCT-VC Document JCTVC-G1103_d6, November 2011, Geneva, Switzerland.

- Chia-Yang Tsai, Chih-Wei Hsu, Ching-Yeh Chen, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei and Arild Fuldseth, “AHG4: Non-cross-tiles loop filtering for independent tiles”, JCT-VC Document, JCTVC-G194, Geneva, Switzerland, November 2011.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).