| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0199 |
| --- | --- |

| Title: | Non-CE9: Derivation process of reference indices for temporal merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima Hideki Takehara JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com takehara.hideki@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the HM5.0, the reference indices of temporal merging candidates are derived by  referring the reference index of the left neighboring block. Because the derivation refers the reference index of neighbouring block in the same coding  unit, the indices of prediction units in the same coding unit can not be derived in parallel processing.

This contribution proposes that the derivation process of reference indices for temporal merging candidates not referring the reference index in the same coding unit. Therefore the indices of prediction units in the same coding unit can be derived in parallel processing.

The simulation results report that the proposed technique provides no coding loss for random access settings and low delay B settings.

# Introduction

In the HM5.0, the reference indices of temporal merging candidates are derived by referring the reference index of the left neighbouring block. Because the derivation refers the reference index of neighbouring block in the same coding  unit, the indices of prediction units in the same coding unit are not derived in parallel processing.

This contribution proposes the derivation process of reference indices for temporal merging candidates not referring the reference index of neighbouring block in the same coding unit. Therefore the indices of prediction units in the same coding unit can be derived in parallel processing.

# Algorithm Description

## Current algorithm (HM5.0)

In the HM5.0, the reference indices of temporal merging candidates are derived from the reference indices of the left neighboring block A1.

Figure 1 Referred neighboring block in the HM5.0

## Proposed algorithm

### Proposal 1

In the Proposal 1, if PartMode of the current CU is equal to PART_2Nx2N or PART_2NxN or PART_2NxnU or PART_2NxnD, the reference indices of temporal merging candidates are derived from the reference indices of the left neighboring block A1. If PartMode of the current CU is equal to PART_Nx2N or PART_nLx2N or PART_nRx2N, the reference indices of temporal merging candidates are derived from the reference indices of the above neighboring block B1.

Figure 2 Referred neighboring block referred in Proposal 1

### Proposal 2

In the Proposal 2, if PartIdx of the currnet PU is equal to 0, the reference indices of temporal merging candidates are derived from the reference indices of the neighboring block A1. Otherwise, the reference indices of temporal merging candidates are set equal to 0.

Figure 3 referred neighboring block referred in Proposal 2

### Proposal 3

In the Proposal 3, the reference indices of temporal merging candidates are always set equal to 0.

## Implementation

### Software

These proposed techniques are implemented into HM5.0 software.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

## Proposal 1

Table 2, Table 3 and Table 4 present the simulation results of Proposal 1 for random access, random access 10 and low delay B, respectively.

Table 2 Simulation results of Proposal 1 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table 3 Simulation results of Proposal 1 for random access 10

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4 Simulation results of Proposal 1 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.2% | 0.0% | -0.3% | -0.1% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.2% | 0.0% | 0.0% | 0.8% | 0.1% |
| Class E | 0.0% | -0.6% | -0.1% | 0.1% | -0.3% | 0.5% |
| Overall | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.2% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | 0.0% | -0.3% | 0.0% | -0.3% | -0.6% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

The simulation results report that the Proposal 1 provides no coding loss for random access and random access 10 settings and low delay B settings. (See the attached excel sheet)

## Proposal 2

Table 5,Table 6 and Table 7 present the simulation results of Proposal 2 for random access, random access 10 and low delay B, respectively.

Table 5 Simulation results of Proposal 2 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6 Simulation results of Proposal 2 for random access 10

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 7 Simulation results of Proposal 2 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.6% | 0.0% | 0.0% | 0.4% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | -0.2% | 0.2% | 0.0% | 0.7% | 0.3% |
| Class E | 0.0% | 0.3% | 0.0% | 0.0% | -0.5% | 0.3% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.3% |
|  | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class F | -0.2% | -0.3% | -0.2% | 0.0% | 0.1% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

The simulation results report that the Proposal 2 provides no coding loss for random access and random access 10 settings and low delay B setting. (See the attached excel sheet)

## Proposal 3

Table 8, Table 9 and Table 10 present the simulation results of Proposal 3 for random access, random access 10 and low delay B, respectively.

Table 8 Simulation results of Proposal 3 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table 9 Simulation results of Proposal 3 for random access 10

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 10 Simulation results of Proposal 3 for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.8% | 0.0% | 0.3% | 0.5% |
| Class C | 0.1% | 0.1% | 0.2% | 0.0% | 0.3% | 0.2% |
| Class D | 0.2% | 0.5% | 1.1% | 0.1% | 1.0% | 0.8% |
| Class E | 0.2% | 0.6% | 0.4% | 0.1% | 0.4% | 0.7% |
| Overall | 0.1% | 0.4% | 0.7% | 0.1% | 0.5% | 0.6% |
|  | 0.1% | 0.4% | 0.6% | 0.1% | 0.5% | 0.6% |
| Class F | -0.2% | -0.1% | -0.5% | -0.1% | -0.7% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

The simulation results report that the Proposal 3 provides no coding loss for random access and random access 10 settings and 0.1% BD-rate loss for low delay B setting. (See the attached excel sheet as for the more detailed results)

# Conclusions

We recommend that Proposal 1 or Proposal 2 is adopted into CD and next HM.

# References

[1] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G, Geneva, November 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

[3] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Changes

## Proposal 1

8.4.2.2.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows..

- If PartMode of the current prediction unit is PART_2Nx2N or PART_2NxN or PART_2NxnU or PART_2NxnD and the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- Otherwise if PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and the prediction unit covering luma location ( xP + nPSW − 1,  yP – 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xP + nPSW − 1,  yP – 1 ].

- Otherwise, refIdxLX is set equal to 0.

## Proposal 2

8.4.2.2.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows..

- If PartIdx of current prediction unit is equal to 0 and the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- Otherwise, refIdxLX is set equal to 0.

## Proposal 3

8.4.2.2.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is set equal to 0.