Proposal.1. weak offset values for partition boundary

#### Modification process for sample adaptive offset

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a pair of variables ( rx, ry ) specifying the region index,

–	a sample location ( x0, y0 ) specifying the top-left sample of the current largest coding unit,

–	a variable saoDepth specifying the split depth of the region,

–	a variable saoWidthInLCUs specifying the number of largest CUs in width of the current region

–	a variable saoHeightInLCUs specifying the number of largest CUs in height of the current region.

Output of this process is a modified picture buffer for the chroma component cIdx.

Depending on sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ], the following applies:

–	If sao_split_flag[cIdx][saoDepth][rx][ry] is equal to 1 and saoDepth is smaller than SaoMaxDepth, the following ordered stesps apply:

- Variable nS is set equal to ( 1 << Log2MaxCUSize ).

- Variables saoWidthInLCU1, saoWidthInLCU2, saoHeightInLCU1, saoHeightInLCU2, x1, y1, x2, and y2 are derived as follows:

saoWidthInLCU1 = saoWidthInLCUs >> 1		(8-1)
	saoWidthInLCU2 = saoWidthInLCUs – saoWidthInLCU1	(8-2)
	saoHeightInLCU1 = saoHeightInLCUs >> 1		(8-3)
	saoHeightInLCU2 = saoHeightInLCUs – saoHeightInLCU1	(8-4)

x1 = x0		(8-5)
	y1 = y0		(8-6)
	x2 = x1 + saoWidthInLCU1 * nS		(8-7)
	y2 = y1 + saoHeightInLCU1 * nS		(8-8)

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx, 2*ry ), the top-left sample position of largest CU ( x0, y0 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU1, and the variable saoHeightInLCUs set equal to saoHeightInLCU1.

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx+1, 2*ry ), the top-left sample position of largest CU ( x1, y0 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU2, and the variable saoHeightInLCUs set equal to saoHeightInLCU1.

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx, 2*ry+1 ), the top-left sample position of largest CU ( x0, y1 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU1, and the variable saoHeightInLCUs set equal to saoHeightInLCU2.

- The sample adaptive offset process for region as specified in this subclause is invoked with the component index cIdx, the region index ( 2*rx+1, 2*ry+1 ), the top-left sample position of largest CU ( x1, y1 ), the variable saoDepth set equal to ( saoDepth+1 ), the variable saoWidthInLCUs set equal to saoWidthInLCU2, and the variable saoHeightInLCUs set equal to saoHeightInLCU2.

- Otherwise, (sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following ordered steps apply:

- Variable nS is set equal to ( 1 << Log2MaxCUSize ).

- Modification process for samples in a largest coding unit covering ( iC, jC ) with iC = x0 + k*nS for k = 0..saoWidthInLCU – 1 and jC = y0 + j*nS for j = 0..saoHeightInLCU – 1 specified in subclause Error: Reference source not found is invoked with the component index cIdx, the top-left sample of current largest coding unit ( iC, jC ), the region index ( rx, ry ), the split depth saoDepth, the offset array saoValueArray set equal to SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ], the top-left sample of the current region (sx, sy) with sx = x0 and sy = y0, the bottom-right sample of the current region (ex, ey) with ex = x0 + saoWidthInLCU *nS - 1 and ex = x0 + saoWidthInLCU *nS - 1, and the largest coding unit size nS as inputs and the output is the modified picture buffer of chroma component cIdx.

##### Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the region index,

–	a variable saoDepth specifying the split depth of the region,

–	an array saoValueArray specifying offset values,

–	a block size nS.

–	a sample position ( sx, sy ) specifying the the top-left sample of the current region.

–	a sample position ( ex, ey ) specifying the the bottom-right sample of the current region.

Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of chroma component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Error: Reference source not found.

- A variable edgeIdx is specified as

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-9)

- Variables dist2HL, dist2HR, dist2VT, dist2VB, dist2Boundary are derived as follows:

dist2HL = xC + i – sx									(8-xxx)

dist2HR = ex – ( xC + i ) 								(8-xxx)

dist2VT = yC + j – sy									(8-xxx)

dist2VB = ey – ( yC + j )									(8-xxx)

- if cIdx is equal to 0,

dist2Boundary = Min ( Min (dist2HL, dist2HR, dist2VT, dist2VB ), 4 )			(8-xxx)

- Otherwise, if cIdx is equal to one of the values of 1 or 2,

dist2Boundary = Min ( Min (dist2HL, dist2HR, dist2VT, dist2VB ), 2 )			(8-xxx)

- The reconstructed picture buffer is modified as follows:

- if cIdx is equal to 0,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

(saoValueArray[ edgeTable[ edgeIdx ] ]*dist2Boundary ) >>2	(8-xxx)

- Otherwise, if cIdx is equal to one of the values of 1 or 2,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

(saoValueArray[ edgeTable[ edgeIdx ] ]*dist2Boundary ) >>1	(8-xxx)

-

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 6, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- Variables dist2HL, dist2HR, dist2VT, dist2VB, dist2Boundary are derived as follows:

dist2HL = xC + i – sx									(8-xxx)

dist2HR = ex – ( xC + i )	 								(8-xxx)

dist2VT = yC + j – sy									(8-xxx)

dist2VB = ey – ( yC + j )									(8-xxx)

- if cIdx is equal to 0,

dist2Boundary = Min ( Min ( dist2HL, dist2HR, dist2VT, dist2VB ), 4 )			(8-xxx)

- Otherwise, if cIdx is equal to one of the values of 1 or 2,

dist2Boundary = Min ( Min ( dist2HL, dist2HR, dist2VT, dist2VB ), 2 )			(8-xxx)

- The reconstructed picture buffer is modified as follows:

- if cIdx is equal to 0,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

(saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ]*dist2Boundary ) >> 2

(8-xxx)

- Otherwise, if cIdx is equal to one of the values of 1 or 2,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

(saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ]*dist2Boundary ) >> 1

(8-xxx)

-

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Error: Reference source not found.

–	Otherwise (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS-1 and j = 0..nS-1	(8-12)

Proposal.2. weak offset values for category boundary

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 6, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- The reconstructed picture buffer is modified as follows:

- if ( recPicture[ xC + i, yC + j ] % (1 << 5) ) is equal to one of the values of 0 or ( 1 << bandShift ) – 1,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ]				(8-xxx)

- Otherwise, if ( recPicture[ xC + i, yC + j ] % (1 << 5) ) is equal to one of the values of 1 or ( 1 << bandShift ) – 2,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

( saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ] >> 1 ) (8-xxx)

- Otherwise,

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ] 	(8-xxx)

-

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Error: Reference source not found.