| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0201 WG11 Number: m23075 |
| --- | --- |

| Title: | Non-CE8: Method of removing boundary artifacts for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toru Matsunobu Hisao Sasai Takahiro Nishi | Tel: Email: | +81-6-6900-9689 matsunobu.toru@jp.panasonic.com sasai.hisao@jp.panasonic.com nishi.takahiro@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, visual artifact caused by SAO[1] in HM5.0 was reported and an approach to remove the artifact was proposed. The artifact was reportedly observed at SAO partition boundaries and category boundaries. In the proposal, a weak offset value is added to reconstructed samples along with the boundaries. It was reported that the artifact was not observed when this proposal is applied on top of HM5.0. Experimental results reportedly showed that the proposed changes can be introduced on top of HM5.0 with 0.1%-0.3% average BD-rate loss for the common test conditions.

# Introduction

We observed that it occasionally results in two types of visual artifact. Fig.1 shows an example of the first type of visual artifact. BasketballDrill sequence is compressed using all intra high complexity configuration with QP 37. Fig.1 (a) is the original 6th frame. (b) is the part of original 6th frame. (c) is the result of HM5.0. (d) is the result of HM5.0 without SAO. (e) is the difference between (c) and (d). There is a vertical sharp line in (c). This is a visible partition boundary of SAO. Fig.2 shows an example of the second type of visual artifact. Spincalendar sequence[2] which is available to MPEG-2 is compressed using all intra high complexity configuration with QP 37. Fig.2 is indicated in the same way as Fig. 1. There is an artifact like ‘spot’ in (c). It is a visible category boundary of SAO. The issues should be fixed if SAO would be available in HEVC.

(a) Original

(b) Original	(c) HM5.0	(d) HM5.0 without SAO  (e) Diff. of (c) and (d)

Fig.1. Subjective image quality comparison – BasketballDrill (6th frame, QP37, all intra HE configuration)

(a) Original

(b) Original	(c) HM5.0	(d) HM5.0 without SAO  (e) Diff. of (c) and (d)

Fig.2. Subjective image quality comparison – Spincalendar (1st frame, QP37, all intra HE configuration)

# Problems

SAO adds offset values to reconstructed samples at each partition and category individually. Fig.3 shows an example of partitioning of a picture. Fig.4 shows an example of classifying samples in a partition by band offset. By SAO, spatial continuity is broken on the partition boundary and the category boundary. Fig.5 shows an example of the discontinuity that occurs on a partition boundary. A blue line indicates intensities before SAO, and a red line indicates intensities after SAO. The difference between neighboring samples on the partition boundary is increased by addition of very different offset values. This can be visible as a visual artifact. Especially such a artifact is easily observed in flat and gradation.

Fig.3. Partitioning of picture          Fig.4. Classifying samples by band offset

Fig.5. Discontinuity on partition boundary

# Solutions

Because it is difficult to compensate reconstructed samples without partitioning and classifying, we propose to add weak offset values to samples near a partition boundary and a category boundary.

Proposal 1. weak offset for Partition boundary

Fig.6 shows how to weaken offset values near the partition boundary. The cells indicate samples of a reconstructed image, and a figure in each cell indicates a weight coefficient that is multiplied by offset values. The nearer the partition boundary, the smaller the offset value becomes.

Fig.6. Partition boundary

Proposal 2. weak offset for Category boundary

Fig.7 shows how to weaken offset values near category boundary of band offset. The nearer the category boundary, the smaller the offset value becomes.

Fig.7. Category boundary

# Experimental Results

BD-rate performance

The proposed method performance relative to the HM5.0 software was checked under the common test condition described in JCTVC-G1200[3]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. Table 1.1 shows BD-rate of proposal 1. Table 1.2 shows BD-rate of proposal 2. Table 1.3 shows BD-rate of the combination of proposal 1 and 2.

Subjective image quality

Fig. 8 and Fig. 9 shows comparison with the reconstructed images. They are results coded on QP37 for all intra HE configuration. (f) is the result of proposal 1. (g) is the result of proposal 2. (h) is the result of the combination of proposal 1 and 2. The sequence of Fig. 8 is BasketballDrill, and the sequence of Fig.9 is Spincalendar. Image quality of the proposed method has made improvement comparing with the HM5.0 anchor.

# Conclusion

Visual artifact caused by SAO in HM5.0 was found and an approach to remove the artifact was proposed in this contribution. We confirmed that the artifact was removed by this proposal on top of HM5.0. Considering the good trade-off between the noticeable subjective quality improvement and the not significant BR-rate loss (less than 0.3% in average), we suggest to consider the inclusion of this proposed changes in the HM/CD or in a CE for further study.

Table.1.1. BD-rate results of proposal 1 for partition boundary

Table.1.2. BD-rate results of proposal 2 for category boundary

Table.1.3. BD-rate results of the combination of proposal 1 and 2

(c) HM5.0   　　　(f) proposal 1          (g) proposal 2      　　(h) combination of

(f) and (g)

Fig.8. Subjective image quality comparison – BasketballDrill (6th frame, QP37, all intra HE configuration)

(c) HM5.0   　　　(f) proposal 1          (g) proposal 2      　　(h) combination of

(f) and (g)

Fig.9. Subjective image quality comparison – Spincalendar (1st frame, QP37, all intra HE configuration)

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1103, JCT-VC 7th Meeting, Geneva, Nov. 2011.

- “Description of new High Definition Video Test Sequences”, ISO/IEC JTC1/SC29/WG11, MPEG02/N5125, Shanghai, Oct, 2002.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, JCT-VC 7th Meeting, Geneva, Nov. 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).