| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0202 |
| --- | --- |

| Title: | PU structure for intra NxN |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

The PU structure for intra NxN is not clear in the HM5.0 implementation. On the other hand, the WD5 description does not match the HM5.0 software implementation.

This contribution recommends that the prediction unit syntax described in Committee Draft corresponds to HM implementation by the smallest possible change.

To achieve such purposes, it is suggested that both Committee Draft and HM are slightly changed. The prediction information for both luma and chroma prediction blocks in the same location is coded in the same prediction unit. More specifically, each of the intra prediction mode for each luma and chroma prediction block whose partition index PartIdx are equal to 0 is coded in the same prediction unit in the case of PART_NxN as well as PART_2Nx2N.

# Introduction

The PU structure for intra NxN is not clear in the HM5.0 implementation. On the other hand, the WD5 [1] description does not match the HM5.0 software implementation.

This contribution recommends that the prediction information for both luma and chroma prediction blocks in the same location is coded in the same prediction unit.

# Current and Proposed PU structure

## NxN PU structure for 4:2:0 sampling

Partitioning methods of chroma samples in the PART_NxN case differ between MODE_INTRA and MODE_INTER in the HM5.0. Figure 1 illustrates each prediction unit structure for PART_NxN in a coding unit for 4:2:0 sampling.

Figure 1 PART_NxN PU structure in a coding unit for 4:2:0 sampling

The location of the top-left of the chroma prediction block is equal to the location of the top-left of the luma prediction block whose partition index PartIdx is equal to 0. Therefore, the partition index PartIdx of the chroma prediction block is equal to 0.

## Intra PU structure in HM5.0 implementation

In the HM5.0 software implementation, a chroma intra prediction mode is coded/decoded after four sets of luma intra prediction mode if prediction partition mode PartMode is equal to PART_NxN. It seems that each of the intra prediction mode for each luma and chroma prediction block whose PartIdx is equals to 0 is coded in the different PUs. The chroma intra prediction mode (whose PartIdx is equal to 0) is derived using the luma intra prediction mode whose PartIdx is equal to 0.

The following is the function of entropy decoding process of intra prediction mode for luma and chroma samples in a coding unit in the HM5.0 implementation.

Void TDecEntropy::decodePredInfo ( TComDataCU* pcCU, UInt uiAbsPartIdx, UInt uiDepth, TComDataCU* pcSubCU ) { PartSize eMode = pcCU->getPartitionSize( uiAbsPartIdx ); if( pcCU->isIntra( uiAbsPartIdx ) ) // If it is Intra mode, encode intra prediction mode. { if( eMode == SIZE_NxN ) // if it is NxN size, encode 4 intra directions. { UInt uiPartOffset = ( pcCU->getPic()->getNumPartInCU() >> ( pcCU->getDepth(uiAbsPartIdx) << 1 ) ) >> 2; // if it is NxN size, this size might be the smallest partition size. decodeIntraDirModeLuma( pcCU, uiAbsPartIdx, uiDepth+1 ); decodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset, uiDepth+1 ); decodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset*2, uiDepth+1 ); decodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset*3, uiDepth+1 ); decodeIntraDirModeChroma( pcCU, uiAbsPartIdx, uiDepth ); } else // if it is not NxN size, encode 1 intra directions { decodeIntraDirModeLuma ( pcCU, uiAbsPartIdx, uiDepth ); decodeIntraDirModeChroma( pcCU, uiAbsPartIdx, uiDepth ); } } else // if it is Inter mode, encode motion vector and reference index { decodePUWise( pcCU, uiAbsPartIdx, uiDepth, pcSubCU ); } }

It seems that the PU for intra chroma is defined independently. Figure 2 illustrates one of supposed PU structures in a CU in the HM5.0 implementation.

Figure 2 PU strucuture in HM5.0 implementation

The examples of the syntax structures corresponding to the HM5.0 implementation are shown in Chapter 7.

## Intra PU structure in WD5 description

Meanwhile, WD5 description does not match the HM5.0 implementation.

In WD5 description, the intra prediction mode for chroma samples is always coded after intra prediction mode for luma samples located at the same location in the same PU when PredMode is MODE_INTRA. However, intra prediction mode for chroma samples whose PartIdx is equals to 1, 2 or 3 are unnecessary in the case of PART_NxN.

Figure 3 illustrates PU structure in a coding unit in WD5.

Figure 3 PU structure in WD5 description

## Proposed intra PU structure

This contribution recommends that the prediction information for both luma and chroma prediction blocks in the same location is coded in the same PU.

### Proposed PU structure in 4:2:0 chroma sampling

In 4:2:0 sampling, each of the intra prediction mode for each luma and chroma prediction block whose partition index PartIdx are equal to 0 is coded in the same PU in the case of PART_NxN as well as PART_2Nx2N. More specifically, the syntax elements of luma and chroma intra prediction mode are coded in the following PU strucuture. Figure 4 illustrates proposed PU strucuture in a coding unit. Because the chroma intra prediction mode can be derived using the luma intra prediction mode in the same PU, the PU structure is simple and straightforward.

Figure 4 Proposed PU structure in 4:2:0 chroma sampling

### PU structure in 4:2:2 and 4:4:4 chroma sampling

We assume that PART_NxN PU structures are defined as illustrated in Figure 5 and Figure 6 in 4:2:2 and 4:4:4 sampling, respectively. On that condition, we recommend that the syntax elements of luma and chroma intra prediction mode are coded in the following PU structures. Figure 7 illustrates the proposed PU structure in a coding unit. In 4:2:2 and 4:4:4 sampling as well as 4:2:0, the proposed PU structure provides that the syntax elements of both luma and chroma intra prediction mode can be included in the same PU.

Figure 5 PART_NxN PU structure in a coding unit in 4:2:2 chroma sampling

Figure 6 PART_NxN PU structure in a coding unit in 4:4:4 chroma sampling

Figure 7 Proposed PU structure in 4:2:2 and 4:4:4 chroma sampling

## Implementation

### Software

This proposed technique is implemented into HM5.0 software.

■TDecEntropy.cpp

Void TDecEntropy::decodePredInfo ( TComDataCU* pcCU, UInt uiAbsPartIdx, UInt uiDepth, TComDataCU* pcSubCU ) { PartSize eMode = pcCU->getPartitionSize( uiAbsPartIdx ); if( pcCU->isIntra( uiAbsPartIdx ) ) // If it is Intra mode, encode intra prediction mode. { if( eMode == SIZE_NxN ) // if it is NxN size, encode 4 intra directions. { UInt uiPartOffset = ( pcCU->getPic()->getNumPartInCU() >> ( pcCU->getDepth(uiAbsPartIdx) << 1 ) ) >> 2; // if it is NxN size, this size might be the smallest partition size. decodeIntraDirModeLuma( pcCU, uiAbsPartIdx, uiDepth+1 ); decodeIntraDirModeChroma( pcCU, uiAbsPartIdx, uiDepth ); decodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset, uiDepth+1 ); decodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset*2, uiDepth+1 ); decodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset*3, uiDepth+1 ); decodeIntraDirModeChroma( pcCU, uiAbsPartIdx, uiDepth ); } else // if it is not NxN size, encode 1 intra directions { decodeIntraDirModeLuma ( pcCU, uiAbsPartIdx, uiDepth ); decodeIntraDirModeChroma( pcCU, uiAbsPartIdx, uiDepth ); } } else // if it is Inter mode, encode motion vector and reference index { decodePUWise( pcCU, uiAbsPartIdx, uiDepth, pcSubCU ); } }

■TEncEntropy.cpp

Void TEncEntropy::encodePredInfo( TComDataCU* pcCU, UInt uiAbsPartIdx, Bool bRD ) { if( bRD ) { uiAbsPartIdx = 0; } PartSize eSize = pcCU->getPartitionSize( uiAbsPartIdx ); if( pcCU->isIntra( uiAbsPartIdx ) ) // If it is Intra mode, encode intra prediction mode. { if( eSize == SIZE_NxN ) // if it is NxN size, encode 4 intra directions. { UInt uiPartOffset = ( pcCU->getPic()->getNumPartInCU() >> ( pcCU->getDepth(uiAbsPartIdx) << 1 ) ) >> 2; // if it is NxN size, this size might be the smallest partition size. encodeIntraDirModeLuma( pcCU, uiAbsPartIdx ); encodeIntraDirModeChroma( pcCU, uiAbsPartIdx, bRD ); encodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset ); encodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset*2 ); encodeIntraDirModeLuma( pcCU, uiAbsPartIdx + uiPartOffset*3 ); encodeIntraDirModeChroma( pcCU, uiAbsPartIdx, bRD ); } else // if it is not NxN size, encode 1 intra directions { encodeIntraDirModeLuma ( pcCU, uiAbsPartIdx ); encodeIntraDirModeChroma( pcCU, uiAbsPartIdx, bRD ); } } else // if it is Inter mode, encode motion vector and reference index { encodePUWise( pcCU, uiAbsPartIdx, bRD ); } }

### Syntax structure

The prediction unit syntax of this proposal is defined as presented in Table 1 when only 4:2:0 sampling is considered like current WD5 syntax and HM5.0 software.

Table 1 predition unit syntax of this proposal in 4:2:0 sampling

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| if( PartIdx == 0 ) /* Part_2Nx2N and PART_NxN */ |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| …… |  |
| } |  |
| } |  |

The prediction unit syntax of this proposal is defined as presented in Table 2 when monochrome, 4:2:0, 4:2:2 and 4:4:4 are considered.

Table 2 prediction unit syntax of this proposal 
in monochrome, 4:2:0, 4:2:2 and 4:4:4 sampling

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| if( ( ChromaArrayType != 0 && PartIdx == 0 ) ¦¦ ( ChromaArrayType == 2 && PartIdx == 2 ) ¦¦ ChromaArrayType == 3 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| …… |  |
| } |  |
| } |  |

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table 3.

Table 3 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 4 presents the simulation results for intra only.

Table 4 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

Our recommendations are as follows:

- The prediction unit syntax described in Committee Draft should correspond exactly to HM by the smallest possible change.

- The proposed PU structure is adopted in Committee Draft and next HM.

  - CD: The prediction unit syntax is modified as presented in Table 1 for 4:2:0 sampling.

  - HM: The coding order of intra prediction mode is modified.

Desirable further studies are as follows:

- The prediction unit structures are defined as illustrated in Figure 5 and Figure 6 for 4:2:2 or 4:4:4 sampling.

- The prediction unit syntax is modified as presented in Table 2 for monochrome, 4:2:0, 4:2:2 or 4:4:4 sampling.

# References

[1] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G, Geneva, November 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

[3] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The syntax structures complying with HM5.0 are presented without changing HM implementation.

## Syntax structure 1 (SS1)

The coding unit and prediction unit syntax complying with HM5.0 is defined as presented in Table 5 and Table 6.

In this syntax structure, prediction unit for intra chroma is independently defined.

Table 5 SS1: coding unit syntax complying with HM5.0

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| IntraChroma = 0 |  |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| if( PredMode = = MODE_INTRA ) |  |
| IntraChroma = 1 |  |
| prediction_unit( x0, y0, log2CUSize) /* for chroma intra pred mode */ |  |
| } |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| …… |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| if( PredMode = = MODE_INTRA ) |  |
| IntraChroma = 1 |  |
| prediction_unit( x0, y0, log2CUSize) /* for chroma intra pred mode */ |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |

Table 6 SS1: prediction unit syntax complying with HM5.0

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && pcm_enabled_flag && log2CUSize >= Log2MinIPCMCUSize && log2CUSize <= Log2MaxIPCMCUSize && IntraChroma == 0 ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| if ( IntraChroma == 0) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| } else { /* IntraChroma == 1 */ |  |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } |  |
| } else { |  |
| if ( IntraChroma == 0) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } else { /* IntraChroma == 1 */ |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| …… |  |
| } |  |
| } |  |

## Syntax structure 2 (SS2)

The prediction unit syntax complying with HM5.0 is defined as presented in Table 7.

In this syntax structure, chroma intra prediction mode of MODE_INTRA and PART_NxN in the prediction unit the PartIdx of which is equal to 3.

Note: The chroma intra prediction mode [PartIdx=3] is derived using the luma intra prediction mode [PartIdx=0]. Semantics needs to be changed. The location of the chroma intra prediction mode [PartIdx=3] is different from the location of the luma intra prediction mode [PartIdx=3].

Table 7 SS2: prediction unit syntax complying with HM5.0

| prediction_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && pcm_enabled_flag && log2CUSize >= Log2MinIPCMCUSize && log2CUSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| if ( PartMode == PART_2Nx2N ) { |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } else if (PartIdx == 3) { /* PartMode == PART_NxN */ |  |
| x1 = x0 - ( 1 << (log2CUSize-1) ) |  |
| y1 = y0 - ( 1 << (log2CUSize-1) ) |  |
| intra_chroma_pred_mode[ x1 ][ y1 ] | ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x1 ][ y1 ] == 3 : intra_chroma_pred_mode[ x1 ][ y1 ] == 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| …… |  |
| } |  |
| } |  |