# Working Draft description of JCTVC-H0204 Proposal 2

This document describes the Working Draft description of JCTVC-H0204 Proposal 2 modified from Working Draft 5.0 (d9).

### 7.4.9	Transform coefficient semantics

The transform coefficient levels are parsed into the arrays transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block.

When PredMode is equal to MODE_INTRA, different scanning orders are used. The array ScanType[ log2TrafoSize − 2 ][ IntraPredMode ], specifying the scanning order for various luma transform block sizes and intra prediction modes, is derived as specified in Table 7-1Error: Reference source not found.

Table 7-1 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 2 | 2 | 0 | 0 |
| 2-3 | 0 | 0 | 0 | 0 |
| 4-5 | 1 | 1 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 |
| 7-8 | 2 | 2 | 0 | 0 |
| 9-10 | 0 | 0 | 0 | 0 |
| 11-12 | 1 | 1 | 0 | 0 |
| 13-14 | 0 | 0 | 0 | 0 |
| 15-16 | 2 | 2 | 0 | 0 |
| 17-19 | 0 | 0 | 0 | 0 |
| 20-23 | 1 | 1 | 0 | 0 |
| 24-27 | 0 | 0 | 0 | 0 |
| 28-31 | 2 | 2 | 0 | 0 |
| 32-33 | 0 | 0 | 0 | 0 |
| 34 | 0 | 0 | 0 | 0 |
| 35 | 0 | 0 | 0 | 0 |

cu_qp_delta can change the value of QPYspecifies the difference between a luma quantization parameter to be used and its prediction for a quantization group of coding units where the quantization group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The derivation process for quantization parameters as specified in suclause 8.5.1 is invoked.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order that is available in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice. In addition, for the first set of quantization group of coding units in a tile QPY,PREV is initially set equal to SliceQPY

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

The values of QP’Cb and QP’Cr are derived as

QP’Cb = Clip3(0, 51, QP’Y + chroma_cb_qp_offset)	(7-3)

QP’Cr = Clip3(0, 51, QP’Y + chroma_cr_qp_offset)	(7-4)

### 8.5.1	Derivation process for quantization parameters

Inputs of this process are:

–	luma location ( xB, yB ) specifying the top-left luma sample of the current quantization group of coding units relative to the top-left luma sample of the current picture,

–	a variable CurrTbAddr specifying the treeblock address of the treeblock containing the current quantization group of coding units,

–	a variable QPY,A specifying the luma quantization parameter of the quantization group of coding units covering ( xB-1,  yB ),

–	a variable tbAddrA specifying the treeblock address of treeblock containing the quantization group of coding units covering ( xB-1,  yB ),

–	a variable QPY,B specifying the luma quantization parameter of the quantization group of coding units covering ( xB,  yB-1 ),

–	a variable , tbAddrB specifying the treeblock address of treeblock containing the quantization group of coding units covering ( xB,  yB-1 ),

–	a variable QPY,PREV specifying the luma quantization parameter of the previous quantization group of coding units in decoding order,

–	a variable cu_qp_delta specifying the difference between a luma quantization parameter to be used and its prediction for a quantization group of coding units,

–	a variable QpBdOffsetY specifying the value of the luma quantisation parameter range offset,

–	variables chroma_cb_qp_offset, chroma_cr_qp_offset specifying the offset to the luma quantization parameter QP’Y used for deriving QP’Cb and QP’Cr , respectively.

Output of this process is:

–	luma quantization parameter QP’Y,

–	chroma quantization parameters QP’Cb and QP’Cr,

For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice. In addition, for the first set of quantization group of coding units in a tile QPY,PREV is initially set equal to SliceQPY.

The predictive luma quantization parameter QPY,PRED is derived as follows:

- If the quantization group of coding units covering ( xB-1,  yB ) in the current slice is not available or tbAddrA is not equal to CurrTbAddr, then QPY,A is set equal to QPY,PREV.

- If the quantization group of coding units covering ( xB,  yB-1 ) in the current slice is not available or tbAddrB is not equal to CurrTbAddr, then QPY,B is set equal to QPY,PREV.

- QPY,PRED =  (QPY,A + QPY,B + 1) >> 1		(8-xx)

The value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-xx)

The value of luma quantization parameter QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-xx)

The values of chroma quantization parameters QP’Cb and QP’Cr are derived as

QP’Cb = Clip3(0, 51, QP’Y + chroma_cb_qp_offset)	(8-xx)

QP’Cr = Clip3(0, 51, QP’Y + chroma_cr_qp_offset)	(8-xx)