| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0204 |
| --- | --- |

| Title: | Non-CE4: Compatible QP prediction with RC and AQ |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Masayoshi Nishitani Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com nishitani.masayoshi@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution proposes spatial QP prediction method under restricting QP reference across LCU boundary. The proposed method is based on the CE4 Subtest 1.3 and modified to use QP of the preceding QGCU (Quantization Group of Coding Unit) more frequently for further rate control friendliness. In the combination with the QP prediction based on intra prediction direction in CE4 Subtest1.3, it can be expected to achieve higher efficiency improvement. The simulation results in the case of MaxCUDepthDQP=3(8x8) is reported as follows. The proposed method with the QP prediction based on intra prediction direction provides 0.5%, 0.4%, 0.3%, 0.3%, 0.2%, 0.3% and 0.3% luminance BD-rate gains for AIHE, AILC, RAHE, RALC, RAHE10, LDHE, and LDLC configurations, respectively. The proposed method without the QP prediction based on intra prediction direction provides 0.3%, 0.3%, 0.3%, 0.3%, 0.2%, 0.3% and 0.3% luminance BD-rate gains  for AIHE, AILC, RAHE, RALC, RAHE10, LDHE, and LDLC configurations, respectively.

# Introduction

In CE4 Subtest 1 [1], it was reported for delta QP signaling that spatial QP prediction derived the predicted QP by using QPs of neighbouring QGCUs  (Quantization Group of Coding Unit), which were on the left and above of the current QGCU. However, in the last meeting, it was taken up as the problem that spatial QP prediction was restricted to use QPs of neighbouring QGCUs across LCU boundary. If neighbouring QGCUs are not within the current LCU, QPs of those QGCU are not available. Therefore, the predicted QP is not adequate in QP prediction, and the larger magnitude of delta QP is, the more bits will be required.

To resolve the above mentioned problem, this contribution is based on the CE4 Subtest 1.3 and modified to use QP of the preceding QGCU more frequently for further rate control friendliness.

# Algorithm Description

## Current algorithm (HM5.0)

In the HM5.0 [2], QP of the left neighbouring QGCU is employed as the predicted QP. If the left neighbouring QGCU in the current slice is not available, QP of the preceding QGCU in scan order that is available in the current slice is employed as the predicted QP. However, in CE4 Subtest 1, the QP of the preceding QGCU in scan order is always employed as the predicted QP, which is defined as Subtest 1.1　(the anchor of CE4 Subtest 1).

This proposal is based on CE4 subtest 1.3 CE4 subtest 1.3 (JCTVC-G1028 [3]). It is simpler because conditional judgements are less. In addition, it is more friendliness for rate control because more preceding QP is used in QP calculation.

## Proposed algorithm

### Proposal 1

In the following condition, each neighbouring QGCU which is not within the current LCU is regarded as not available in the same way as in CE4 Subtest 1.3.

In Proposal 1, the predicted QP is derived as following steps:

- If the top-left-most CU in the current QGCU is intra-coded and either one of the following conditions (a) or (b) is satisfied, the predicted QP is derived along intra prediction direction of the top-left-most PU with the method of CE4 subtest 1.3 (JCTVC-G1028 [3]):

- intraPredOrder is more than 4 and less than 18, and the above neighbouring QGCU is available.

- intraPredOrder is more than 20 and less than 34, and the left neighbouring QGCU is available.

where intraPredOrder is derived by Table 8-6 in HM5.0.

- Otherwise, the predicted QP is derived as following steps:

  - If the left neighbouring QGCU is not available, the QPL is set equal to QP of the preceding QGCU.

  - If the above neighbouring QGCU is not available, the QPA is set equal to QP of the preceding QGCU.

  - The predicted QP is derived as follows:
	predictedQP=QPL+QPA+1>>1.

Remarks:

- QPL: QP of the left neighbouring QGCU

- QPA: QP of the above neighbouring QGCU

### Proposal 2

In Proposal 2, the predicted QP is derived by only step 2 of Proposal 1.

The proposed method is a rate-control-friendly method to substitute QP of the preceding QGCU for QPs of neighbouring QGCUs if not available. As the QP of the preceding QGCU is derived just before QP of the current QGCU in scan order by rate control technique, it is considered that the QP of the preceding QGCU is close to QP of the current QGCU. Therefore, if neighbouring QGCUs are not available, it is appropriate to substitute the QP of the preceding QGCU for QPs of those QGCUs.

## Implementation

### Software

The proposed QP prediction algorithm is implemented onto the software for HM 5.0 with the CE4 subtest 1.

# Experiments

## Test conditions

Experiments to evaluate performance of the proposed method are conducted under the CE4 Subtest 1 test conditions that are based on the JCTVC common test condition defined in [4], with MaxCUDQPSize equal to 3(8x8) and 2(16x16).

## Experimental environment

The experimental environment used is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

### Proposal 1

#### MaxCUDepthDQP=3(8x8)

In the case of MaxCUDepthDQP=3(8x8), Table 2, Table 3, Table 4, and Table5 present the simulation results for intra only, random access, random access 10 and low delay, respectively.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.4% | -0.4% | -0.4% | -8.2% | -0.3% | -0.3% | -0.3% | -8.4% |
| Class B | -0.4% | -0.4% | -0.4% | -10.4% | -0.3% | -0.3% | -0.3% | -10.7% |
| Class C | -0.5% | -0.5% | -0.5% | -12.0% | -0.5% | -0.5% | -0.4% | -12.4% |
| Class D | -0.5% | -0.5% | -0.4% | -12.1% | -0.4% | -0.4% | -0.4% | -12.2% |
| Class E | -0.7% | -0.8% | -0.7% | -12.0% | -0.7% | -0.7% | -0.7% | -13.9% |
| Overall | -0.5% | -0.5% | -0.5% | -11.2% | -0.4% | -0.4% | -0.4% | -11.7% |
|  | -0.5% | -0.5% | -0.5% |  | -0.4% | -0.4% | -0.4% |  |
| Class F | -0.3% | -0.3% | -0.3% | -7.9% | -0.2% | -0.2% | -0.3% | -8.8% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 3 Simulation results for random access

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.3% | -0.3% | -0.3% | -5.3% | -0.4% | -0.2% | -0.3% | -5.3% |
| Class B | -0.3% | -0.2% | -0.3% | -6.2% | -0.3% | -0.1% | -0.1% | -6.0% |
| Class C | -0.4% | -0.3% | -0.3% | -8.1% | -0.4% | -0.3% | -0.3% | -7.9% |
| Class D | -0.3% | -0.3% | -0.3% | -7.6% | -0.3% | -0.5% | -0.3% | -7.7% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.3% | -7.0% | -0.3% | -0.3% | -0.2% | -6.9% |
|  | -0.3% | -0.3% | -0.3% |  | -0.3% | -0.3% | -0.2% |  |
| Class F | -0.1% | -0.3% | -0.2% | -5.9% | -0.3% | -0.2% | -0.1% | -6.3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |

Table 4 Simulation results for random access 10

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.3% | -0.3% | -4.3% |
| Class B | -0.3% | -0.4% | -0.3% | -6.2% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.2% | -0.3% | -0.3% | -5.3% |
|  | -0.2% | -0.3% | -0.3% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 5 Simulation results for low delay B

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | 0.3% | -4.3% | -0.2% | -0.1% | 0.0% | -3.8% |
| Class C | -0.3% | -0.6% | -0.4% | -6.2% | -0.4% | -0.5% | -0.1% | -6.0% |
| Class D | -0.3% | -0.3% | 0.0% | -5.9% | -0.4% | -0.6% | -0.6% | -6.1% |
| Class E | -0.2% | -0.6% | -0.1% | -3.6% | -0.3% | -1.0% | -0.3% | -4.4% |
| Overall | -0.3% | -0.4% | 0.0% | -5.1% | -0.3% | -0.5% | -0.2% | -5.0% |
|  | -0.3% | -0.4% | -0.1% |  | -0.3% | -0.5% | -0.3% |  |
| Class F | -0.2% | -0.4% | -0.2% | -3.6% | -0.2% | -0.7% | -0.1% | -3.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |

The simulation results report that the proposed method provides 0.4-0.5% BD-rate [5] gain for intra only, 0.3% BD-rate gain for random access, 0.2% BD-rate gain for random access 10 and 0.3% BD-rate gain for low delay B settings (See the attached excel sheet as for the more detailed results).

#### MaxCUDepthDQP=2(16x16)

In the case of MaxCUDepthDQP=2(16x16), Table 6, Table 7, Table 8, and Table 9 present the simulation results for intra only, random access, random access 10 and low delay, respectively.

Table 6 Simulation results for intra only

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.2% | -0.1% | -5.8% | -0.1% | -0.1% | -0.1% | -5.5% |
| Class B | -0.2% | -0.2% | -0.2% | -8.2% | -0.1% | -0.1% | -0.1% | -8.1% |
| Class C | -0.2% | -0.1% | -0.2% | -9.5% | -0.2% | -0.2% | -0.2% | -9.4% |
| Class D | -0.1% | -0.2% | -0.1% | -8.6% | -0.1% | -0.1% | -0.1% | -8.7% |
| Class E | -0.4% | -0.4% | -0.4% | -11.0% | -0.3% | -0.2% | -0.3% | -11.4% |
| Overall | -0.2% | -0.2% | -0.2% | -8.8% | -0.2% | -0.1% | -0.1% | -8.8% |
|  | -0.2% | -0.2% | -0.2% |  | -0.2% | -0.1% | -0.2% |  |
| Class F | -0.1% | 0.0% | -0.1% | -6.6% | -0.1% | -0.2% | -0.1% | -7.1% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

Table 7 Simulation results for random access

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.1% | -4.0% | -0.2% | -0.1% | -0.1% | -4.1% |
| Class B | -0.1% | -0.2% | 0.1% | -5.0% | -0.2% | 0.0% | -0.1% | -4.7% |
| Class C | -0.2% | -0.1% | 0.1% | -6.4% | -0.2% | -0.2% | -0.1% | -6.4% |
| Class D | -0.2% | 0.0% | -0.2% | -6.1% | -0.2% | -0.2% | -0.2% | -6.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.1% | 0.0% | -5.5% | -0.2% | -0.1% | -0.1% | -5.5% |
|  | -0.2% | -0.1% | 0.0% |  | -0.2% | -0.1% | -0.1% |  |
| Class F | -0.2% | -0.2% | -0.1% | -4.7% | -0.1% | -0.2% | -0.1% | -4.9% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 97% |  |  |  |

Table 8 Simulation results for random access 10

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | 0.0% | 0.2% | -3.4% |
| Class B | -0.1% | -0.2% | -0.2% | -5.0% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | -4.3% |
|  | -0.1% | -0.1% | 0.0% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 9 Simulation results for low delay B

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.2% | -3.9% | -0.2% | -0.2% | -0.5% | -3.6% |
| Class C | -0.2% | -0.3% | -0.3% | -5.6% | -0.1% | -0.3% | -0.2% | -5.5% |
| Class D | -0.1% | 0.0% | -0.2% | -5.4% | -0.2% | 0.3% | -0.3% | -5.5% |
| Class E | -0.2% | -1.1% | -0.9% | -3.8% | -0.2% | -0.6% | 0.5% | -3.9% |
| Overall | -0.1% | -0.4% | -0.3% | -4.7% | -0.2% | -0.2% | -0.2% | -4.6% |
|  | -0.1% | -0.5% | -0.3% |  | -0.2% | -0.2% | -0.1% |  |
| Class F | -0.2% | -0.2% | -0.1% | -2.7% | 0.0% | 0.1% | -0.7% | -2.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 99% |  |  |  |

The simulation results report that the proposed method provides 0.2% BD-rate gain for intra only, 0.2% BD-rate gain for random access, 0.1% BD-rate gain for random access 10 and 0.1-0.2% BD-rate gain for low delay B settings (See the attached excel sheet as for the more detailed results).

### Proposal 2

#### MaxCUDepthDQP=3(8x8)

In the case of MaxCUDepthDQP=3(8x8), Table 10, Table 11, Table 12, and Table 13 present the simulation results for intra only, random access, random access 10 and low delay, respectively.

Table 10 Simulation results for intra only

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.2% | -0.2% | -4.2% | -0.1% | -0.1% | -0.1% | -3.9% |
| Class B | -0.3% | -0.3% | -0.3% | -6.7% | -0.2% | -0.2% | -0.2% | -6.7% |
| Class C | -0.4% | -0.3% | -0.4% | -8.9% | -0.3% | -0.3% | -0.3% | -9.1% |
| Class D | -0.4% | -0.4% | -0.3% | -9.6% | -0.3% | -0.3% | -0.3% | -9.7% |
| Class E | -0.3% | -0.3% | -0.3% | -6.0% | -0.3% | -0.3% | -0.4% | -6.7% |
| Overall | -0.3% | -0.3% | -0.3% | -7.5% | -0.3% | -0.3% | -0.3% | -7.6% |
|  | -0.3% | -0.3% | -0.3% |  | -0.3% | -0.3% | -0.3% |  |
| Class F | -0.1% | -0.1% | -0.1% | -4.3% | -0.1% | -0.2% | -0.1% | -4.0% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 11 Simulation results for random access

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.2% | -0.1% | -0.1% | -3.8% | -0.4% | -0.2% | -0.3% | -3.7% |
| Class B | -0.2% | -0.2% | -0.2% | -4.9% | -0.3% | 0.0% | 0.0% | -4.6% |
| Class C | -0.3% | -0.1% | -0.3% | -6.8% | -0.3% | -0.4% | -0.2% | -6.8% |
| Class D | -0.3% | -0.4% | -0.3% | -7.0% | -0.3% | -0.5% | -0.4% | -6.9% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.2% | -5.8% | -0.3% | -0.3% | -0.2% | -5.7% |
|  | -0.3% | -0.2% | -0.2% |  | -0.3% | -0.3% | -0.2% |  |
| Class F | -0.1% | -0.1% | -0.1% | -3.6% | -0.2% | -0.2% | -0.2% | -3.5% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

Table 12 Simulation results for random access 10

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.1% | -0.5% | -3.2% |
| Class B | -0.2% | -0.3% | -0.1% | -4.9% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.2% | -0.2% | -0.3% | -4.2% |
|  | -0.2% | -0.2% | -0.3% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 13 Simulation results for low delay B

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.2% | -4.0% | -0.3% | -0.3% | -0.4% | -3.5% |
| Class C | -0.3% | -0.3% | -0.1% | -5.9% | -0.3% | -0.2% | -0.1% | -5.5% |
| Class D | -0.3% | -0.5% | -0.2% | -6.0% | -0.3% | -0.4% | 0.0% | -5.8% |
| Class E | -0.1% | -0.4% | -0.6% | -3.3% | -0.2% | -1.4% | 0.1% | -4.0% |
| Overall | -0.2% | -0.4% | -0.2% | -4.8% | -0.3% | -0.5% | -0.1% | -4.7% |
|  | -0.2% | -0.4% | -0.3% |  | -0.3% | -0.5% | 0.0% |  |
| Class F | -0.1% | -0.2% | -0.1% | -2.5% | -0.1% | 1.0% | 0.1% | -2.4% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |

The simulation results report that the proposed method provides 0.3% BD-rate gain for intra only, 0.3% BD-rate gain for random access, 0.2% BD-rate gain for random access 10 and 0.2-0.3% BD-rate gain for low delay B settings (See the attached excel sheet as for the more detailed results).

#### MaxCUDepthDQP=2(16x16)

In the case of MaxCUDepthDQP=2(16x16), Table 14, Table 15, Table 16, and Table 17 present the simulation results for intra only, random access, random access 10 and low delay, respectively.

Table 14 Simulation results for intra only

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.1% | -0.1% | -3.0% | 0.0% | 0.0% | -0.1% | -2.7% |
| Class B | -0.1% | -0.1% | -0.1% | -5.4% | -0.1% | -0.1% | -0.1% | -5.2% |
| Class C | -0.2% | -0.1% | -0.1% | -7.7% | -0.1% | -0.1% | -0.1% | -7.7% |
| Class D | -0.1% | -0.2% | -0.1% | -7.7% | -0.1% | -0.1% | -0.1% | -7.7% |
| Class E | -0.2% | -0.2% | -0.2% | -5.3% | -0.2% | -0.1% | -0.1% | -5.4% |
| Overall | -0.1% | -0.1% | -0.1% | -6.1% | -0.1% | -0.1% | -0.1% | -6.1% |
|  | -0.1% | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |  |
| Class F | 0.0% | 0.1% | 0.0% | -3.3% | 0.0% | -0.1% | 0.0% | -3.4% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 15 Simulation results for random access

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | -0.2% | 0.0% | -3.2% | -0.2% | -0.3% | -0.1% | -3.3% |
| Class B | -0.1% | 0.0% | 0.1% | -4.1% | -0.1% | -0.1% | 0.0% | -4.1% |
| Class C | -0.2% | 0.0% | 0.0% | -5.8% | -0.2% | -0.2% | 0.0% | -5.9% |
| Class D | -0.2% | -0.1% | -0.3% | -5.8% | -0.2% | 0.0% | -0.1% | -6.0% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.2% | -0.1% | 0.0% | -4.9% | -0.2% | -0.1% | -0.1% | -5.0% |
|  | -0.2% | -0.1% | -0.1% |  | -0.2% | -0.1% | -0.1% |  |
| Class F | -0.1% | -0.2% | -0.1% | -2.8% | -0.2% | -0.2% | -0.2% | -2.9% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 98% |  |  |  |

Table 16 Simulation results for random access 10

|  | Random Access HE-10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. |
| Class A (8bit) | -0.1% | 0.1% | 0.1% | -2.9% |
| Class B | -0.1% | -0.1% | 0.0% | -4.2% |
| Class C |  |  |  |  |
| Class D |  |  |  |  |
| Class E |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | -3.6% |
|  | -0.1% | 0.0% | 0.0% |  |
| Class F |  |  |  |  |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 17 Simulation results for low delay B

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | -0.2% | -3.7% | -0.2% | -0.3% | -0.7% | -3.5% |
| Class C | -0.2% | -0.2% | -0.1% | -5.3% | -0.2% | 0.2% | 0.0% | -5.3% |
| Class D | -0.1% | -0.2% | 0.2% | -5.5% | -0.2% | 1.0% | 0.2% | -5.5% |
| Class E | -0.3% | 0.0% | -0.5% | -3.6% | -0.2% | 1.0% | 0.2% | -3.7% |
| Overall | -0.2% | -0.2% | -0.1% | -4.5% | -0.2% | 0.4% | -0.1% | -4.5% |
|  | -0.2% | -0.2% | -0.1% |  | -0.2% | 0.4% | -0.1% |  |
| Class F | -0.2% | 0.1% | -0.3% | -2.0% | 0.0% | -0.2% | -0.4% | -1.9% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 97% |  |  |  | 98% |  |  |  |

The simulation results report that the proposed method provides 0.1% BD-rate gain for intra only, 0.2% BD-rate gain for random access, 0.1% BD-rate gain for random access 10 and 0.2% BD-rate gain for low delay B settings (See the attached excel sheet as for the more detailed results).

# Conclusions

This contribution has proposed spatial QP prediction method. The proposed method is based on the CE4 Subtest 1.3 and modified to use QP of the preceding QGCU more frequently for further rate control friendliness. The proposed method is composed simple algorithm, and is available tool for HEVC. It is recommended that introducing the proposed method into the QP prediction of HEVC is considered.

# References

[1] “Description of Core Experiment 4: Quantization”,

JCTVC-G1204r3, Geneva, November 2011

[2] “WD5: Working Draft 5of High-Efficiency Video Coding”,

JCTVC-G, Geneva, November 2011

[3] H. Aoki et al, “Non-CE4: Rate control friendly spatial QP prediction”,
   JCTVC-G1028, Geneva, November 2011

[4] “Common test conditions and software reference configurations”,

JCTVC-G1200, Geneva, November 2011

[5] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Working Draft description of JCTVC-H0204 Proposal 2

The Working Draft description of JCTVC-H0204 Proposal 2 modified from Working Draft 5.0 (d9) is described in JCTVC-H0204_WDdescription.doc.

- – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

-

-

-

-

-