| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0208_r1 |
| --- | --- |

| Title: | CE5.a: Quantization for transform skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Matteo Naccari, Andrea Gabriellini, Marta Mrak BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK | Emails: | matteo.naccari@bbc.co.uk, andrea.gabriellini@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

The purpose of this experiment is to evaluate the efficiency of modified quantisation, as required for transform skipping (TSM), according to JCTVC-G575. Proposed modifications are dependent on transform skipping modes and block sizes and the modifications adjust signal levels when the transform is skipped in any direction. Additionally, a test is performed, as defined in CE5 description (JCTVC-G1205) where weighting quantisation matrices on top of HM-5.0 are used together with TSM. The results show that modifications to quantisation are needed for achieving larger coding gains with TSM. Moreover, the integration of TSM with quantisation matrices is achieved.

# Quantisation in transform skipping

When transform skipping is used, it is necessary to perform normalization of the coefficients as defined in JCTVC-G575. The quantization (both forward and inverse) is performed using a modified quantization factor:

uiQ' = (uiQ  bscale) >> bshift, 	(1)

where uiQ' = uiQ for full 2D transform, uiQ is HM-5.0 quantization factor (inverse or forward), and bscale and bshift are dependent on TSM modes and the current block dimensions. The block scale is defined as:

bscale=1,if TS0scaleheight,if TS1scalewidth,if TS2scaleheight⋅scalewidth,if TS3

where scale is defined as in Table 1.

Table 1 Scaling factors for vectors of N elements.

| N | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| scale | 128 | 181 | 256 | 362 |

The block shift bshift corresponds to the shifts of skipped transforms. Therefore, for the inverse quantization in HM-5.0, the blocks shift is defined as:

bshift=0,if TS0SHIFT_INV_1ST,if TS1SHIFT_INV_2ND - g_uiBitIncrement,if TS2SHIFT_INV_1ST+SHIFT_INV_2ND - g_uiBitIncrement,if TS3.

# Transform skipping and quantisation matrices

Quantisation matrices introduced in JCTVC-G880 assume 2D transform of underlying coefficients. On the other hand, when TSM is used only a 1D transforms can be applied, or transforms in both directions can be skipped. In such cases the usage of the current quantization matrices should be carefully analyzed.  In particular the following issues should be addressed:

1) Whether the values for the current quantization matrices still apply to the TSM case.

2) If they can be used, how to select them.

To address the first issue, the Human Visual System (HVS) model considered in JCTVC-G880 should be considered. This model depicts a nonlinear relation between the radial spatial frequency and the HVS sensitivity to the distortion introduced at different frequency bands. The radial frequency is defined as:

fr(i,j)=f(i)2+f(j)2

where f(i) and f(j) denote the horizontal and vertical discrete frequencies in the DCT domain, respectively. The values for each DCT band of these frequencies are given by:

f(i)=i−1Δ×2N, 	f(j)=j−1Δ×2M

where Δ denotes the display dot pitch and N, M denote respectively, the horizontal and vertical block dimensions. The final Quantization Matrix (QM) value for DCT band (i,j) is given as QM(i,j) = λ(fr(i,j)) where λ denotes the nonlinear relation for the chosen HVS model in JCTVC-G880. By applying TSM over a block, turns into applying a 1D transform over the block lines or columns depending on which direction is skipped. In term of the DCT horizontal and vertical frequencies, this leads to have either f(i) or f(j) equal to zero. In this case, QM is given by QM(u,0) or QM(0,v) depending on which direction is skipped. Therefore, the quantization matrices used in the current version of the HM can be still used. Finally, when both the transforms are skipped (i.e. no transform) the quantization takes place in the pixel domain and therefore no quantization matrix is used.

Given this consideration to address point 1) above, point 2) can be now addressed as well. In particular the following selection process for a X  Y block of pixels, where X and Y are block width and block height respectively, is proposed to derive the TSM quantization matrix QMTSM as follows:

QMTSM(i,j)=1if TrC=TrR=0QM(TrC⋅j⋅XY,TrR⋅i⋅YX)otherwise,	(2)

where QM is the quantization matrix for square blocks of size M  M, where M = max(X, Y), and 2D transform of block is assumed, QM(y, x) are elements of QM, TrC and TrR select the first column or the first row of QM, in case the transform is skipped in one direction only, and are defined as follows:

In (2), divisions Y / X and X / Y enable selection of appropriate row / column from QM, in case where the block is not a square block.

The depicted quantization matrix value selection can be also graphically represented as reported in Figure 1 and Figure 2. This design enables full compatibility with HVS model used in JCTVC-G880, from which the default quantisation matrices in HM (assuming 2D transform) are derived. Full compatibility means that reusing given default matrices in here described way leads to the same matrices that would be derived from HVS model in case skipping is used.

Figure 1: Quantization matrix values selection for transform skip on the vertical direction.

Figure 2: Quantization matrix values selection for transform skip on the horizontal direction.

# Tests and results

The first test evaluates efficiency of modified quantisation, when TSM is used, as defined in Section 1 and JCTVC-G575. The efficiency is evaluated by disabling modifications to the quantisation, i.e. by selecting bscale = 1 and bshift = 0 in (1).

The tests are performed using basic TSM software, built on top of HM-5.0, available to CE5 participants.

The results are summarised in Table 2 and Table 3. It can be seen that without changes in quantisation TSM does not provide significant gain.

Table 2 Losses associated with disabling TSM quantisation: 
Comparison of the TSM without related changes in quantisation, with TSM with associated changes in quantisation as the reference (all on top of HM-5.0)

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.1% |  |  |  |
| Class B | 0.3% | 0.0% | -0.1% |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% |  |  |  |
|  | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class B | 0.3% | -0.1% | -0.2% | 0.7% | 0.1% | -0.2% |
| Class C | 0.5% | 0.3% | 0.0% | 1.0% | 0.2% | 0.0% |
| Class D | 0.8% | 0.1% | 0.3% | 1.6% | -0.1% | -0.3% |
| Overall | 0.4% | 0.1% | 0.0% | 0.9% | 0.1% | -0.1% |
|  | 0.4% | 0.1% | 0.1% | 0.9% | 0.1% | -0.1% |
| Class F | 1.8% | 0.8% | 0.9% | 2.7% | 1.5% | 1.5% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 0.4% | -0.3% | -0.1% | 1.0% | 0.7% | 0.4% |
| Class C | 0.8% | 0.1% | 0.2% | 1.6% | 0.2% | -0.2% |
| Class D | 1.3% | -0.4% | -0.5% | 2.2% | -1.1% | -1.7% |
| Class E | 0.2% | -0.8% | -0.2% | 1.5% | -1.4% | -0.6% |
| Overall | 0.7% | -0.3% | -0.2% | 1.5% | -0.3% | -0.5% |
|  | 0.7% | -0.2% | -0.1% | 1.5% | -0.2% | -0.5% |
| Class F | 4.0% | 2.3% | 2.4% | 5.8% | 3.5% | 3.7% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 3 Comparison of the TSM without related changes in quantisation (on top of HM-5.0), with HM-5.0 as the reference

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% |  |  |  |
|  | 0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 132% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class F | -0.3% | -0.2% | -0.1% | -0.3% | -0.1% | -0.2% |
| Enc Time[%] | 132% |  |  | 121% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 0.1% | 0.2% | 0.3% | 0.0% | -0.1% | 0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.2% |
| Class D | 0.1% | -0.1% | -0.1% | 0.0% | 0.4% | 0.0% |
| Class E | 0.1% | -0.2% | 0.1% | 0.1% | -0.2% | 0.1% |
| Overall | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% |
|  | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class F | -0.7% | -0.9% | -0.9% | -0.8% | -1.0% | -0.6% |
| Enc Time[%] | 126% |  |  | 116% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

The second test enables usage of TSM with quantisation matrices. Therefore, TSM was implemented on top of the first version of HM-5.0 branch that supports quantisation matrices (HM-5.0-dev-qmatrix). The test was performed using the default quantisation matrices.

In order to demonstrate working integration of TSM and quantisation matrices, tests were carried out on all mandatory test point and BD-rate was computed. However, since BD-rate is not appropriate measure for HVS-related tools the actual values reported here should not be considered correlated to actual visual quality.

Related results are reported in Tables 4-6. In addition to the comparison of TSM integrated with quantisation matrices and the basic software that supports quantisation matrices (HM-5.0-dev-qmatrix), both the tested and the reference software are compared to HM-5.0, for further information.

Table 4 Comparison of the TSM harmonised with quantisation matrices, with HM-5.0-dev-qmatrix as the reference

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.1% | -0.5% | -0.3% |  |  |  |
| Class B | -0.2% | 0.9% | 1.6% |  |  |  |
| Overall | -0.1% | 0.3% | 0.7% |  |  |  |
|  | -0.1% | 0.3% | 0.7% |  |  |  |
| Enc Time[%] | 136% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | -0.2% | -0.1% | 0.0% | -0.8% | -0.6% |
| Class B | -0.2% | 0.8% | 1.5% | -0.8% | 0.3% | 1.6% |
| Class C | -1.1% | 1.1% | 1.6% | -1.9% | 0.9% | 2.0% |
| Class D | -1.4% | 2.5% | 2.5% | -2.3% | 2.6% | 2.8% |
| Overall | -0.7% | 1.2% | 1.6% | -1.4% | 0.9% | 1.7% |
|  | -0.7% | 1.1% | 1.5% | -1.4% | 0.8% | 1.6% |
| Class F | -1.0% | 0.2% | 0.0% | -1.6% | -0.1% | -1.0% |
| Enc Time[%] | 138% |  |  | 126% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | -0.3% | 2.3% | 3.0% | -1.0% | 2.4% | 3.9% |
| Class C | -1.1% | 2.6% | 3.1% | -2.7% | 3.0% | 4.4% |
| Class D | -1.5% | 4.7% | 4.5% | -2.9% | 6.2% | 7.5% |
| Class E | -0.2% | 1.8% | 0.5% | -1.6% | 3.4% | 2.5% |
| Overall | -0.8% | 2.9% | 2.9% | -2.0% | 3.7% | 4.6% |
|  | -0.8% | 2.8% | 2.9% | -2.1% | 3.5% | 4.5% |
| Class F | -2.9% | 0.1% | -0.4% | -5.8% | -2.3% | -3.2% |
| Enc Time[%] | 130% |  |  | 118% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 5 Comparison of the TSM harmonised with quantisation matrices, with HM-5.0 as the reference

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 2.0% | 1.6% | 3.5% |  |  |  |
| Class B | 3.5% | -2.3% | -2.6% |  |  |  |
| Overall | 2.8% | -0.5% | 0.1% |  |  |  |
|  | 2.8% | -0.5% | 0.1% |  |  |  |
| Enc Time[%] | 133% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 3.7% | 2.3% | 0.4% | 6.1% | -15.9% | -22.7% |
| Class B | 3.4% | -1.9% | -2.6% | 4.9% | -17.2% | -20.6% |
| Class C | 6.8% | 3.3% | 2.3% | 9.1% | -5.8% | -8.4% |
| Class D | 6.7% | 0.5% | 0.4% | 8.3% | -10.3% | -11.2% |
| Overall | 5.2% | 0.7% | -0.1% | 7.1% | -12.1% | -15.1% |
|  | 5.2% | 0.6% | 0.0% | 7.1% | -12.0% | -15.0% |
| Class F | 9.5% | 7.9% | 9.5% | 14.0% | 7.3% | 10.1% |
| Enc Time[%] | 135% |  |  | 125% |  |  |
| Dec Time[%] | 107% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 1.9% | 1.0% | 1.1% | 2.6% | -10.0% | -12.3% |
| Class C | 3.3% | 3.4% | 3.2% | 5.5% | -3.1% | -4.5% |
| Class D | 2.4% | 2.7% | 2.1% | 3.7% | -7.0% | -7.1% |
| Class E | 1.0% | -0.2% | -1.0% | 1.7% | -25.8% | -25.6% |
| Overall | 2.2% | 1.8% | 1.5% | 3.4% | -10.5% | -11.5% |
|  | 2.1% | 1.8% | 1.6% | 3.4% | -10.3% | -11.2% |
| Class F | 3.7% | 4.3% | 6.1% | 6.1% | 1.5% | 3.1% |
| Enc Time[%] | 128% |  |  | 117% |  |  |
| Dec Time[%] | 108% |  |  | 106% |  |  |

Table 6 Comparison of HM-5.0-dev-qmatrix, with HM-5.0 as the reference

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 1.9% | 2.1% | 3.8% |  |  |  |
| Class B | 3.7% | -3.0% | -4.0% |  |  |  |
| Overall | 2.9% | -0.7% | -0.5% |  |  |  |
|  | 2.9% | -0.8% | -0.4% |  |  |  |
| Enc Time[%] | 98% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 3.5% | 2.5% | 0.4% | 6.1% | -15.5% | -22.4% |
| Class B | 3.6% | -2.6% | -3.9% | 5.8% | -17.6% | -21.3% |
| Class C | 8.0% | 2.1% | 0.7% | 11.2% | -6.9% | -10.3% |
| Class D | 8.3% | -1.9% | -2.0% | 11.0% | -12.8% | -13.5% |
| Overall | 6.0% | -0.5% | -1.6% | 8.7% | -13.2% | -16.5% |
|  | 6.0% | -0.4% | -1.5% | 8.7% | -13.0% | -16.2% |
| Class F | 10.6% | 7.6% | 9.7% | 16.0% | 7.1% | 11.4% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 2.1% | -1.1% | -1.7% | 3.5% | -12.0% | -14.9% |
| Class C | 4.5% | 0.8% | 0.2% | 8.4% | -5.9% | -8.4% |
| Class D | 3.9% | -1.7% | -2.1% | 6.8% | -12.1% | -12.9% |
| Class E | 1.2% | -2.0% | -1.5% | 3.4% | -28.0% | -27.2% |
| Overall | 3.0% | -0.9% | -1.3% | 5.5% | -13.5% | -15.1% |
|  | 2.9% | -0.9% | -1.2% | 5.5% | -13.2% | -14.7% |
| Class F | 6.8% | 4.3% | 6.7% | 12.9% | 3.4% | 6.6% |
| Enc Time[%] | 98% |  |  | 99% |  |  |

# Summary

The purpose of this test was two-fold - evaluation of modified quantisation to support transform skipping and harmonisation of transform skipping with quantisation matrices. A straight-forward solution for reusing quantisation matrices in transform skipping is devised. Moreover, it has been shown that modified quantisation in cases where transform is skipped is essential for obtaining significant coding gains.

# Patent rights declaration

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).