| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0211_r1 |
| --- | --- |

| Title: | CE5.g: Integration of tools CE5.a to CE5.e |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Mrak, Andrea Gabriellini, Matteo Naccari, David Flynn (BBC R&D) Glenn Van Wallendael (Ghent University) Rajan Joshi (Qualcomm) |  | marta.mrak@bbc.co.uk glenn.vanwallendael@ugent.be rajanj@qualcomm.com |
| Source: | British Broadcasting Corporation, Ghent University, Qualcomm |  |  |

_____________________________

# Abstract

This report presents results for integration of tools related to Transform Skip Mode (TSM) that is applied to motion compensated residuals. Related tools are individually evaluated in CE5 test (CE5.a to CE5.e). TSM tools were integrated in HM-5.0. The performance was evaluated for different RQT levels.

# Integration of tools supporting transform skipping for motion compensated blocks

Transform Skip Mode (TSM) with additional tools was proposed for application on motion compensated blocks in JCTVC-G1003. Table 1 summarises the tools used in integrated TSM version, based on HM-5.0.

Table 1 Tools and experiments related to integrated TSM

| Tool | Description | Status |
| --- | --- | --- |
| HM-5.0_tsm | Basic implementation of TSM in HM-5.0 used as a starting point for development in CE5 | Basic source code used for integration of TSM tools |
| CE5.a | Changes to quantisation related to TSM | TSM modifications to quantisation are used in the integrated version; only common test conditions are considered in this test, so quantisation matrices were not used |
| CE5.b | Transform skipping choices based on block parameters | Integrated software was evaluated for all block parameters defined by common conditions |
| CE5.c | Mode dependant significance scans for transform skip | This tool is used in integrated TSM. |
| CE5.d | Coefficients coding for transform skipping | This tool is used in integrated TSM |
| CE5.e | Parallelizable context for significance coding of large transform blocks | This proposal is withdrawn. |
| Bug fix | Scaling related to TSM on NSQT blocks. | A bug in HM-5.0_tsm was detected and related bug fix is included in the integrated version. The bug fix is documented in this contribution. |

# Correction of the basic TSM software

The basic TSM software was released to CE5 with tag HM-5.0_tsm. The tools studied in tests CE5.a to CE5.d were based on that software. Before integration of tools for CE5.g a bug was detected and its fix (Table 1) was included in the integrated TSM software. The bug is related to some transform skip modes (when any transform is skipped), and only for NSQT blocks. Results from Table 2 show that the bug fix brings small gains of up to 0.1%. It does not influence the low complexity settings.

Table 1 Bug fix summary

------------------- source/Lib/TLibCommon/TComTrQuant.cpp -------------------- index 3a484f5..e1a2195 100644 @@ -2829,15 +2829,6 @@ Void TComTrQuant::xDeQuant( const TCoeff* pSrc, Int* pDes, Int iWidth, Int iHeig const TCoeff* piQCoef = pSrc; Int* piCoef = pDes; -#if NSQT - if( iWidth != iHeight ) - { - UInt uiWidthBit = g_aucConvertToBit[ iWidth ] + 2; - UInt uiHeightBit = g_aucConvertToBit[ iHeight ] + 2; - iWidth = 1 << ( ( uiWidthBit + uiHeightBit) >> 1 ); - iHeight = iWidth; - } -#endif #if TSM unsigned int tsmScale = 1, tsmShift = 0; if (!hasVerTx (code)) @@ -2851,6 +2842,15 @@ Void TComTrQuant::xDeQuant( const TCoeff* pSrc, Int* pDes, Int iWidth, Int iHeig tsmShift += SHIFT_INV_2ND - g_uiBitIncrement; } #endif +#if NSQT + if( iWidth != iHeight ) + { + UInt uiWidthBit = g_aucConvertToBit[ iWidth ] + 2; + UInt uiHeightBit = g_aucConvertToBit[ iHeight ] + 2; + iWidth = 1 << ( ( uiWidthBit + uiHeightBit) >> 1 ); + iHeight = iWidth; + } +#endif if ( iWidth > (Int)m_uiMaxTrSize ) { iWidth = m_uiMaxTrSize;

Table 2 Comparison of integrated TSM version:
 tested version includes the bug fix and the reference is integrated version without the bug fix.

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class B | 0.0% | 0.0% | -0.2% |
| Class C | -0.1% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | -0.7% |
| Class E | 0.0% | -0.4% | -0.2% |
| Overall | 0.0% | -0.1% | -0.3% |
|  | 0.0% | -0.1% | -0.3% |
| Class F | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 105% |  |  |

# Experimental results for integration of TSM tools

Results reported in this section aims to evaluate the performance of integrated TSM tools ("TSM" in Tables 3-7). All tests used common test conditions, except the maximum depth for RQT on inter predicted blocks which is varied (taking values of 1, 2 and 3, where 3 is the default value in HM-5.0). The results show a varying level of additional complexity, due to the RDO search of more transform options enabled with deeper RQTs.

Table 3 Comparison of the TSM under common conditions, relatively to the HM-5.0

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.1% | -0.2% | 0.0% |  |  |  |
| Class B | -0.2% | -0.2% | -0.1% |  |  |  |
| Overall | -0.1% | -0.2% | -0.1% |  |  |  |
|  | -0.1% | -0.2% | -0.1% |  |  |  |
| Enc Time[%] | 128% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class B | -0.3% | -0.2% | -0.1% | -0.7% | -0.3% | 0.0% |
| Class C | -0.5% | -0.3% | -0.2% | -1.0% | -0.4% | 0.0% |
| Class D | -0.8% | -0.3% | -0.2% | -1.5% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.3% | -0.2% | -0.9% | -0.3% | 0.0% |
|  | -0.4% | -0.3% | -0.2% | -0.9% | -0.3% | 0.0% |
| Class F | -2.1% | -1.1% | -1.1% | -2.9% | -1.7% | -1.8% |
| Enc Time[%] | 130% |  |  | 118% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.3% | 0.3% | -1.1% | -0.9% | -0.1% |
| Class C | -0.7% | -0.3% | -0.1% | -1.6% | -0.6% | 0.3% |
| Class D | -1.2% | 0.4% | 0.4% | -2.2% | 1.0% | 0.9% |
| Class E | -0.3% | 0.3% | 0.6% | -1.5% | 1.5% | 1.0% |
| Overall | -0.6% | 0.2% | 0.3% | -1.6% | 0.1% | 0.5% |
|  | -0.6% | 0.1% | 0.2% | -1.5% | 0.0% | 0.4% |
| Class F | -4.7% | -3.0% | -3.3% | -6.6% | -5.3% | -4.6% |
| Enc Time[%] | 124% |  |  | 114% |  |  |
| Dec Time[%] | 101% |  |  | 96% |  |  |

Table 4 Comparison of the TSM with one level of RQT splitting for inter coded blocks 
(--QuadtreeTUMaxDepthInter=2), relatively to the HM-5.0

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.2% | 0.1% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% |  |  |  |
|  | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 113% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | -0.1% | -0.1% | 0.1% | -0.2% | 0.0% |
| Class B | 0.0% | 0.2% | 0.3% | -0.5% | -0.2% | 0.1% |
| Class C | -0.3% | -0.2% | 0.0% | -0.9% | -0.3% | 0.1% |
| Class D | -0.7% | 0.0% | -0.2% | -1.4% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | 0.0% | 0.0% | -0.8% | -0.1% | 0.1% |
|  | -0.3% | 0.0% | 0.0% | -0.8% | -0.2% | 0.0% |
| Class F | -2.0% | -1.0% | -1.1% | -2.8% | -1.5% | -1.7% |
| Enc Time[%] | 113% |  |  | 109% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 1.5% | 1.5% | -0.8% | -0.5% | -0.2% |
| Class C | -0.5% | 0.2% | 0.5% | -1.5% | -0.3% | 0.1% |
| Class D | -1.0% | 1.1% | 1.3% | -2.1% | 1.3% | 1.5% |
| Class E | 0.0% | 2.6% | 1.9% | -1.3% | 1.7% | 0.9% |
| Overall | -0.4% | 1.3% | 1.3% | -1.4% | 0.4% | 0.5% |
|  | -0.4% | 1.2% | 1.2% | -1.4% | 0.4% | 0.5% |
| Class F | -4.5% | -2.7% | -2.9% | -6.4% | -4.7% | -4.8% |
| Enc Time[%] | 111% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 5 Comparison of the TSM with one level of RQT splitting for inter coded blocks 
(--QuadtreeTUMaxDepthInter=2), relatively to the HM-5.0 with the same RQT setting

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.0% | -0.2% | -0.2% |  |  |  |
| Class B | -0.3% | -0.2% | 0.0% |  |  |  |
| Overall | -0.2% | -0.2% | -0.1% |  |  |  |
|  | -0.2% | -0.2% | -0.1% |  |  |  |
| Enc Time[%] | 121% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.3% | -0.2% | 0.0% | -0.3% | -0.2% |
| Class B | -0.3% | -0.1% | 0.0% | -0.7% | -0.4% | 0.0% |
| Class C | -0.5% | -0.3% | -0.2% | -1.1% | -0.4% | 0.0% |
| Class D | -0.9% | -0.1% | -0.2% | -1.6% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.2% | -0.1% | -1.0% | -0.3% | 0.0% |
|  | -0.4% | -0.2% | -0.1% | -1.0% | -0.3% | 0.0% |
| Class F | -2.2% | -1.3% | -1.3% | -2.9% | -1.7% | -1.8% |
| Enc Time[%] | 122% |  |  | 115% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.5% | 0.4% | -1.1% | -0.9% | -0.4% |
| Class C | -0.8% | -0.2% | 0.1% | -1.7% | -0.7% | 0.1% |
| Class D | -1.3% | 0.7% | 0.8% | -2.2% | 0.6% | 1.4% |
| Class E | -0.4% | 1.6% | 0.9% | -1.6% | 1.7% | 0.7% |
| Overall | -0.7% | 0.6% | 0.5% | -1.6% | 0.0% | 0.4% |
|  | -0.7% | 0.5% | 0.4% | -1.6% | -0.1% | 0.4% |
| Class F | -4.8% | -3.0% | -2.8% | -6.4% | -4.0% | -4.4% |
| Enc Time[%] | 118% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 6 Comparison of the TSM without RQT splitting for inter coded blocks 
(--QuadtreeTUMaxDepthInter=1), relatively to the HM-5.0

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.6% | 0.7% | 0.6% |  |  |  |
| Class B | 0.5% | 0.7% | 0.7% |  |  |  |
| Overall | 0.6% | 0.7% | 0.7% |  |  |  |
|  | 0.6% | 0.7% | 0.7% |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.7% | 0.3% | 0.2% | 0.5% | -0.2% | -0.1% |
| Class B | 0.5% | 0.7% | 0.8% | 0.1% | 0.0% | 0.2% |
| Class C | 0.2% | 0.2% | 0.5% | -0.3% | 0.0% | 0.4% |
| Class D | -0.1% | 0.1% | 0.1% | -0.8% | 0.3% | 0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.4% | 0.5% | -0.2% | 0.1% | 0.2% |
|  | 0.3% | 0.3% | 0.4% | -0.2% | 0.0% | 0.2% |
| Class F | -1.7% | -0.8% | -0.9% | -2.5% | -1.5% | -1.6% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.6% | 3.2% | 3.4% | -0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 1.1% | 1.4% | -0.9% | 0.1% | 0.5% |
| Class D | -0.4% | 1.7% | 2.0% | -1.4% | 1.3% | 1.8% |
| Class E | 0.4% | 2.9% | 2.3% | -1.0% | 1.3% | 0.6% |
| Overall | 0.2% | 2.2% | 2.3% | -0.8% | 0.6% | 0.7% |
|  | 0.2% | 2.1% | 2.2% | -0.8% | 0.6% | 0.6% |
| Class F | -3.9% | -2.3% | -2.6% | -5.7% | -4.5% | -4.5% |
| Enc Time[%] | 96% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |

Table 7 Comparison of the TSM without RQT splitting for inter coded blocks 
(--QuadtreeTUMaxDepthInter=1), relatively to the HM-5.0 with the same RQT setting

|  | Random Access HE-10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |  |
| Class A | 0.0% | -0.2% | -0.2% |  |  |  |
| Class B | -0.3% | -0.2% | 0.0% |  |  |  |
| Overall | -0.2% | -0.2% | -0.1% |  |  |  |
|  | -0.2% | -0.2% | -0.1% |  |  |  |
| Enc Time[%] | 113% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.2% |
| Class B | -0.3% | -0.2% | 0.0% | -0.8% | -0.3% | 0.1% |
| Class C | -0.6% | -0.3% | -0.1% | -1.1% | -0.4% | 0.0% |
| Class D | -0.9% | -0.2% | -0.2% | -1.6% | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -0.2% | -0.1% | -1.0% | -0.2% | 0.1% |
|  | -0.5% | -0.2% | -0.1% | -1.0% | -0.2% | 0.0% |
| Class F | -2.2% | -1.2% | -1.3% | -2.9% | -1.8% | -1.9% |
| Enc Time[%] | 113% |  |  | 109% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | 0.5% | 0.7% | -1.1% | -0.6% | -0.4% |
| Class C | -0.9% | 0.1% | 0.4% | -1.7% | -0.6% | 0.0% |
| Class D | -1.5% | 0.3% | 1.0% | -2.3% | 0.4% | 1.6% |
| Class E | -0.3% | 1.2% | 0.5% | -1.7% | 1.5% | 0.9% |
| Overall | -0.8% | 0.5% | 0.6% | -1.7% | 0.0% | 0.5% |
|  | -0.8% | 0.4% | 0.6% | -1.7% | 0.1% | 0.4% |
| Class F | -4.7% | -3.1% | -3.1% | -6.5% | -5.3% | -4.5% |
| Enc Time[%] | 108% |  |  | 105% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  |

# Patent rights declarations

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ghent University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).