| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0215 |
| --- | --- |

| Title: | Market needs and practicality of sub-picture based CPB operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Satoshi Shimada, Junpei Koyama and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN Alberto Duenas 2315 N. First Street, San Jose, CA 95131 USA | Tel: Email: Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com sshimada@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com +1-408-943-7150 alberto@cavium.com |
| Source: | FUJITSU LABORATORIES LTD. and CAVIUM INC |  |  |

_____________________________

# Abstract

Sub-picture based operation of CPB (Coded Picture Buffer) for interoperable ultra low delay video transmission was proposed at the 7th JCT-VC meeting.

This contribution provides information which confirms the needs and the practicality of the proposed operation to the current HRD specification.

# Introduction

JCTVC-G188[1] at the 7th JCT-VC meeting proposed sub-picture based operation of CPB for interoperable ultra low delay video transmission. Specifically, removal time of a sub-picture (e.g. a Treeblock line) is newly introduced in the HRD specification.

The idea behind the proposal is that there are ways to average the number of bits of each sub-picture of a picture. One example is the vertical intra line refresh scheme explained in JCTVC-G183. When the number of bits of each sub-picture is controlled in such a way, an encoder encodes a parameter M in the bitstream which implies arrival times of the last bit of a M sub-pictures are uniformly distributed between removal times of two consecutive pictures. The arrival time of i-th sub-picture in the n-th picture, tr( n, i ) is defined as

tr( n, i ) = tr ( n - 1 ) + ( tr ( n ) - tr ( n - 1 ) ) * i / M

, where tr( n ) is the removal time of the n-th picture.

An decoder can start the decoding process of a sub-picture at the arrival time tr( n, i ), before tr( n ), since it is ensured that all bits required to decode the sub-picture are already arrived at the decoder bitstream buffer. As the result, the decoder can finish the decoding of whole picture {(M-1)*picture_period/M} [sec] earlier than a decoder conforming to the existing HRD specification. In other word, the codec delay can be reduced less than one picture period.

The result of the discussion was "further study encouraged" due to the lack of study on this subject by the group in spite of interest from a number of participants.

In order to discuss further on this subject, this contribution provides information regarding the market needs and practicality of an ultra low delay video transmission system. It is noted that the specification of the proposed CPB operation remains unchanged from JCTVC-G188.

# Needs and practicality of ultra low delay video transmission system

## Presence of AVC-based ultra-low delay codec in the market

The following table shows the examples of AVC[4] based ultra-low delay codec in the market. Apparently there are strong needs for ultra-low delay video transmission. It is important to mention that sub-frame end to end latency cannot be achieved when following the method described on the AVC hypothetical reference decoder in Annex C.

Table 1  Example of AVC-conforming ultra low delay codec in the market

| Vendor /Product name | Latency | URL |
| --- | --- | --- |
| CAVIUM CNW5302 | 2 ms | http://www.cavium.com/PureVu-3D_CNW5302.html |
| NEC VC-7710/VD-7710 | 10 ms | http://www.nec.com/global/prod/nw/broadcast/pdf/5codec/VC7710-VD7710_e_r34.pdf |
| Hitachi HU-200EI/DI | 8-10 ms | http://www.hitachi-keu.com/content/broadcast/hdtv_broadcast_production_cameras/accessories_hu_200_ei_di_e.htm |
| IBEX HLD-3000 | 10 ms | http://www.lsisys.co.jp/en/p0000.html |
| HHI | 3 ms | http://www.hhi.fraunhofer.de/en/departments/image-processing/applications/ultra-low-latency-video-codec/ |

## Ultra low delay video monitor

One doubt regarding the presence of an ultra low delay monitor was expressed at the discussion. That means the ultra low delay video transmission system might not be practical because of large latency at a monitor side if the monitor has a frame buffer and stores a picture in more than one frame period for post-processing or other purposes.

To answer the doubt, some participants informed that an ultra low delay monitor is available in the market.

Actually, in the LCD monitor market, there are many monitors from several display vendors which have so-called "through mode" (or "instant mode" or "game mode") for gaming users. Computer game enthusiasts require ultra low delay (ultimately zero) between the operation of a controller and the display of the result. The "through mode" provides low latency less than 1 frame period by skipping the storage of a picture in a frame buffer. One example of such monitor is Toshiba 32ZP2 (http://www.toshiba.co.jp/regza/lineup/zp2/index_j.htm (Japanese)) with 3ms delay for 2D video.

# Consideration of CABAC's delay

## Overview

The BAC (Binary Arithmetic Coder) of WD5 [5] (which is derived from AVC) has currently a fundamental issue. This issue is that the worst case latency between the input of one bin and the output of corresponding bit(s) is infinite. This is due to the presence of an outstanding bit in the encoder BAC.

At the 6th meeting, JCTVC-F059[3] was proposed to remove this problem by introducing the mechanism which adjusts a coding interval and flushes all outstanding bits when the number of outstanding bits becomes large. However the proposal was not accepted for further investigation. Below sentences are excerpted from the meeting notes:

One expert said that the same thing was discussed already in AVC and SVC standardization, and there would be other ways to solve it. In fact, the suggested approach would add complexity to the decoder (particularly in software).

No other experts expressed the opinion that this is an important issue.

Actually, this problem affects to practicality of the proposed CPB operation. An encoder might not be able to ensure removal time of each sub-picture because the bit(s) corresponding to the last bin of a sub-picture could potentially be output at the end of the picture containing the sub-picture.

One approach to solve this problem is to insert a slice boundary at end of each sub-picture. This approach may introduce degradation of coding efficiency and it does have some additional implementation limitations.

The authors consider there are additional ways to set the upper bound of the BAC latency by proper control at an encoder side with minimal loss in coding efficiency and small complexity increase compared with the current design. One example is shown in the following section, but other options could be possible to address this issue and led to a low delay entropy codec.

## Example of encoder control for ensuring removal time of a sub-picture

An encoder checks the number of outstanding bits in a BAC (binary arithmetic coder) when arithmetic coding of all bins corresponding to a sub-picture (excluding the last sub-picture in a picture) is finished. If the number of outstanding bits is not equal to zero, bits corresponding to the last bin of the sub-picture remain in the BAC.

Then the encoder checks the number of output bits when the BAC operates arithmetic coding of all bins in the first N treeblocks in the following sub-picture. If the number of output bits is not equal to zero, bits corresponding to the last bin of the sub-picture are already output from the BAC. This is because PubBit() process inside the renormalization process always performs flushing of all outstanding bits after putting one bit.

The entropy encoder inserts an entropy slice boundary between N-th treeblock and (N+1) treeblock in the sub-picture only if the number of output bits is equal to zero.

The encoder delays the removal time of each sub-picture N-treeblock interval, regardless the occurrence of entropy slice insertion.

This scheme has two merits. One merit is minimal loss in coding efficiency. This is because the frequency of entropy slice insertion becomes lower by setting proper value of N. This is based on the fact that the probability of flushing outstanding bits becomes large as the number of following bins becomes large. The usage of an entropy slice instead of a normal slice also contributes to minimal loss.

Another merit is that entropy slice insertion is decoupled with pixel reconstruction. This means a CABAC engine can inserts an entropy slice boundary in parallel with source coding.

# Conclusion

Described information shows that there is a need in the market for an ultra-low delay video transmission system. Moreover, it is proven that there are ways to set the upper bound of the BAC latency by proper control at an encoder side with minimal loss in coding efficiency.

We propose adoption of the enhancement of the CPB operation in JCTVC-G188 in the next version of the HEVC WD and HM.

# References

[1] K.Kazui et al., "Enhancement on operation of coded picture buffer", JCTVC-G188, November 2011, Geneva Switzerland

[2] K.Kazui, J.Koyama, and A.Nakagawa, "Industry needs of very low delay coding", JCTVC-F147, July 2011, Torino Italy

[3] T.-D.Chuang et al., "CABAC with Constrained Outstanding Bits", JCTVC-F059, July 2011, Torino Italy

[4] Thomas Wiegand and Gary Sullivan, “Draft ITU-T Recommendation and Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)”, JVT-G050, March 2003 Pattaya

[5] “WD5: Working Draft 5 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- G1103, 7th Meeting: Geneva, CH, November 2011.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Cavium Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).