| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0216 WG11 Number: m23091 |
| --- | --- |

| Title: | Non-CE9: Modification of derivation process for MVP candidate in AMVP/Merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, two modified methods regarding the derivation process of MVP candidate were proposed to avoid a mismatch due to the difference of the supported motion vector bit precision between encoder and decoder. (1) Not appending MVP candidate which exceeds the specified ranges after scaling (2) Clipping MVP candidate into the specified ranges after scaling. Experimental results reportedly showed no loss for all test conditions relative to HM5.0.

# Introduction

In MPEG4/AVC, the ranges of motion vector are restricted based on profiles and level in Annex A such as below,

- Vertical motion vector component range for luma motion vectors does not exceed MaxVmvR in units of luma frame samples, where MaxVmvR is specified in Table A–1

- Horizontal motion vector range does not exceed the range of -2048 to 2047.75, inclusive, in units of luma samples

Based on this restriction, a scaled motion vector component range for temporal direct is also restricted with such as below NOTE,

- NOTE4 – mvL0 and mvL1 cannot exceed the ranges specified in Annex A.

In HM5.0, there is no restriction of the ranges for a scaled MVP (Motion Vector Predictor) candidate. Therefore, there are some possibilities that a mismatch happens in the decoded pictures between encoder and decoder due to the difference of the supported motion vector bit precision. For example, if the encoder supports 32 bit precision for motion vector, a scaled MVP candidate, which exceeds 16 bit precision after scaling, is used for the motion compensation directly. However, in that case, if the decoder supports only 16 bit precision for motion vector, the value of scaled MVP candidate may become wrong due to overflow after scaling, and as the result, a mismatch happens in the decoded pictures between encoder and decoder.

To avoid such a mismatch, it should be considered to introduce some restrictions or designs for the scaled MVP candidate. One approach is to add a NOTE such as below like MPEG4/AVC style.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-1)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-2)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-3)

mvLXCol =  Sign( DistScaleFactor * mvLXA ) *( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 )	(8-4)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-5)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-6)

NOTE – mvLXCol cannot exceed the ranges specified in Annex X.

This method does not affect a normative decoding process. However, an encoder has to take care of it by using some special tricks. For example, it would be considered that if the value of a scaled MVP candidate exceeds the specified ranges, an encoder controls to select Intra prediction mode for current PU, even though in the case of Inter prediction mode is effective. For the other trick, it would be considered that an encoder traces a scaled MVP candidate, which exceeds the specified ranges after scaling, during the pruning process for redundant candidate and creating process for the additional candidate such as a combined merging candidate, and it controls not to select the MVP candidate which is related to the scaled MVP candidate. These kinds of methods are too complicated so that there are some possibilities to confuse encoder designers.

# Proposed Method

In this contribution, two normative methods are proposed to avoid a mismatch due to the difference of the supported motion vector bit precision between encoder and decoder.

- Not appending MVP candidate which exceeds the specified ranges

- Clipping MVP candidate into the specified ranges

## Solution 1: Not appending MVP candidate which exceeds the specified ranges

In this method, the value of the MVP candidate is checked after scaling, and if it exceeds the specified ranges, it is not appended into MVP candidate list. This method affects a normative decoding process, however, it is not needed for an encoder to use any tricks for the mode decision to avoid above issue.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-7)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-8)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-9)

mvLXCol =  Sign( DistScaleFactor * mvLXA ) *( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 )	(8-10)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-11)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-12)

  - If mvLXCol exceeds the ranges specified in Annex X, it is not appended into MVP candidate list

## Solution 2: Clipping MVP candidate into the specified ranges

In this method, the value of the MVP candidate is checked after scaling, and if it exceeds the specified ranges, it is clipped into the specified ranges. This method also affects a normative decoding process, however, it is not needed for an encoder to use any tricks for the mode decision to avoid above issue.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-13)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-14)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-15)

mvLXCol =  Sign( DistScaleFactor * mvLXA ) *( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 )	(8-16)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-17)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-18)

  - If mvLXCol exceeds the ranges specified in Annex X, it is clipped into the ranges.

# Experimental results

Proposed methods were implemented on HM5.0 software and their performance relative to the HM5.0 software were evaluated under the common test condition described in JCTVC-G1200 [1]. In this evaluation, the bit precision of the motion vector was restricted to 16 bit, which can sufficiently cover the motion vector range for 8Kx4K images.

Table 1, 2 show RD performance and execution time of the solution 1 and solution 2 relative to HM5.0. In both cases, proposed methods show no loss on average in all test conditions. The difference in the encoding and decoding time is not obvious. In HM5.0, it is considered that the motion vector, which exceeds 16 bit precision, is not used so that there is no difference in the both methods

Table 1: Experimental result of solution 1

Table 2: Experimental result of solution 2

# Conclusion

In this contribution, two modified methods regarding the derivation process of MVP candidate are proposed to avoid a mismatch due to the difference of the supported motion vector bit precision between encoder and decoder. Experimental results show that proposed methods show no loss for all test conditions relative to HM5.0. In order to avoid such a mismatch, we suggest one of the proposed methods be adopted in the HM/CD of HEVC.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).