| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0217 WG11 Number: m23092 |
| --- | --- |

| Title: | CE9: Cross-check report of BP05 by Panasonic |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result of CE9 BP05 in JCTVC-G1209. The experimental results perfectly match with the one provided by NTT DOCOMO in the rate-distortion performance.

- Experimental Condition

Table 1 shows summary of the test on BP05 in CE9 [1]. The purpose of BP05 is to test the 2nd motion estimation of list 0/1 (depending on picture) for bi-prediction. The software to be tested was provided by NTT DOCOMO and its performance relative to the HM5.0 software was checked under the common test condition described in JCTVC-G1200 [2]. It was confirmed that BP05 method was implemented correctly. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

Table 1: Summary of the test on BP05 in CE9

| Test | Description | Related Doc. |
| --- | --- | --- |
| BP05 | HM5.0 with the 2nd motion estimation for bi-prediction | JCTVC-G305 |

# Results

R-D performance and execution time of each experimental result are summarized in Table 2. Detailed results are included in the attached excel sheet. It was confirmed that these results perfectly match with the one provided by NTT DOCOMO.

Table 2: Experimental result of BP05

# Conclusion

The result of CE9 BP05 was confirmed. An experimental result shows that BP05 saves 0.5-0.7% bits for random access scenario and 0.5-0.8% bits for low delay scenario on average. Encoding time is 18-30% increased relative to HM5.0.

# References

- Il-Koo Kim et al., “Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge Operations”, JCTVC-G1209, Geneva, November 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-G1200, Geneva, November 2011.