| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0222 |
| --- | --- |

| Title: | Introduction of AVC's derivation process of chroma motion vector for interlace format support |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes to introduce the chroma motion vector adjustment scheme of the AVC specification for 4:2:0 interlace format into HEVC. The scheme was originally proposed by multiple organizations at the 7th JCT-VC meeting. This contribution also proposes to include interlaced video related parameters into sequence parameter set and slice header similarly as in the AVC specification.

The benefit of the proposed scheme is the significant improvement of coding efficiency in chroma components without increasing computational complexity. Our preliminary experiment reported in JCTVC-G196 shows 0.4 dB on average, up to 1.0 dB BD-PSNR improvement at random access, high-efficiency configuration.

The authors asserts there is no danger in adopting the proposed scheme because it was well verified through the AVC standardization process.

# Introduction

There were 9 contributions regarding the support of interlace format at the 7th JCT-VC meeting. Especially 4 contributions from 4 different organizations[1][4][5][6] proposed to introduced the chroma motion vector adjustment scheme[7] of the AVC specification.

The proposed scheme is used in the derivation process of a chroma motion vector from a luma motion vector. The chroma motion vector is adjusted according to the parities of the current picture and the reference picture of the luma motion vector when an interlaced video in 4:2:0 chroma format is processed. It was reported in our contribution JCTVC-G196 that 0.4 dB on average, up to 1.0 dB BD-PSNR improvement can be achieved with the proposed scheme at random access, high-efficiency configuration.

A hot discussion on two position contributions[2][3] was held at the plenary session. No review was held for individual contributions. As the result of the discussion, an AHG with the mandate of creating the candidate text for an interlaced format indicator was established.

# Proposal

## Chroma motion vector adjustment

We propose to introduce the chroma motion vector adjustment scheme of the AVC specification for 4:2:0 interlace format into HEVC.

As stated above, the proposed scheme achieves the significant improvement of coding efficiency in chroma components without increasing computational complexity. It is crucial to improve coding efficiency as much as possible also for interlace format in order that HEVC is adopted in the broadcasting industry since there still exists a huge demand for interlace support.

We believe the introduction of the proposed scheme does not delay the schedule of the HEVC standardization work since this proposed scheme is simple and was well verified through the AVC standardization process.

## Interlaced video related parameters

We also propose to include interlaced video related parameters in sequence parameter set and slice header similarly as in the AVC specification.

Since the chroma motion vector adjustment scheme needs a flag which indicates the parity of each picture, interlaced video related parameters shall be specified in somewhere required for constructing the luma or chroma samples by the decoding process.

It is reasonable to borrow AVC's syntax structure rather than defining new container for interlaced video related parameters.

# Suggested WD text

## Sequence parameter set RBSP syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| [Ed. (BB): x y padding syntax missing here, present in HM software ] |  |
| chroma_format_idc | ue(v) |
| progressive_sequence | u(1) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| [Ed. (BB): chroma bit depth present in HM software but not used further ] |  |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| max_num_ref_frames | ue(v) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if ( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if ( !uniform_spacing_flag ) { |  |
| for ( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for ( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause Error: Reference source not found. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive. When chroma_format_idc is not present, it shall be inferred to be equal to 1 (4:2:0 chroma format).

progressive_frame indicates the scanning method of an input source. progressive_frame shall be equal to 1 when an input source is a progressive video. progressive_frame shall be equal to 0 when an input source is an interlaced video. When progressive_frame is equal to 0, each field picture in an interlaced video is treated as a progressive frame with halved picture height and doubled frame rate of the interlaced video. When progressive_frame is not present, it shall be inferred to be equal to 1.

## Slice header syntax and semantics

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| if( !progressive_frame) { |  |
| bottom_field_flag | u(1) |
| } |  |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| [Ed. (BB): coded as absolute value slice_qp in HM software, should be implemented together with pic_init_qp_minus26 ] |  |
| inherit_dbl_params_from_APS_flag | u(1) |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| [Ed. (BB): Although it was agreed to use AVC-like deblocking control syntax, WD and HM should be changed to make use of the AVC-like syntax.] |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| byte_align( ) |  |
| alf_cu_control_param( ) |  |
| byte_align( ) |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| 5_minus_max_num_merge_cand | ue(v) |
| for( i = 0; i < num_substreams_minus1 + 1; i++ ){ |  |
| substream_length_mode | u(2) |
| substream_length[i] | u(v) |
| } |  |
| } |  |

bottom_field_flag equal to 1 specifies that the slice is part of a picture whose parity is equal to "bottom field". bottom_field_flag equal to 0 specifies that the slice is part of a picture whose parity is equal to "top field". When this syntax element is not present for the current slice, it shall be inferred to be equal to 0.

## Derivation process for chroma motion vector

[Subclause 8.4.2.1.10 is replaced with the following]

This subclause is invoked when chroma_format_idc is not equal to 0.

Inputs to this process are a luma motion vector mvLX and a reference index refIdLX.

Output of this process is a chroma motion vector mvCLX.

A chroma motion vector is derived from the corresponding luma motion vector.

For the derivation of the chroma motion vector mvCLX, the following applies.

–	If chroma_format_idc is not equal to 1 or progressive_frame is equal to 1 , the horizontal and vertical components of the chroma motion vector mvCLX are derived as

mvCLX[ 0 ] = mvLX[ 0 ]	(8-1)

mvCLX[ 1 ] = mvLX[ 1 ]	(8-2)

–	Otherwise (chroma_format_idc is equal to 1 and progressive_frame is equal to 0), only the horizontal component of the chroma motion vector mvCLX[ 0 ] is derived using Equation 8-153. The vertical component of the chroma motion vector mvCLX[ 1 ] is dependent on bottom_field_flag of the current picture and the reference picture, which is referred by the reference index refIdxLX. mvCLX[ 1 ] is derived from mvLX[ 1 ] according to Table 8-X.

- Table 8-X – Derivation of the vertical component of the chroma vector when progressive_frame is equal to 1

| Parity conditions |  | mvCLX[ 1 ] |
| --- | --- | --- |
| Reference picture (refIdxLX) | Current picture |  |
| Top field | Bottom field | mvLX[ 1 ] + 2 |
| Bottom field | Top field | mvLX[ 1 ] − 2 |
| Otherwise |  | mvLX[ 1 ] |

# Conclusion

We recommend to introduce the chroma motion vector adjustment scheme of the AVC specification into HEVC. We also recommend to include interlaced video related parameters into sequence parameter set and slice header.

# References

[1] J.Koyama et al., "Modification of derivation process of motion vector information for interlace format", JCTVC-G196, November 2011, Geneva Switzerland

[2] K.Chono et al., "Request to establish AHG on interlaced format support in HEVC standard", JCTVC-G912, November 2011, Geneva Switzerland

[3] D.Singer, "Interlace profiling in HEVC", JCTVC-G962, November 2011, Geneva Switzerland

[4] K.Chono et al., "On issues for interlaced format support in HEVC standard", JCTVC-G170, November 2011, Geneva Switzerland

[5] V. Jérôme et al., "Interlaced and 4:2:2 color format support in HEVC standard", JCTVC-G877, November 2011, Geneva Switzerland

[6] O.Bar-Nir, "Picture-adaptive Field/Frame Coding: support for legacy video", JCTVC-G296, November 2011, Geneva Switzerland

[7] A.Nakagawa et al., "Definition of chroma motion vector in field prediction between different parities", JVT-E088, October 2002, Geneva Switzerland

# Patent rights declaration

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).