| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0223 |
| --- | --- |

| Title: | CE8.e.3 Modified SAO edge offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Per Wennersten, Rickard Sjöberg Färögatan 6 16480 Stockholm Semih Esenlik, Matthias Narroschke, Thomas Wedi Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com +49 6103 766 – 1306 semih.esenlik@eu.panasonic.com |
| Source: | Ericsson AB, Panasonic Corporation |  |  |

_____________________________

# Abstract

This is a core experiment report on CE8.e.3 (JCTVC-G490) as described in JCTVC-G1208. Compared to JCTVC-G490 the number of edge offsets have been reduced to 8, no additional line buffer compared to HM5.0 is required for LCU-based processing and when 4 offset is selected the same operations as in HM5.0 can be used.  The result indicates that edges are enhanced both subjectively and objectively. The average BDR gain versus HM5.0 is 0.1% for luma and 1.2% for chroma for the common conditions in JCTVC-G1200. When ALF is turned off for high efficiency configurations an average BDR gain of 1% for luma for low delay P configuration is obtained with a decoding time increase of 2%.

# Introduction

In HM5.0 SAO edge offsets can be used in one of 4 directions, e.g. horizontal, vertical, diagonal from top left to bottom right, or diagonal from bottom left to top right. The specific direction is determined by sao_type_idx = 1 .. 4.

In HM5.0 SAO edge offsets are categorized by:

edgeIdx = 2+k=01Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]]) where recPicture is the picture after deblocking, hPos and vPos are defined below.

Table I Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |

The reconstructed picture buffer is modified as:

recSaoPicture[xC+i, yC+j] = recPicture[xC+i, yC+j] + saoValueArray[edgeTable[edgeIdx]] with

i=0..nS-1 and j=0..nS-1, edgeTable[5]={1, 2, 0, 3, 4} and recSaoPicture is the picture after SAO has been applied. saoValueArray contains the edge offsets for the specific direction.

The edges types that HM4.0 SAO edge offsets address are:

edgeIdx = 0

edgeIdx = 1

edgeIdx = 3

edgeIdx = 4

From above one can see that HM4.0 SAO edge offsets do not distinguish between “left” and “right” edges for edgeIdx=1 or edgeIdx=3 and also require that two pixel values are equal to belong to those categories.

# Proposed edge offsets

To better distinguish between different edges the SAO decoding process for edge offsets is modified as follows for all LCU lines except the 4th line for luma component and 2nd line for chroma components above the LCU boundary:

edgeIdx = 20 + Sign(-2*recPicture[xC+i, yC+j]+recPicture[xC+i+hPos[0], yC+j+vPos[0]]+recPicture[xC+i+hPos[1], yC+j+vPos[1]])+

4k=01(3*k+1) Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]])

where hPos and vPos are same as for HEVC as defined in Table I.

For one line, which is the 4th line for luma component and 2nd line for chroma components above the LCU boundary, edge offsets are computed as:

edgeIdx = 20 +

4k=01(3*k+1) Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]])

In comparison with HEVC the new categorization requires one additional sign, 3 additional multiplications and 3 additional additions. The multiplications can be implemented with shifts. When 4 offsets is selected (saoEoGroupFlag is 0) edgeIdx can be computed as in HM5.0.

The reconstructed picture buffer is modified as:

recSaoPicture[xC+i,yC+j]=recPicture[xC+i,yC+j]+ saoValueArray[edgeTable[edgeIdx]] with i=0..nS-1 and j=0..nS-1, edgeTable[41]=

{1, 1, 0, 0, 2, 2, 0, 5, 0, 7, 0, 0, 0, 0, 0, 0, 2, 2, 0, 0, 0, 0, 0, 3, 3, 0, 0, 0, 0, 0, 0, 6, 0, 8, 0, 3, 3, 0, 0, 4, 4}

The proposed categorization switches between 4 edge offsets and 8 edge offsets as indicated by saoEoGroupFlag. In comparison with HEVC saoValueArray can contain 4 additional edge offsets but still less than the maximum which is obtained also with HEVC when 16 band offsets are used. The same categorization is used for luma and chroma components. Example of edges that are supported are shown below:

4 offsets

8 offsets

Prediction for the edge offset of the local maxima is also used.

Syntax:

| sao_offset_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_offset_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } else { |  |
| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | ue(v) |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] != 0 ){ |  |
| if(sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] <5){ |  |
| sao_eo_group_flag[ cIdx ][ saoDepth ][ rx ][ ry ] | u(1) |
| if(sao_eo_group_flag[ cIdx ][ saoDepth ][ rx ][ ry ]) |  |
| NumSaoClass[ sao_type_idx ] = 8 |  |
| else |  |
| NumSaoClass[ sao_type_idx ] = 4 |  |
| } |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | se(v) |
| } |  |
| } |  |
| } |  |

Semantics:

saoEoGroupFlag is set equal to sao_eo_group_flag [ cIdx ][ saoDepth ][ rx ][ ry ] which is defined above.

saoValueArray is set equal to SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ] which is defined below.

An array SaoOffsetVal is specified as

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 0 ] = 0

if(sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ]  < 5){

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0, 2..NumSaoCategory – 1

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1] =

(sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ 3]- sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ 0]) << ( bitDepth – Min( bitDepth, 10 ) )

with i = 3

if(saoEoGroupFlag==0){

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 5]  = 0

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 6]  = 0

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 7]  = 0

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 8]  = 0

}

}else{

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1 ] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0..NumSaoCategory – 1

}

| - sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | - NumSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4/8 | - 1D 0-degree edge |
| - 2 | - 4/8 | - 1D 90-degree edge |
| - 3 | - 4/8 | - 1D 135-degree edge |
| - 4 | - 4/8 | - 1D 45-degree edge |
| - 5 | - 16 | - Central band |
| - 6 | - 16 | - Side band |

# Line Memory Analysis

During the 7th JCT-VC Meeting in Geneva, it was pointed out by the group that the proposal increased line memory requirements for LCU-based coding process. Therefore a slight modification is incorporated in the current version of the proposal in order to avoid the line memory increase.

In HM5.0, Sample Adaptive Offset operation requires 0.25 lines of line memory (for luma and each of chroma components) as it was suggested in the Summary Report of CE8 – Non-deblocking Loop Filtering [2]. The line memory requirement for SAO is smaller than 1 line in HM5.0 since actual sample values do not need to be stored in the memory. Instead partial pixel classification results (given by the sign of the difference of two neighboring pixels) can be stored in the memory.

However the second order gradient operation that is given below increases the line memory requirement to 1 line, since it requires actual sample values to be stored in the line memory.

Sign(-2*recPicture[xC+i, yC+j]+recPicture[xC+i+hPos[0], yC+j+vPos[0]]+recPicture[xC+i+hPos[1], yC+j+vPos[1]])

As a solution, the second order gradient is not computed at the 4th line above the bottom LCU boundary in order to avoid the increase in line memory (The value of the second order gradient is set to 0). This means that for one line in an LCU only 4 different edge offsets are used as in the case of HM5.0. For the rest of the pixels in the LCU either 4 or 8 different edge offsets are used depending on the value of saoEoGroupFlag syntax element.

The line memory requirement of the proposal is therefore reduced back to 0.25 lines as in the case of HM5.0 with the line memory amendment. The proposed line memory reduction method has no impact on the coding gain and visual quality.

# Results

## BDR and encoding/decoding time measurements vs HM5.0

|  | All Intra HE |  |  | Random Access HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0,0% | -0,4% | -0,1% | -0,1% | -1,2% | -0,3% | 0,0% | -1,5% | -1,1% |
| Class B | 0,0% | -0,6% | -0,8% | -0,1% | -1,4% | -1,3% | -0,1% | -1,1% | -1,4% |
| Class C | 0,0% | -0,3% | -0,2% | 0,0% | -0,9% | -0,7% |  |  |  |
| Class D | 0,0% | -0,1% | -0,1% | 0,0% | -0,5% | -1,2% |  |  |  |
| Class E | 0,0% | -0,4% | -0,6% |  |  |  |  |  |  |
| Overall | 0,0% | -0,4% | -0,4% | -0,1% | -1,0% | -1,0% | -0,1% | -1,3% | -1,3% |
|  | 0,0% | -0,4% | -0,4% | -0,1% | -1,0% | -1,0% | 0,0% | -1,3% | -1,3% |
| Class F | -0,1% | -0,6% | -0,6% | -0,2% | -1,1% | -1,1% |  |  |  |
| Enc Time[%] | X% |  |  | X% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 104% |  |  |

|  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,2% | -2,7% | -2,4% | -0,2% | -1,9% | -1,8% |
| Class C | -0,2% | -2,3% | -1,9% | -0,1% | -1,5% | -1,5% |
| Class D | -0,2% | -1,1% | -3,0% | -0,1% | -0,2% | -3,0% |
| Class E | -0,2% | -0,2% | 0,2% | -0,2% | -0,8% | 0,3% |
| Overall | -0,2% | -1,7% | -1,9% | -0,1% | -1,2% | -1,6% |
|  | -0,2% | -1,7% | -1,9% | -0,1% | -1,2% | -1,7% |
| Class F | -0,4% | -1,9% | -1,8% | -0,3% | -1,6% | -1,6% |
| Enc Time[%] | X% |  |  | X% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Encoding time marked with X is unreliable. Decoding time is computed on a single PC.

## BDR and encoding/decoding time measurements vs HM5.0 No ALF

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0,0% | -0,4% | -0,2% | 0,0% | -0,7% | -0,3% |
| Class B | 0,0% | -0,6% | -0,7% | -0,3% | -1,4% | -1,3% |
| Class C | 0,0% | -0,3% | -0,2% | -0,2% | -1,0% | -0,8% |
| Class D | 0,0% | -0,2% | -0,2% | -0,2% | -0,5% | -1,4% |
| Class E | 0,0% | -0,6% | -0,8% |  |  |  |
| Overall | 0,0% | -0,4% | -0,5% | -0,2% | -1,0% | -1,0% |
|  | 0,0% | -0,4% | -0,5% | -0,2% | -1,0% | -1,1% |
| Class F | -0,1% | -0,8% | -0,8% | -0,3% | -1,3% | -1,3% |
| Enc Time[%] | X% |  |  | X% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

|  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,6% | -3,0% | -2,3% | -1,6% | -2,2% | -1,9% |
| Class C | -0,6% | -2,5% | -2,4% | -0,6% | -1,9% | -1,8% |
| Class D | -0,3% | -1,2% | -4,0% | -0,2% | -1,1% | -3,4% |
| Class E | -0,6% | -0,7% | 0,0% | -1,8% | -0,4% | -0,2% |
| Overall | -0,5% | -2,0% | -2,3% | -1,1% | -1,5% | -2,0% |
|  | -0,5% | -2,1% | -2,3% | -1,0% | -1,5% | -1,9% |
| Class F | -0,7% | -1,9% | -1,8% | -0,5% | -2,0% | -1,8% |
| Enc Time[%] | X% |  |  | X% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Encoding time marked with X is unreliable. Decoding time is computed on a single PC.

## BDR and decoding time versus HM-5.0 when both CE8.e.3 has ALF=0 and SAO=1

|  | Y | U | V | YUV | dectime | enctime |
| --- | --- | --- | --- | --- | --- | --- |
| AIHE | 0,0% | -0,4% | -0,5% | -0,1 | 100% | X% |
| AILC | 0,0% | -0,3% | -0,3% | -0,1 | 101% | 100% |
| RAHE | -0,2% | -1,0% | -1,0% | -0,4 | 103% | X% |
| RALC | -0,2% | -1,3% | -1,4% | -0,5 | 103% | 100% |
| LBHE | -0,5% | -2,0% | -2,3% | -0,9 | 103% | X% |
| LBLC | -0,6% | -2,4% | -3,0% | -1,1 | 103% | 100% |
| LPHE | -1,1% | -1,5% | -2,0% | -1,2 | 102% | X% |
| LPLC | -1,0% | -2,0% | -2,5% | -1,3 | 103% | 101% |
| Average | -0,5% | -1,4% | -1,6% | -0,7 | 102% | 100% |

This shows the benefit of using SAO according to CE8.e.3 compared to HM-5.0 when both have ALF=0 and SAO=1. Here it is seen that extending SAO according to CE8.e.3 BDR improvements of (0.5%, 1.4%, 1.6%) is achieved at a decoding time increase of 2% with no change in encoding time.

## BDR and decoding time versus HM-5.0 when CE8.e.3 has SAO=1

|  | Y | U | V | YUV | dectime | enctime |
| --- | --- | --- | --- | --- | --- | --- |
| RAHE10bit | -0,1% | -1,3% | -1,3% | -0,4 | 104% | 100% |
| AIHE | 0,0% | -0,4% | -0,4% | -0,1 | 100% | 119% |
| AILC | -0,7% | -2,8% | -3,4% | -1,3 | 105% | 86% |
| RAHE | -0,1% | -1,0% | -1,0% | -0,3 | 102% | 108% |
| RALC | -1,4% | -5,2% | -5,8% | -2,4 | 106% | 89% |
| LBHE | -0,2% | -1,7% | -1,9% | -0,6 | 102% | 113% |
| LBLC | -2,8% | -10,9% | -12,7% | -5,1 | 106% | 86% |
| LPHE | -0,1% | -1,2% | -1,6% | -0,5 | 102% | 118% |
| LPLC | -8,9% | -13,5% | -14,9% | -10,2 | 108% | 100% |
| Average | -1,8% | -4,6% | -5,2% | -2,6 | 104% | 102% |

This shows the benefit of using SAO according to CE8.e.3 compared to conditions that not uses SAO for LC configurations. Here it is seen that BDR improvements of (1.8%, 4.6%, 5.2%) is achieved at a decoding time increase of 4%.

## Visual inspection

Generally the proposal performs subjectively similar as the anchor. However we can see more areas where the proposal have sharper edges and less coding noise than the anchor than we can see areas where the proposal looks worse than the anchor.

# Conclusions

It has been shown that by adding the possibility to use more specific edge offsets when needed both the subjective and the objective quality can be improved. The average BDR for luma is 0.1% and chroma 1.2% for common conditions in JCTVC-G1200. The most improvements are seen for low delay configurations. If ALF is not used for HE an average BDR reduction of 1% for luma can be obtained for low delay P. The encoding time is similar to HM5.0 and decoding time is increased by 2% compared to HM5.0 for common conditions in JCTVC-G1200.

# References

[1] K.Andersson, P. Wennersten, R. Sjöberg, ”Modified SAO edge offsets”, JCTVC-G490, Switzerland, November 2011

[2] T. Yamakage et al., “CE8: Summary report of Core Experiment on non-deblocking loop filtering”, JCTVC-G038, Switzerland, November 2011

[3] T. Yamakage et al. “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering”, JCTVC-G1208r1, Switzerland, November 2011

[4] F. Bossen, “Common test conditions and software reference configurations” JCTVC-G1200, Switzerland, November 2011

[5] “WD5: Working Draft 5 of High-Efficiency Video Coding”, JCTVC-G1103, Switzerland, November 2011

# Patent rights declaration(s)

Ericsson AB may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).