| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0224 m23097 |
| --- | --- |

| Title: | CE11: Sign Data Hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon CLARE (Orange Labs), Félix HENRY (Orange Labs) Joël JUNG (Orange Labs) Stéphane PATEUX (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange.com felix.henry@orange.com joelb.jung@orange.com stephane.pateux@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution presents CE11 subtest “Sign Data Hiding”, a method to embed sign information into the residue. More specifically, the sign of the first non-nul coefficient of a residual block is not always explicitly written in the bitstream. Instead, at decoding time, the sign is directly inferred from the parity of the sum of quantized coefficients, using a predefined convention. At encoding time, if the sign of the first non-nul coefficient and the parity do not match, one of the quantized coefficients is changed in order to obtain the desired parity. The quantized coefficient changed is the one providing the smallest Rate-Distortion (RD) penalty. To determine it, RD costs computed during the RDOQ stage are used. This alternate way of transmitting the sign of the first non-nul coefficient is dependent on a decoder-side decision function. The proposal yields an average BD-rate compression performance of -0.6% (I), -0.5% (RA), -0.6% (LDB) in High Efficiency configurations with an average encoder runtime of 103% (I), 103% (RA) and 103% (LDB) and decoder runtime of 99% (I), 99% (RA) and 100% (LDB) compared to HM5.0 anchors.

# Introduction

Hiding one syntax element into another part of the signal is a known method of improving the compression performance. In JCTVC-A114 [1], it was proposed to use the parity of the sum of the quantized residual block to convey the information relative to the transform used during encoding (DCT of KLT). In [2], the Most Probable Mode index is hidden into the parity of the sum of the quantized residue. In JCTVC-E428 [3], the index of the transform (DCT or DST) is hidden, this time in the parity of the last non-zero coefficient of the residue.

The present proposal aims at conveying sign information in the parity of the sum of the quantized residual block.

The present proposal was initially proposed during the 7th JCTVC meeting as JCTVC-G271 [5], crosschecked by the BBC in JCTVC-G889 [6].

The present contribution is cross-checked by RIM in contribution document JCTVC-H0480 [7].

The present proposal is part of CE11 on Transform Coefficient Coding, the source code and performance were transmitted by mail to CE11 participants on Jan 5th, 2012.

# Proposed modification of the decoder

In this proposal, the entropy decoding step of the sign of the first non-nul coefficient of the quantized residual block is modified.

First, a decision step is performed in order to determine whether the sign is to be decoded the normal way (current Working Draft of HEVC) or the alternate way. The decision consists in comparing the number of coefficients between the first and last non-nul coefficients in their scanning order with a predefined threshold defined by a syntax element. If this number is lower than the predefined threshold, the sign is decoded the normal way, otherwise it is decoded the alternate way.

Second, when it is decided to decode the sign the alternate way, the sign is inferred from the parity of the sum of the quantized coefficients, following a predefined convention (even corresponds to “+” and odd to “-“).

# Essential modifications of the encoder

The entropy encoding of the sign of the first non-nul coefficient of the quantized residual block is modified.

First, a decision step is performed in order to determine whether the sign is to be encoded the normal way (current Working Draft of HEVC) or the alternate way. The decision consists in comparing the number of coefficients between the first and last non-nul coefficients in their scanning order with a predefined threshold defined by a syntax element. If this number is lower than a predefined threshold, the sign is encoded the normal way, otherwise it is decoded the alternate way. Of course, this process is strictly identical to the one happening at decoding side, to avoid any mismatch.

Second, when it is decided to encode the sign the alternate way, the sign is not encoded using the entropy coder. Instead, the sign is compared to the sum of the quantized coefficients according to the convention. If they do not match, the quantized coefficients need to be modified so that the parity matches the sign. The way these coefficients are modified is left to the implementers of the encoder. A specific encoder is proposed in the next section.

# Specific implementation of the encoder

In this section we describe a suggested way of implementing the encoder. This implementation is the one that has been used to produce the performance presented in this proposal. It is important to note that very different implementations can be designed, with different compression - complexity tradeoffs.

The main specificity of the encoder resides in the way the quantized coefficients of the residue should be modified before encoding, so that the parity of their sum matches the sign of the first non-nul coefficient, when such a modification is needed (this will happen for about 50% of all CUs). In our specific implementation, we chose the following approach:

- We change one of the quantized coefficients by adding or removing 1

- We may only change those quantized coefficient that are located from the first non-nul quantized coefficient to the last non-nul one (inclusive)

- The change may not transform the first non-nul quantized coefficient into zero (there would be a decoder mismatch since the sign inferred from the parity would assigned to another coefficient)

- The change may not transform the last non-nul quantized coefficient into zero if this makes the number of coefficients between the first and last non-nul coefficients smaller than the decision threshold mentioned in previous sections (there would be a decoder mismatch since the decoder would attempt to entropy-decode the sign instead of inferring it from the parity)

Finally, in our implementation of the encoder, we choose the quantized coefficient modification that produces the lowest RD cost. To this end, and in order to reduce the computational complexity, we re-use the RD costs computed during RDOQ stage. However, RDOQ explores only modifications of non-nul coefficients. Consequently, and in order to increase the chances of finding a modification with a low RD cost, we also explored the following modifications that are not performed during the original RDOQ:

- Zero quantized coefficients can be shifted by one (in the direction of the original value of the coefficient)

- The quantized coefficients having an amplitude of one can be shifted by one in both directions

The modification finally applied to the quantized residual block is the one producing the lowest RD cost.

# Decision threshold

A specific threshold is used if the CU is using intra or inter prediction, and if the CU belongs to luma or chroma components. Consequently there are four threshold transmitted in the PPS.

# Performance

## Test conditions

We have integrated Sign Data Hiding in HM5.0.

We have tested our proposal with the following conditions:

•	High efficiency

•	Intra, Random access, Low delay, Low Delay P

•	JCTVC-G1200 [4] test conditions

Moreover, the thresholds used for decisions are the following (Table 1):

Table 1: Decision thresholds for  Sign Data Hiding

| CU type | Intra | Inter |
| --- | --- | --- |
| Luma | 5 | 3 |
| Chroma | 5 | 5 |

For instance, for a CU of Intra type containing luma data, the encoder will encode the sign of the first non-nul coefficient using Cabac if there are strictly less than 5 coefficients between the first non-nul quantized coefficient and the last one (inclusive), otherwise it will embed the sign information in the parity of the sum of the quantized residual coefficients.

## Results

Table 2 presents the performance of the proposed modification compared to anchors using the base configurations. Table 3 shows results for Low QP configuration.

Table 2: performance results using base configurations

Table 3: performance results using low QP

# Advantages

We assert that the proposed approach has several interesting characteristics:

- The gain is consistent across sequences (except sequences with very few residues like Class E)

- The gain is consistent across components (luma & chroma)

- The gain is quite homogenous across the compression range (slightly better at low QP)

- Different encoders can choose their own runtime/compression tradeoff

# Conclusion

In the present proposal, a method to embed sign information in the parity of the sum of the quantized residual coefficients is presented and tested. The proposal yields an average BD-rate compression performance of -0.6% (I), -0.5% (RA), -0.6% (LD) with an average encoder runtime of 103% (I), 103% (RA) and 103% (LD) and decoder runtime of 99% (I), 99% (RA) and 100% (LDB) compared to the HM5.0 anchors. The method is proposed for adoption into HEVC working draft and reference software.

# Bibliography

[1] JCTVC-A114, “Video coding technology proposal by France Telecom, NTT, NTT DoCoMo, Panasonic and Technicolor”, I. Amonou et AL, 1st JCTVC meeting, Dresden, April 2010

[2]	J-M. Thiesse, J. Jung, and M. Antonini, “Data hiding of Intra prediction information in chroma samples for video compression,” Proc. IEEE International Conference on Image Processing, ICIP 2010, pp. 2861-2864, Hong Kong, September 2010.

[3] JCTVC-E428, “Low Complexity Embedding of Information in Transform Coefficients”, R. Cohen, S. Rane, A. Vetro, H. Sun, 5th JCTVC meeting, Geneva, March 2011

[4] JCTVC-G1200, “Common test conditions and software reference configurations” F. Bossen, 7th JCTVC meeting, Geneva, November 2011

[5] JCTVC-F271, “Sign Data Hiding”, G. Clare, F. Henry, J. Jung, 7th JCTVC meeting, Geneva, November 2011

[6] JCTVC-G889 “Crosscheck of JCTVC-G271 on Sign Data Hiding”, A. Gabriellini, M. Mrak (BBC), 7th JCTVC meeting, Geneva, November 2011

[7] JCTVC-H0480 “Cross-check of CE11 Subtest3: Orange Labs’ Sign Data Hiding” J. Wang, X. Yu, D. He, 8th JCTVC meeting, Jan Jose, February 2012

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# ANNEX: WD Text modifications

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_synchro | u(v) |
| sign_data_hiding_flag | u(1) |
| if ( sign_data_hiding ) { |  |
| tsig_intra_luma | u(3) |
| tsig_intra_chroma | u(3) |
| tsig_inter_luma | u(3) |
| tsig_inter_chroma | u(3) |
| } |  |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| [Ed. (BB): not present in HM software, signaled in slice header as absolute value slice_qp, should be implemented to be used for slice_qp_delta] |  |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| [Ed. (BB): not present in HM software, should be implemented ] |  |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| tile_info_present_flag | u(1) |
| if( tile_info_present_flag = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Residual coding syntax

| residual_coding_cabac( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| if (sign_data_hiding && significant_coeff_flag[xC][yC]) { |  |
| dhFirstX = xC |  |
| dhFirstY = yC |  |
| dhFirstIdx = n+offset |  |
| } |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if (sign_data_hiding && significant_coeff_flag[xC][yC]) { |  |
| dhFirstX = xC |  |
| dhFirstY = yC |  |
| dhFirstIdx = n+offset |  |
| } |  |
| } |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if (sign_data_hiding) |  |
| sumAbs += coeff_abs_level_minus3[n]+3 |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| if (sign_data_hiding) |  |
| signHidden = numCoeff-dhFirstIdx+1 >= tsig |  |
| for( n = numCoeff − 1; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if (!sign_data_hiding ¦¦ !signHidden ¦¦ xC != dhFirstX ¦¦ yC != dhFirstY) |  |
| coeff_sign_flag[ n ] |  |
| else |  |
| coeff_sign_flag[n] = sumAbs%2 |  |
| if( coeff_sign_flag[n] ) |  |
| transCoeffLevel[x0][y0][trafoDepth][cIdx][dhFirstIdx] *= -1 |  |
| } |  |
| } |  |

#### Picture parameter set RBSP semantics

sign_data_hiding_flag identifies whether sign data hiding is used or not in the current frame. If the value of sign_data_hiding_flag is zero, the sign of the first non-zero coefficient of a residual block should be decoded. If the value of sign_data_hiding_flag is one, the sign may or may not be decoded, depending on the value of the data hiding decision threshold tsig.

The column position dhFirstX and the row position dhFirstY and the scan order position dhFirstIdx of the last non-zero coefficent in scanning order are derived as follows.

dhFirstX	=  LastSignificantCoeffX
dhFirstY	=  LastSignificantCoeffY
dhFirstIdx	=  numCoeff

The data hiding decision threshold tsig is derived as follows.

tsig = cIdx== 0? 
	(PredMode == MODE_INTRA ? tsig_intra_luma : tsig_inter_luma)
	: (PredMode == MODE_INTRA ? tsig_intra_chroma : tsig_inter_chroma)

tsig_intra_luma specifies the number of non-zero coefficients that should be present in a luma intra residual block for sign data hiding to be performed. If tsig_intra_luma is zero or if the number of non-zero coefficient is strictly smaller than tsig_intra_luma , the sign of the first non-zero coefficient of the residual block should be decoded, otherwise it should not be decoded. The value of tsig_intra_luma shall be in the range of 0 to 7, inclusive.

tsig_intra_chroma specifies the number of non-zero coefficients that should be present in a chroma intra residual block for sign data hiding to be performed. If tsig_intra_chroma is zero or if the number of non-zero coefficient is strictly smaller than tsig_intra_chroma, the sign of the first non-zero coefficient of the residual block should be decoded, otherwise it should not be decoded. The value of tsig_intra_chroma shall be in the range of 0 to 7, inclusive.

tsig_inter_luma specifies the number of non-zero coefficients that should be present in a luma inter residual block for sign data hiding to be performed. If tsig_inter_luma is zero or if the number of non-zero coefficient is strictly smaller than tsig_inter_luma , the sign of the first non-zero coefficient of the residual block should be decoded, otherwise it should not be decoded. The value of tsig_inter_luma shall be in the range of 0 to 7, inclusive.

tsig_inter_chroma specifies the number of non-zero coefficients that should be present in a chroma inter residual block for sign data hiding to be perfomed. If tsig_inter_chroma is zero or if the number of non-zero coefficient is strictly smaller than tsig_inter_chroma, the sign of the first non-zero coefficient of the residual block should be decoded, otherwise it should not be decoded. The value of tsig_inter_chroma shall be in the range of 0 to 7, inclusive.