| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0225r1 |
| --- | --- |

| Title: | CE8.e.5 Combination of CE8.e2 and CE8.e.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Per Wennersten, Rickard Sjöberg Färögatan 6 16480 Stockholm Semih Esenlik, Matthias Narroschke, Thomas Wedi Monzastr. 4c, 63225 Langen, Germany Guillaume Laroche, Tangi Poirier, Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 Cesson-Sevigne, France | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com +49 6103 766 – 1306 semih.esenlik@eu.panasonic.com +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Ericsson AB, Panasonic Corporation, Canon Research Centre France |  |  |

_____________________________

# Abstract

This is a core experiment report on a combination of JCTVC-G490 and JCTVC-G246 as described in JCTVC-G1208. Compared to JCTVC-G490 the number of edge offsets have been reduced to 8, no additional line buffer compared to HM5.0 is required for LCU-based processing and when 4 offset is selected the same operations as in HM5.0 can be used. Compared to JCTVC-G246, the number of Band offsets have been reduced to only 3 classifications. Moreover, the number of offsets for Band offsets has been reduced from 16 to 8. The result indicates that edges are enhanced both subjectively and objectively. The average BDR gain versus HM5.0 is 0.1% for luma and 3% for chroma for the common conditions in JCTVC-G1200 with a decoding time increase of 1%. With YUV BDR (6Y+U+V)/8 the average YUV BDR gain is 0.8%. When ALF is turned off for high efficiency configurations an average BDR gain of 1% for luma for low delay P configuration is obtained with a decoding time increase of 2%.

# Introduction

## Edge offsets in HM5.0

In HM5.0 SAO edge offsets can be used in one of 4 directions, e.g. horizontal, vertical, diagonal from top left to bottom right, or diagonal from bottom left to top right. The specific direction is determined by sao_type_idx = 1 .. 4.

In HM5.0 SAO edge offsets are categorized by:

edgeIdx = 2+k=01Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]]) where recPicture is the picture after deblocking, hPos and vPos are defined below.

Table I Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | 1 |
| vPos[1] | 0 | 1 | 1 | -1 |

The reconstructed picture buffer is modified as:

recSaoPicture[xC+i, yC+j] = recPicture[xC+i, yC+j] + saoValueArray[edgeTable[edgeIdx]] with

i=0..nS-1 and j=0..nS-1, edgeTable[5]={1, 2, 0, 3, 4} and recSaoPicture is the picture after SAO has been applied. saoValueArray contains the edge offsets for the specific direction.

The edges types that HM4.0 SAO edge offsets address are:

edgeIdx = 0

edgeIdx = 1

edgeIdx = 3

edgeIdx = 4

From above one can see that HM4.0 SAO edge offsets do not distinguish between “left” and “right” edges for edgeIdx=1 or edgeIdx=3 and also require that two pixel values are equal to belong to those categories.

## Band offsets in HM5.0

The current HEVC Band Offset classification uses 2 classes, corresponding to two groups of pixel values as depicted in Figure 1 (cf. JCTVC-D122). The intensity range is divided into 32 ranges. Each SAO Band Offset group uses half of the intensity range. The first group is the central range of pixel values containing 16 classes (ranges) and the second group is composed of the 16 classes corresponding to the extremities of the whole intensity interval.

Figure 1: Band Offset classification of current HM5.0.

This classification is useful when the signal of a frame area covers all the intensity of pixel values. But when the signal is compacted to a restricted range of pixel values and when this range is not centered on the middle of the intensity interval, this classification is not adapted.

For the band offset classifications 16 offset values need to be store for each SAO partition of the frame. The buffer size needed to store all SAO parameters for a frame is proportional to the number of SAO partitions and to the sizes of SAO parameters. The worst case is reached when the number of partitions is the maximum and when all these partitions are classified in band offsets where 16 offsets need to be store. In this proposal the number of offsets for the band offset classification is reduced from 16 to 8.

# Proposed

## Edge offsets

To better distinguish between different edges the SAO decoding process for edge offsets is modified as follows for all LCU lines except the 4th line for luma component and 2nd line for chroma components above the LCU boundary:

edgeIdx = 20 + Sign(-2*recPicture[xC+i, yC+j]+recPicture[xC+i+hPos[0], yC+j+vPos[0]]+recPicture[xC+i+hPos[1], yC+j+vPos[1]])+

4k=01(3*k+1) Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]])

where hPos and vPos are same as for HEVC as defined in Table I.

For , which is the 4th line for luma component and 2nd line for chroma components above the LCU boundary, edge offsets are computed as:

edgeIdx = 20 +

4k=01(3*k+1) Sign(recPicture[xC+i, yC+j]-recPicture[xC+i+hPos[k], yC+j+vPos[k]])

In comparison with HEVC the new categorization requires one additional sign, 3 additional multiplications and 3 additional additions. The multiplications can be implemented with shifts. When 4 offsets is selected (saoEoGroupFlag is 0) edgeIdx can be computed as in HM5.0.

The reconstructed picture buffer is modified as:

recSaoPicture[xC+i,yC+j]=recPicture[xC+i,yC+j]+ saoValueArray[edgeTable[edgeIdx]] with i=0..nS-1 and j=0..nS-1, edgeTable[41]=

{1, 1, 0, 0, 2, 2, 0, 5, 0, 7, 0, 0, 0, 0, 0, 0, 2, 2, 0, 0, 0, 0, 0, 3, 3, 0, 0, 0, 0, 0, 0, 6, 0, 8, 0, 3, 3, 0, 0, 4, 4}

The proposed categorization switches between 4 edge offsets and 8 edge offsets as indicated by saoEoGroupFlag. In comparison with HEVC saoValueArray can contain 4 additional edge offsets but still less than the maximum which is obtained also with HEVC when 16 band offsets are used. The same categorization is used for luma and chroma components. Example of edges that are supported are shown below:

4 offsets

8 offsets

Prediction for the edge offset of the local maxima is also used.

Syntax:

| sao_offset_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_offset_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } else { |  |
| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | ue(v) |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] != 0 ){ |  |
| if(sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] <5){ |  |
| sao_eo_group_flag[ cIdx ][ saoDepth ][ rx ][ ry ] | u(1) |
| if(sao_eo_group_flag[ cIdx ][ saoDepth ][ rx ][ ry ]) |  |
| NumSaoClass[ sao_type_idx ] = 8 |  |
| else |  |
| NumSaoClass[ sao_type_idx ] = 4 |  |
| } |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | se(v) |
| } |  |
| } |  |
| } |  |

Semantics:

saoEoGroupFlag is set equal to sao_eo_group_flag [ cIdx ][ saoDepth ][ rx ][ ry ] which is defined above.

saoValueArray is set equal to SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ] which is defined below.

An array SaoOffsetVal is specified as

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 0 ] = 0

if(sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ]  < 5){

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0, 2..NumSaoCategory – 1

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1] =

(sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ 3]- sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ 0]) << ( bitDepth – Min( bitDepth, 10 ) )

with i = 3

if(saoEoGroupFlag==0){

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 5]  = 0

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 6]  = 0

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 7]  = 0

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 8]  = 0

}

}else{

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i+1 ] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0..NumSaoCategory – 1

}

| - sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | - NumSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4/8 | - 1D 0-degree edge |
| - 2 | - 4/8 | - 1D 90-degree edge |
| - 3 | - 4/8 | - 1D 135-degree edge |
| - 4 | - 4/8 | - 1D 45-degree edge |
| - 5 | - 8 | - BO_8o_ 1 |
| - 6 | - 8 | - BO_8o_ 2 |
| - 7 | - 8 | - BO_8o_ 3 |

### Line Memory Analysis of edge offsets

During the 7th JCT-VC Meeting in Geneva, it was pointed out by the group that the proposal increased line memory requirements for LCU-based coding process. Therefore a slight modification is incorporated in the current version of the proposal in order to avoid the line memory increase.

In HM5.0, Sample Adaptive Offset operation requires 0.25 lines of line memory (for luma and each of chroma components) as it was suggested in the Summary Report of CE8 – Non-deblocking Loop Filtering [2]. The line memory requirement for SAO is smaller than 1 line in HM5.0 since actual sample values do not need to be stored in the memory. Instead partial pixel classification results (given by the sign of the difference of two neighboring pixels) can be stored in the memory.

However the second order gradient operation that is given below increases the line memory requirement to 1 line, since it requires actual sample values to be stored in the line memory.

Sign(-2*recPicture[xC+i, yC+j]+recPicture[xC+i+hPos[0], yC+j+vPos[0]]+recPicture[xC+i+hPos[1], yC+j+vPos[1]])

As a solution, the second order gradient is not computed at the 4th line above the bottom LCU boundary in order to avoid the increase in line memory (The value of the second order gradient is set to 0). This means that for one line in an LCU only 4 different edge offsets are used as in the case of HM5.0. For the rest of the pixels in the LCU either 4 or 8 different edge offsets are used depending on the value of saoEoGroupFlag syntax element.

The line memory requirement of the proposal is therefore reduced back to 0.25 lines as in the case of HM5.0 with the line memory amendment. The proposed line memory reduction method has no impact on the coding gain and visual quality.

## Band offsets

In this subset “e” of this Core Experiment, 3 proposed types of Band Offsets are replacing to the current Band Offset classifications illustrated in Figure 1. The classification BO_8o_1, BO_8o_2 and BO_8o_3 correspond, respectively to classes BO_NB_8_SUB_1, BO_NB_4_SUB_2 and BO_NB_2_SUB_4 as described in JCTVC-G246. Compared to HM5.0, the number of offsets is changed and is reduced from 16 to 8. So these classifications need always 8 offsets. The number of bits checked for the classification BO_8o_2 and BO_8o_3 need to be modified compared to the HM5.0. So, for BO_8o_2 and BO_8o_3 respectively 6 and 7 bits are checked for the classification. Yet, these additional 2 bits checking for hardware implementation does not affect the worst case complexity of the SAO loop filtering. Indeed, the worst case complexity is reached with the Edge Offset classification.

| Name | Class | Nb pixel values per class |
| --- | --- | --- |
| BO_8o_ 1 |  | 8 |
| BO_8o_2 |  | 4 |
| BO_8o_3 |  | 2 |

Figure 2: Classification used for Band Offset.

## Center of the Band Offset

For the proposed Band Offset types, the center of each classification is transmitted for each SAO partition. This center is coded by considering a subdivision of the intensity interval into 32 ranges. The center of a class is predicted by the center of the intensity range and a residual is then coded with a unary code.

# Results

## BDR and encoding/decoding time measurements vs HM5.0

|  | All Intra HE |  |  | Random Access HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | -0,1% | -0,8% | -1,4% | -0,2% | -1,9% | -1,6% | 0,0% | -3,1% | -2,9% |
| Class B | 0,1% | -1,8% | -2,4% | -0,1% | -3,3% | -3,4% | -0,1% | -2,6% | -3,0% |
| Class C | 0,1% | -1,1% | -1,1% | 0,0% | -2,0% | -1,7% |  |  |  |
| Class D | 0,1% | -0,8% | -0,9% | 0,0% | -1,6% | -2,2% |  |  |  |
| Class E | 0,1% | -2,7% | -2,7% |  |  |  |  |  |  |
| Overall | 0,1% | -1,5% | -1,7% | -0,1% | -2,3% | -2,4% | -0,1% | -2,8% | -3,0% |
|  | 0,1% | -1,5% | -1,7% | -0,1% | -2,3% | -2,4% | -0,1% | -2,8% | -3,0% |
| Class F | 0,1% | -1,8% | -1,4% | -0,1% | -2,5% | -2,3% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 101% |  |  |

|  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,3% | -5,4% | -5,3% | -0,3% | -4,5% | -4,8% |
| Class C | -0,2% | -3,4% | -3,3% | -0,2% | -3,0% | -3,1% |
| Class D | -0,2% | -2,3% | -4,5% | -0,1% | -1,9% | -4,8% |
| Class E | -0,5% | -4,5% | -3,2% | -0,5% | -4,2% | -3,9% |
| Overall | -0,3% | -4,0% | -4,2% | -0,3% | -3,4% | -4,2% |
|  | -0,3% | -4,0% | -4,1% | -0,3% | -3,4% | -4,2% |
| Class F | -0,7% | -3,8% | -4,1% | -0,5% | -3,4% | -3,5% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

## BDR and encoding/decoding time measurements vs HM5.0 No ALF

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0,1% | -0,8% | -1,3% | -0,1% | -1,5% | -1,5% |
| Class B | 0,1% | -1,7% | -2,2% | -0,3% | -3,3% | -3,3% |
| Class C | 0,1% | -1,1% | -1,0% | -0,2% | -2,2% | -1,9% |
| Class D | 0,1% | -0,7% | -0,8% | -0,1% | -1,8% | -2,5% |
| Class E | 0,1% | -2,5% | -2,5% |  |  |  |
| Overall | 0,1% | -1,4% | -1,6% | -0,2% | -2,3% | -2,5% |
|  | 0,1% | -1,4% | -1,6% | -0,2% | -2,4% | -2,5% |
| Class F | -0,1% | -1,9% | -1,5% | -0,3% | -2,7% | -2,3% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

|  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,7% | -5,5% | -5,3% | -1,6% | -4,8% | -4,9% |
| Class C | -0,6% | -3,5% | -3,9% | -0,7% | -3,3% | -3,3% |
| Class D | -0,3% | -2,5% | -5,9% | -0,3% | -2,4% | -5,4% |
| Class E | -0,9% | -4,5% | -4,5% | -2,0% | -4,5% | -4,5% |
| Overall | -0,6% | -4,1% | -4,9% | -1,1% | -3,8% | -4,6% |
|  | -0,6% | -4,1% | -4,9% | -1,1% | -3,8% | -4,5% |
| Class F | -0,9% | -3,7% | -3,7% | -0,6% | -3,6% | -3,1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

## BDR and decoding time versus HM-5.0 when both CE8.e.5 has ALF=0 and SAO=1

|  | Y | U | V | YUV | dectime | enctime |
| --- | --- | --- | --- | --- | --- | --- |
| AIHE | 0,1% | -1,4% | -1,6% | -0,3 | 100% | 99% |
| AILC | 0,0% | -1,1% | -1,3% | -0,3 | 101% | 99% |
| RAHE | -0,2% | -2,3% | -2,5% | -0,7 | 102% | 100% |
| RALC | -0,2% | -3,2% | -3,6% | -1,0 | 102% | 99% |
| LBHE | -0,6% | -4,1% | -4,9% | -1,6 | 102% | 100% |
| LBLC | -0,7% | -5,3% | -6,0% | -2,0 | 102% | 99% |
| LPHE | -1,1% | -3,8% | -4,6% | -1,9 | 102% | 100% |
| LPLC | -1,1% | -4,9% | -5,9% | -2,2 | 102% | 99% |
| Average | -0,5% | -3,3% | -3,8% | -1,2 | 102% | 99% |

This shows the benefit of using SAO according to CE8.e.5 compared to HM-5.0 when both have ALF=0 and SAO=1. Here it is seen that extending SAO according to CE8.e.5 BDR improvements of (0.5%, 3.3%, 3.8%) is achieved at a decoding time increase of 2% with no change in encoding time.

## BDR and decoding time versus HM-5.0 when CE8.e.5 has SAO=1

|  | Y | U | V | YUV | dectime | enctime |
| --- | --- | --- | --- | --- | --- | --- |
| RAHE10bit | -0,1% | -2,8% | -3,0% | -0,8 | 101% | 100% |
| AIHE | 0,1% | -1,5% | -1,7% | -0,4 | 100% | 99% |
| AILC | -0,7% | -3,5% | -4,4% | -1,5 | 105% | 100% |
| RAHE | -0,1% | -2,3% | -2,4% | -0,6 | 102% | 100% |
| RALC | -1,4% | -7,0% | -7,8% | -2,9 | 105% | 100% |
| LBHE | -0,3% | -4,0% | -4,2% | -1,2 | 101% | 100% |
| LBLC | -2,9% | -13,5% | -15,3% | -5,8 | 105% | 99% |
| LPHE | -0,3% | -3,4% | -4,2% | -1,2 | 101% | 99% |
| LPLC | -9,0% | -16,1% | -17,8% | -11,0 | 107% | 99% |
| Average | -1,6% | -6,0% | -6,8% | -2,8 | 103% | 100% |

This shows the benefit of using SAO according to CE8.e.5 compared to conditions that not uses SAO for LC configurations. Here it is seen that BDR improvements of (1.6%, 6.0%, 6.8%) is achieved at a decoding time increase of 3% with no change in encoding time.

## Visual inspection

Generally the proposal performs subjectively similar as the anchor. However we can see more areas where the proposal have sharper edges and less coding noise than the anchor than we can see areas where the proposal looks worse than the anchor.

# Conclusions

It has been shown that by adding the possibility to use more specific edge offsets and Band offsets when needed both the subjective and the objective quality can be improved. The average YUV BDR is improved by 0.8% (average BDR luma 0.1%, chroma 3%) for common conditions in JCTVC-G1200. The most improvements are seen for low delay configurations. If ALF is not used for HE an average BDR reduction of 1% for luma can be obtained for low delay P. The encoding time is similar to HM5.0 and decoding time is increased by 1% compared to HM5.0 for common conditions in JCTVC-G1200.

# References

K.Andersson, P. Wennersten, R. Sjöberg, “Modified SAO edge offsets”, JCTVC-G490, Switzerland, November 2011

G. Laroche, T. Poirier, P. Onno, “On additional SAO Band Offset classifications”, JCTVC-G246, Switzerland, November 2011

C.-M. Fu, C.-Y. Chen, Y.-W. Huang, and S. Lei, “TE8 Subtest3: Picture Quadtree-based Adaptive Offset”, JCTVC-D122, Korea, January. 2011.

T. Yamakage et al., “CE	8: Summary report of Core Experiment on non-deblocking loop filtering”, JCTVC-G038, Switzerland, November 2011.

T. Yamakage et al., “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering”, JCTVC-G1208, Switzerland, November 2011

F. Bossen, “Common test conditions and software reference configurations” JCTVC-G1200, Switzerland, November 2011

“WD5: Working Draft 5 of High-Efficiency Video Coding”, JCTVC-G1103, Switzerland, November 2011

# Patent rights declaration(s)

Ericsson AB may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).