| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0226 |
| --- | --- |

| Title: | On derivation of luma QP predictor for Wavefront Parallel Processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Satoshi Shimada, Kimihiko Kazui, Junpei Koyama and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 sshimada@jp.fujitsu.com kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes to fix the bug in the derivation process of the QP predictor of the first quantization group (QG) of each TreeBlock (TB) line when Wavefront Parallel Processing (WPP) is used. In the WD5/HM5, the QP predictor of the first QG in the N-th TB line is derived from QP of the last QG in the (N－1)-th TB line. This QP derivation significantly prevents TB line based parallel processing using WPP. It is proposed that the QP predictor of the first QG in the N-th TB line is set to QP of the last QG in M-th TB line where M = N－(num_substreams_minus1 + 1). As the result, the parallelism of WPP is improved.

# Problem Statement

## Overview of Wavefront parallel processing

Wavefront Parallel Processing (WPP) is adopted in WD5[1] as the technique of Treeblock (TB) line based parallel processing. The characteristics of WPP is the following.

- The horizontal position of the processing TB in the M-th TB line is advanced at least 2 TBs from that in the (M+1)-th TB line. See Figure 1. This characteristic ensures the availability of above TB and above-right TB.

- CABAC context status at the first TB in the M-th TB line is initialized using that in the (M - 1)-th TB line in order to reduce the loss in entropy coding efficiency.

- Each processing thread generates the corresponding sub-bitstream in parallel.

Figure 1. Position of processing TBs when WPP is used.

## Problems in Luma QP prediction

The AVC-Style QP prediction scheme was adopted in WD5. The adopted scheme is summarized as follows.

- QP predictor (QPY,PREV) in the first quantization group in a slice is set to SliceQPY.

- QP predictor (QPY,PREV) in the other quantization groups in the slice is set to QP of the previous quantization group.

There exists inconsistency between the AVC-Style QP prediction scheme and the nature of WPP. In Figure 2, X is the first TB of in one TB line. According to the WD5 specification, the QP predictor (QPY,PREV) of X is set to QP of Treeblock Y. Therefore, QPY,PREV of X cannot be derived until deriving QP of Y.

The AVC-Style QP prediction scheme becomes a bottle-neck of parallel processing using WPP. At the encoder-side, cu_qp_delta cannot be calculated until finishing processing of Y. As the result, the entropy coding processing of X cannot be started even if the quantization process of X has been finished. At the decoder-side, the QP predictor of X cannot be derived until finishing processing of Y. As the result, QP of X cannot be derived and inverse quantization of X cannot be started soon. Thus the QP prediction scheme in WD5 is not suitable for WPP.

Figure 2. QP prediction

# Proposal

We propose to modify the derivation of the QP predictor when WPP is used (i.e. entropy_coding_synchro is not equal to zero) in order to maintain the parallelism of WPP.

## QP predictor of First Quantization Group of Each Thread

The QP predictor of the first QG in the N-th TB line is set to SliceQPY, where N = [ 1, num_substreams_minus1 + 1 ].

Alternatively, The QP predictor can be derived from above. In this case, The QP predictor of the first QG in the first TB line is set to SliceQPY, and that in the N-th TB line is set to QP of above QG, or of the last QG in above TB, where N = [ 2, num_substreams_minus1 + 1 ].

## QP predictor of not-First Quantization Group of Each Thread

The QP predictor of the first QG in the N'-th TB line, where (N' > num_substreams_minus1 + 1), is derived from the QP of the last QG in the (N'－(num_substreams_minus1 + 1))-th TB line.

For example, the QP predictor of the first QG in Treeblock X is set to QP of the last QG in Treeblock Z, not in Treeblock Y, as depicted in Figure.3.

Fig-3  Proposed QP prediction when WPP is used.

# Conclusion

We suggest to modify the QP prediction for WPP as proposed in this contribution.

# References

[1] "WD5: Working Draft 5 of High-Efficiency Video Coding", JCTVC-G1103, November 2011, Geneva Switzerland

# Patent rights declaration

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).