| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0228 |
| --- | --- |

| Title: | Non-CE11: Proposed Cleanup for Transform Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Nguyen D. Marpe T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes a cleanup for transform coefficient coding of the current HEVC design. In particular, this document describes a generic scheme for the parameter selection related to the coding of transform coefficient levels with CABAC. To be more specific, the proposed cleanup relies on the same simple subset of rules for the context model selection and the Rice parameter derivation results in a simplification and harmonization. Further, all transform block sizes and shapes apply the proposed context model selection, Rice parameter derivation method, and use the same set of context models for the respective bin index resulting in a unified scheme. By applying the generic parameter selection, the proposed scheme allows the usage of a meaningful number of context models. On the top of that, the cleanup scheme has advantage over the existing coding scheme in terms of coding performance.

# Introduction

The transform coefficient coding in HEVC (WD5 [1]) is mainly an extension of H.264/AVC [2] to block sizes larger than 8x8 with respective advanced context model selection introduced in [3]. In order to achieve a generalized transform coefficient coding scheme regardless of transform block size or shape and scan order, this document proposes a cleanup that uses the same simple subset of rules to derive all context model indices and the Rice parameter allowing the coding of the transform coefficient levels in a single scan pass.

The proposed cleanup has following benefits compared to the current design:

- Generic scheme without any constraints

- Same subset of rules for context model selection and Rice parameter derivation

- Usage of the same subset of rules for all transform block sizes and shapes

- Usage of the same subset of context models for all transform block sizes and shapes

- Enabling the pre-calculation of context models selected for next scan positions with respective Rice parameter

- Meaningful number of context models – only 52 context models instead of 104 for related syntax elements

- Less memory addressing than the current scheme due to single scan pass

# Description

In the current design of HEVC as well as in H.264/AVC, the coding of transform coefficient levels is separated into multiple scan passes. In the first step, the first bin (or the bin index 0, bin0) of the bin string resulting from the binarization of a transform coefficient level is coded. Next, the remaining bins are coded in a further scan pass or in multiple scan passes. The reason for that separation is mainly a better context model selection in terms of coding performance. Often, separation of bins coded with adaptive context models and bins coded with a fixed probability of 0.5 are proposed and has been adopted.

In order to have cleanup and to have a harmonized and unified scheme with reasonable trade-off between coding performance, generalization, and context memory, the separation of the bin0 (the first bin of the bin string, also referred as significant_coeff_flag) and the remaining bins is omitted. Thus, the transform coefficients are coded in the proposed scheme in a single scan pass. For each scan position, neighbours covered by a local template are evaluated as done for bin0 in the current design of HEVC for block sizes larger than 8x8 and for non-square shapes. From this evaluation, context models and the Rice parameter, which controls the adaptive binarization of the remaining absolute value, are derived. To be more specific, the context models for the bin0, bin1 and bin2 are selected (bin1 and bin2 are also referred as coeff_abs_greater1_flag and coeff_abs_greater2_flag). Note that this proposal does not propose modifications of the signaling of the last significant flag, the scan order, the binarization scheme, and the high-level significance flag. Further, note that the proposed scheme is very similar to the existing scheme for the context model selection for bin0 of HEVC and can be regarded as an extension of that scheme.

## Local Template

The proposed scheme codes all information of a transform coefficient in each coding step and therefore no separation depending on the bin index is made as in the current design of HEVC. Thus, the complete information on the transform coefficients lying in the neighborhood covered by the local template is available. Beginning with the last significant scan position as in the current design of HEVC, the local template is applied for all scan positions lying on the scan path regardless of transform block size and shape. The proposed local template is the same as used for the context model selection of the bin0 in the current HEVC design. However, the proposed scheme evaluates all neighbours without any restrictions resulting in a generic design. An example for the local template is given in Fig. 1 for a 8x8 transform block with diagonal scan, where L denotes the last significant scan position, x denotes the current scan position and xi with i ϵ [0,4] denotes the neighbours covered by the local template.

Fig. 1: Local template with diagonal scan

## Context Model Selection and Rice Parameter Derivation

In addition to the number of significant neighbours (num_significant_coeff) covered by the local template as the result of the evaluation in the current HEVC design, the absolute sum of the neighbours is calculated (sum_absolute_level). Note that the absolute sum is available as the proposed scheme codes all information on a transform coefficient level in a single step. From these two values, the absolute sum of each level minus 1 is calculated (sum_absolute_levelMinus1). The local template evaluation of the proposed scheme is summarized in the formulas below with xi stands for the transform coefficient level of a neighbor covered by the local template. Note that num_significant_coeff is used for the context model selection for the bin0 in the current HEVC design for block sizes larger than 8x8 and for non-square shapes.

numsignificantcoeff=δixi

with	δixi=1xi≠00xi=0

sumabsolutelevel=xi

sumabsolutelevelMinus1=δjxi

with	δjx=xi−1xi>00xi=0

sum_absolute_level and sum_absolute_levelMinus1 are used to derive context indices for bin0, bin1, bin2, and to determine the Rice parameter r. For bin0, the resulting sum_absolute_level from the local template evaluation is directly mapped to a context model index with a cut-off value of 5. The same rule is applied for the calculation of the context model index of bin1 and bin2 by using sum_absolute_levelMinus1 and a cut-off value of 4. This derivation rule is summarized in the following with c0 represents the context model index for bin0, c1 the context model index for bin1, and c2 the context model index for bin2.

c0=minsumabsolutelevel,5

c1=minsumabsolutelevelMinus1,4+1

c2=minsumabsolutelevelMinus1,4+1

Note that the derivation rule for bin1 and bin2 are the same.

Fig. 2: Additional context index for different spatial arrangements in WD4 and WD5

Next, an additional offset is calculated as done for bin0 in the current design of HEVC. In the previous design of HEVC (WD4), the additional context index depends on the location of the current scan position. In particular, the additional context index depends on the diagonal of the current scan position as depicted in the left of Fig. 2. That means the scan positions of the first two diagonals have a different context model set and the scan positions of the next three diagonals have a further different context model set than the remaining scan positions. In the latest design of HEVC, the second diagonal is somehow adaptive and depends on the current transform block size as can be seen from the right illustration of Fig. 2. Based on this existing concept of WD4, the proposed scheme uses fixed diagonal bounds for the additional context model index of bin0 equal to the WD4 design as illustrated on the left of Error: Reference source not found. It should be noted that an adaptive bound make no sense in terms of generalization and simplification. In contrast to the scheme in WD4, the second context model index bound is applied for luma only. The same concept is extended to the context model index of bin1 and bin2. For this case, the scan positions consist of additional context model indices are illustrated on the right in Error: Reference source not found.

Fig. 3: Proposed additional context index for different spatial arrangements

The additional context model indices are applied for luma only, while no additional context model indices are used for chroma in the case of bin1 and bin2. The whole concept is summarized in the following formulas, where x denotes the horizontal spatial location inside the transform block of the current scan position, y denotes the vertical spatial location, and cIdx denotes the current plane type with 0 stands for luma.

c0=c0+fx,y,6,2+δkfx,y,6,5,cIdx

c1=c1+δkfx,y,5,3,cIdx+δkfx,y,5,10,cIdx

c2=c2+δkfx,y,5,3,cIdx+δkfx,y,5,10,cIdx

with	fx,y,n,t=nx+y<t0x+y≥t

and	δku,v=uv=00v≠0

Again, note that the formula for c1 is the same as for c2. In addition to the same context model index for the bin1 and bin2, the same context model is used for the coding of bin1 and bin2. Further, for the first scan position in coding order (i.e. the last signification scan position); a separate context model index is used for bin1 and bin2. Note that the first bin index (bin0) is inferred for the last significant scan position. This separate context model is never selected again by the context model selection scheme and is assigned as c1=0.

The Rice parameter r is derived as follows. For each scan position, the parameter is set to 0. Then, the sum_absolute_levelMinus1 is compared against a threshold set tR={3,9,21}. In other words, the Rice parameter is 0 if the sum_absolute_levelMinus1 falls into the first interval, is 1 if sum_absolute_levelMinus1 falls into the second interval and so on. The derivation of the Rice parameter r is summarized in the following.

rx=0123x∈0,3x∈[4,9]x∈10,21x>21

with	x=sumabslolutelevelMinus1

## Further Aspects

The following table gives an overview on the number of context models compared to the current transform coefficient scheme in HEVC.

|  | HM5/WD5 |  | H0228 |  |
| --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma |
| bin0 | 27 | 21 | 18 | 12 |
| bin1 | 24 | 8 | 16 | 6 |
| bin2 | 18 | 6 | 0 | 0 |
| Total | 104 |  | 52 |  |

Further, by using a single scan pass, less memory addressing is required compared to the multiple scan passes scheme as used in the current HEVC design. Furthermore, the context model indices and the Rice parameter can be fully pre-calculated in case of diagonal scan for the next diagonal. For other scans with dependencies, a pre-calculation is also possible. For example, if a pre-calculation results in a value larger than the cut-off values, the context model indices can be derived regardless of the dependent neighbor. The next table shows the modified syntax diagram of the proposed cleanup. As can be seen from the syntax diagram, the issue of different addressing operators for different syntax elements is also resolved.

| residual_coding_cabac( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( max( log2TrafoWidth, log2TrafoHeight ) > 3) { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i = = numLastSubset) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] = 1 |  |
| else |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| else |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater1_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater2_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_minus3[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| coeff_sign_flag[ xC ][ yC ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ xC ][ yC ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ xC ][ yC ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |
| } else { |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) ) |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater1_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater2_flag[ xC ][ yC ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_minus3[ xC ][ yC ] | ae(v) |
| } |  |
| } |  |
| coeff_sign_flag[ xC ][ yC ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ xC ][ yC ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ xC ][ yC ] ) |  |
| } else { |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

# Results

The code base for this experiment is the HM 5.0 software. Simulations run on a cluster with the same hardware configurations. Simulation configurations base on common test conditions defined in [4]. The complete results is available in the enclosed excel sheet.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.7% | -1.0% | -0.9% | -1.0% | -1.2% | -1.2% |
| Class B | -0.6% | -0.6% | -0.7% | -0.9% | -1.0% | -1.0% |
| Class C | -0.7% | -0.4% | -0.5% | -1.1% | -0.9% | -0.9% |
| Class D | -0.9% | -0.5% | -0.6% | -1.3% | -1.0% | -1.1% |
| Class E | -0.9% | -1.0% | -1.1% | -1.2% | -1.5% | -1.5% |
| Overall | -0.7% | -0.7% | -0.7% | -1.1% | -1.1% | -1.1% |
|  | -0.7% | -0.7% | -0.7% | -1.1% | -1.1% | -1.1% |
| Class F | -1.1% | -1.1% | -1.0% | -1.5% | -1.1% | -1.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | -0.3% | -0.3% | -0.4% | -0.6% | -0.4% | -0.8% |
| Class B | -0.3% | -0.2% | -0.1% | -0.5% | -0.4% | -0.5% |
| Class C | -0.6% | -0.1% | -0.1% | -0.8% | -0.4% | -0.5% |
| Class D | -0.7% | -0.3% | -0.2% | -1.0% | -0.1% | -0.6% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -0.2% | -0.2% | -0.7% | -0.3% | -0.6% |
|  | -0.5% | -0.2% | -0.1% | -0.7% | -0.3% | -0.6% |
| Class F | -1.0% | -0.8% | -0.8% | -1.3% | -1.1% | -1.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.5% | -0.3% | -0.6% | -0.3% |
| Class C | -0.3% | 0.4% | 0.4% | -0.7% | -0.3% | 0.0% |
| Class D | -0.4% | -0.1% | 0.2% | -0.7% | 0.7% | 0.1% |
| Class E | 0.0% | 0.7% | -1.4% | -0.1% | 0.0% | 0.1% |
| Overall | -0.2% | 0.3% | 0.0% | -0.5% | -0.1% | -0.1% |
|  | -0.2% | 0.3% | 0.0% | -0.5% | -0.2% | -0.1% |
| Class F | -0.9% | -0.6% | -0.8% | -1.1% | -2.0% | -1.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# References

- B. Bross, W.-J. Han, J. R. Ohm, G. J. Sullivan, and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” Doc. JCTVC-G1103, Geneva, Nov. 2011.

- ITU-T and ISO/IEC, “Advanced Video Coding for Generic Audiovisual Services,” ITU-T Rec. H.264 and ISO/IEC 14496 10, version 13, 2010.

- T. Nguyen et. al., “Improved Context Modeling for Coding Quantized Transform Coefficients in Video Compression,” 28th IEEE Picture Coding Symposium, Nagoya, Dec. 2010

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200, Geneva, Nov. 2011.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).