| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0230 |
| --- | --- |

| Title: | CE4 subtest 2.1: Enhancements of HEVC quantization matrix |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Junichi Tanaka Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN Minhua Zhou Vivienne Sze Texas Instruments Inc., USA | Tel: Email: | +81-50-3750-2740 Yoshitaka.Morigami@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com +1 214 480 3816 zhou@ti.com Sze@ti.com |
| Source: | Sony Corporation, TI |  |  |

_____________________________

# Abstract

This contribution reports the results of CE4 subtest 2.1. Simple extension of AVC quantization matrix to support large transform was adopted in HM5 (JCTVC-G083, G094 and G434). It encodes all matrix coefficients by DPCM with exp-Golomb code. However coding efficiency of matrix data is not high and memory size to store matrix coefficient is large, because HEVC supports up to 32x32 transform. This contribution reports the coding efficiency and complexity of remaining tools in JCTVC-G083, G094 and G434. Two configurations of combination of tools were tested. Option 1 reduces the generated bits for matrix data from 4.3% to 12.1%, while memory size is reduced 12%. Option 2 reduces the generated bits from 10.9% to 25.3%, while memory size is reduced 24%.

# Introduction

HM5 uses the following method to encode quantization matrix.

- Copy mode (copy from the same size of matrix)

- DPCM

- Exp-Golomb coding

This method is simple extension of AVC quantization matrix to support 16x16 and 32/32 matrix. However, memory size to store matrix data is 18 times larger than that of AVC. (See Table 1and Table 2) Therefore the method to reduce this memory size is proposed.

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y | 128 |
| Total |  |  |  | 224 |

Table 1: Memory size of AVC

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y/Cb/Cr | 384 |
| 16x16 | 256 | Intra/Inter | Y/Cb/Cr | 1536 |
| 32x32 | 1024 | Intra/Inter | Y | 2048 |
| Total |  |  |  | 4064 (181.4％) |

Table 2: Memory size of HEVC

# Proposal

On top of HM5.0 quantization matrix compression method, JCTVC-G083, JCTVC-G094 and G434 proposed the following tools.

- Tools for enhancing lossless quantization matrix coding

  - 135 degree symmetry processing

  - Adaptive signed/unsigned exp-Golomb coding

  - Residual coding

- Tools for lossy quantization matrix coding for further compression

  - 32x32/16x16 quantization matrices not transmitted and directly up-sampled from 8x8 ones of same type

  - Matrix up-sampling on decoder for 16x16/32x32 matrices are coded at reduced resolutions

  - Matrix up- and down-sampling for matrix prediction of arbitrary size

  - Residual quantization (not tested in this contribution)

Note that a simple and unified matrix up- and down-sampling method (see section 2.1) is applied in the unified design to minimize data buffer size.

The lossy tools are added on the top of lossless tools, and can be easily turned off.

Two configurations of the combinations of the tools were tested.

## Option1

In this configuration, 4x4 and 8x8 matrix are encoded losslessly. 16x16 and 32x32 are up-sampled by coping nearest neighbor coefficient as below.

- Up-sampling:  up-sampled_matrix(x, y)    = matrix(x/F, y/F)

- Down-sampling: down-sampled_matrx(x, y)  = matrix(x*F, y*F)

where matrix() is input matrix for up/down-sampling, and F is up-down/sampling factor. This is the simplest way of doing matrix up/down sampling w/o needs of additional data buffer to buffer up/down sampled matrices. This method can be used for both 16x16/32x32 matrix up-sampling, and matrix prediction of arbitrary size. Memory size can be reduced as shown in Table 3. In this option only 4x4 and 8x8 matrices are encoded in the bitstream as shown in Figure 1.

Figure 1: Option 1 configuration

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y/Cb/Cr | 384 |
| 16x16 | 256 | Intra/Inter | Y/Cb/Cr | 0 |
| 32x32 | 1024 | Intra/Inter | Y | 0 |
| Total |  |  |  | 480 |

Table 3: Memory size of option 1

## Option2b

In this configuration, 16x16 and 32x32 matrices are down-sampled to 8x8 and then encoded in the bitstream as shown in Figure 2. Up/down-sampling method is the same as option 1. Required memory size is shown in Table 4. In this option, different characteristics of quantization matrix can be defined for 16x16 and 32x32 respectively.

Figure 2: Option 2b configuration

| Matrix size | Num of values | PU Type | Component | Memory Size (byte) |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | Intra/Inter | Y/Cb/Cr | 96 |
| 8x8 | 64 | Intra/Inter | Y/Cb/Cr | 384 |
| 16x16 | 256 | Intra/Inter | Y/Cb/Cr | 384 |
| 32x32 | 1024 | Intra/Inter | Y | 128 |
| Total |  |  |  | 992 |

Table 4: Memory size of option 2b

# Simulation results

The results are summarized in Table 5 and Table 6. Option 1 reduces the generated bits from 4.3% to 12.1%. Option 2b reduces the generated bits from 10.9% to 25.3%.

| (Subtest2.0, anchor) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Total Bit | 8779 | 5037 | 12157 | 15677 | 8739 | 10863 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.66 | 0.73 | 0.7 | 0.66 | 0.67 |
| Dec Time[sec] | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 | 0.01 |
| (Subtest2.1, option1) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| Total Bit | 548 | 372 | 686 | 668 | 412 | 568 |
| MaxError | 30 | 30 | 85 | 47 | 62 | 96 |
| AvgError | 4.11 | 5.23 | 18.23 | 6.32 | 17.35 | 34.11 |
| Enc Time[sec] | 0.67 | 0.65 | 0. 71 | 0.68 | 0.67 | 0.66 |
| Dec Time[sec] | 0.01 | 0.01 | 0.01 | 0.02 | 0.01 | 0.01 |
| Ratio | 6.2% | 7.4% | 5.6% | 4.3% | 4.7% | 5.2% |
| (Subtest2.0, anchor) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 9443 | 5053 | 12707 | 24825 | 9357 | 11259 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.7 | 0.75 | 0.69 | 0.73 | 0.72 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| (Subtest2.1, option1) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 898 | 612 | 1096 | 1470 | 652 | 922 |
| MaxError | 28 | 21 | 103 | 129 | 55 | 143 |
| AvgError | 4.09 | 3.28 | 21.81 | 18.01 | 15.52 | 31.48 |
| Enc Time[sec] | 0.68 | 0.69 | 0.66 | 0.69 | 0.69 | 0.67 |
| Dec Time[sec] | 0.01 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| Ratio | 9.5% | 12.1% | 8.6% | 5.9% | 7.0% | 8.2% |

Table 5: Results of option 1

| (Subtest2.0, anchor) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Total Bit | 8779 | 5037 | 12157 | 15677 | 8739 | 10863 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.66 | 0.73 | 0.7 | 0.66 | 0.67 |
| Dec Time[sec] | 0.01 | 0.02 | 0.02 | 0.01 | 0.02 | 0.01 |
| (Subtest2.1, option2b) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| Total Bit | 1312 | 850 | 1712 | 1704 | 1288 | 1390 |
| MaxError | 39 | 16 | 94 | 40 | 39 | 94 |
| AvgError | 2.39 | 0.67 | 5.30 | 4.94 | 2.62 | 4.48 |
| Enc Time[sec] | 0.65 | 0.64 | 0.66 | 0.65 | 0.65 | 0.66 |
| Dec Time[sec] | 0.02 | 0.01 | 0.01 | 0.02 | 0.01 | 0.01 |
| Ratio | 14.9% | 16.9% | 14.1% | 10.9% | 14.7% | 12.8% |
| (Subtest2.0, anchor) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 9443 | 5053 | 12707 | 24825 | 9357 | 11259 |
| MaxError | 0 | 0 | 0 | 0 | 0 | 0 |
| AvgError | 0 | 0 | 0 | 0 | 0 | 0 |
| Enc Time[sec] | 0.7 | 0.7 | 0.75 | 0.69 | 0.73 | 0.72 |
| Dec Time[sec] | 0.02 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| (Subtest2.1, option1) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 1788 | 1278 | 2578 | 3896 | 1936 | 2240 |
| MaxError | 43 | 8 | 48 | 115 | 20 | 48 |
| AvgError | 2.55 | 0.64 | 5.82 | 14.79 | 2.76 | 4.99 |
| Enc Time[sec] | 0.68 | 0.67 | 0.71 | 0.66 | 0.68 | 0.69 |
| Dec Time[sec] | 0.01 | 0.01 | 0.02 | 0.01 | 0.02 | 0.01 |
| Ratio | 18.9% | 25.3% | 20.3% | 15.7% | 20.7% | 19.9% |

Table 6: Results of option 2b

Lossless coding configuration was also tested. The results of lossless coding is summarized in the attached Excel file.

# Conclusion

This contribution proposes two configuration of the remaining tools in JCTVC-G083, G094 and G434. Option 1 reduces the generated bits for matrix data from 4.3% to 12.1%, while memory size is reduced 12%. Option 2 reduces the generated bits from 10.9% to 25.3%, while memory size is reduced 24%. Since memory size to store quantization matrix in HM5 is large, this method should be included in CD and HM6.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: Proposed syntax and semantics

The detail syntax and semantics of proposed method is summarized in this Annex.

Table 7: syntax

|  | Descriptor |
| --- | --- |
| qscale | ue(v) |
| large_block_size_QM_present_flag | u(1) |
| for(SizeID=0;SizeID<4;SizeID++){ |  |
| for(MatrixID=0;MatrixID<(SizeID==3)?2:6;MaxtirxID++) if (MatrixID < 2 ¦¦ large_block_size_QM_present_flag) { |  |
| pred_mode | ue(v) |
| if(pred_mode==0){ //Copy |  |
| pred_matrix_id_delta | ue(v) |
| } else if(pred_mode==1) {//Pred of same size |  |
| if (MatriID >1) down_sampling_id | u(1)/ue(v) |
| symmetry_flag | u(1) |
| pred_size_id_delta | ue(v) |
| pred_matrix_id_delta | ue(v) |
| if (symmetry_flag ¦¦ MatriID >1) residual_flag | u(1) |
| if(residual_flag) { |  |
| scaling_list_residual(ScalingListResidual[SizeID][MatrixID],symmetry_flag, (1<<(4+(SizeID<<1)))) |  |
| } |  |
| }else if(pred_mode==2){//DPCM |  |
| if (MatriID >1) down_sampling_id | u(1)/ue(v) |
| symmetry_flag | u(1) |
| scaling_list(ScalingList[SizeID][MatrixID],symmetry_flag,(1<<(4+(SizeID<<1)))) |  |
| } |  |
| } |  |
| } |  |